3GPP TSG-SA5 Meeting #158 	S5-246357

Orlando, USA, 18 – 22 November 2024

# Source:	Ericsson, Vodafone, Deutsche Telekom, Telecom Italia

# Title:	Signalling traffic monitoring management operations of the drafted TS28.abc

# Document for:	Approval

Agenda Item:	6.19.22

# 1	Decision/action requested

For approval.

# 2	References

[1]	S5-245981 new WID signalling monitoring

[2]	S5-245336, initial skeleton of draft TS28.abc signalling monitoring

# 3	Rationale

Defines Signalling traffic monitoring management operations

# 4	Detailed proposal

*** START OF NEXT CHANGE ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x2]	3GPP TS 28.532: "Generic management services".

[x3]	3GPP TS 28.533: "Management and orchestration; Architecture framework"

[x14]	3GPP TS 33.501: " Security architecture and procedures for 5G System "

*** START OF NEXT CHANGE ***

# 5	Signalling traffic monitoring management operations (stage 2)

## 5.1	Overview

The STM producer is configured by a STM consumer via a secured link. The network signalling messages at 5GC control plane are targeting to be collected. Any collected signalling traffic messages are forwarded by the STM producer to the STM monitoring system using a secured tunnel.

The STM producer may be configured by a STM consumer located in OAM system. And the STM control object may be enabled/disabled by a STM consumer located in OAM system or located in STM system. This is out of scope of this specification.

The signalling traffic monitoring architecture is shown in Figure 5.1-1.

Figure 5.1-1 signalling traffic monitoring architecture

## 5.2	Provisioning Operations

This clause provides the stage 2 definitions of STM provisioning operations, including creation and deletion, as well as enablement and disablement, for managing STM reporting control objects. According to clause 4.2.2 of 3GPP TS 28.533 [x3], these operations are the MnS component type A. The operations specified in this clause in combination with a NRM (MnS component type B) constitute a MnS, as defined in clause 4.3 of 3GPP TS 28.533 [x3] providing generic provisioning services for supported NRM (MnS component type B) of all MnS.

The creation operation is invoked by STM consumers to request a STM producer to create, one or more STM reporting control objects using createMOI or changeMOIs operations as specified in 3GPP TS 28.532[x2]. Once the STM reporting control object is created, it may be enabled or disabled by the STM consumer.

If the STM reporting control object is enabled by the STM consumer, the STM producer shall start collecting and reporting the signalling messages according to the STM reporting control object definition. The enablement operation may be invoked by the STM consumer using changeMOI containing only the enablement attribute.

If the STM reporting control object is disabled by the STM consumer, the collecting and reporting the signalling messages shall be stopped. The disablement operation may be invoked by the STM consumer using changeMOI containing only the disablement attribute, without STM reporting control object deleting.

If the STM consumer is located in the STM system, it may be only allowed to perform enabling and disabling operation on the STM reporting control object.

In case of a failure to report the requested signalling protocol messages, a STM administrative message may be sent to the STM stream target.

The deletion operation is invoked by STM consumers to request a STM producer to remove one or more STM reporting control objects using deleteMOI or changeMOIs operations as specified in 3GPP TS 28.532[x2]. Upon successful remove of the STM reporting control objects, the STM producer shall stop collecting and reporting any signalling traffic.

The STM provision operations shall be secured according to 3GPP TS 33.501 [x14].

## 5.3	Streaming Services

STM reporting consists of STM Payload that shall be output to streams. STM Payloads are used to carry the captured signalling traffic messages being reported or to convey various administrative messages associated with the signalling traffic collection. STM Stream Target is used by a STM consumer to indicate where the STM report shall be streamed to.

The STM streaming service shall be secured according to 3GPP TS 33.501 [x14].

*** START OF NEXT CHANGE ***

# Annex B (informative):
Plant UML source code

## B.1	STM architecture

The following PlantUML source code is used to describe STM architecture. As depicted by Figure 5.1.-1:

@startuml

frame "Signalling Traffic Monitoring System" as STMsystem {

frame  "STM management" as Zone2 {

rectangle "STM Consumer" as STMConsumer1 #whitesmoke

}

frame "STM collection" as Zone3 {

rectangle "STM Streaming Target" as STMConsumerURI #whitesmoke

}

}

frame "5GC" as Zone1 {

rectangle "5G NF" as NF1 #lightgrey {

rectangle "STM Producer" as STMProducer1 #ivory

}

together {

rectangle "5G NF" as NF2 #lightgrey

rectangle "STM Producer" as STMProducer2 #ivory

}

}

STMProducer2 ~ NF2

STMConsumer1 -[hidden]-- STMConsumerURI

STMConsumer1 <-> STMProducer1  : <&lock-locked>Configuration/Enabling

STMConsumer1 <--> STMProducer2 : <&lock-locked>Configuration/Enabling

STMProducer2 => STMConsumerURI : <&lock-locked>Streaming Reporting

STMProducer1 ==> STMConsumerURI: <&lock-locked>Streaming Reporting

NF1 <.> NF2 : Signalling Traffic to Monitoring

@enduml

*** END OF CHANGE ***