3GPP TSG-SA5 Meeting #158	S5-246374

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | 0196 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS28.105 Correct stage2 and stage3 definition for aIMLInferenceName |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT |  |  |  |  |  | Date: |  |  | 2024-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | An attribute defined as String, especially for the important attribute name “aIMLInferenceName” which is used for multiple IOCs throughout the whole ML model LCM management, is not a good design pattern to support interoperability in multi-vendor scenarios. Also, the allowed values for string are not described in stage 3 openAPI definition. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the the type for the attribute“aIMLInferenceName” to <<Choice>> and each Choice is defined as Enumeration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Implementation issue for attribute “aIMLInferenceName” |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.4.X(new), 7.4a(new), 7.5.1, OpenAPI/TS28105_AiMlNrm.yaml |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1449 at commit 3bd23bf3c464495f32b9bc8597a2c5bdbce4420b |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[3]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[4]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[5]	3GPP TS 32.425: "Telecommunication management; Performance Management (PM); Performance measurements Evolved Universal Terrestrial Radio Access Network (E-UTRAN)".

[6]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[7]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[8]	Void

[9]	3GPP TS 28.405: "Telecommunication management; Quality of Experience (QoE) measurement collection; Control and configuration".

[10]	Void

[11]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[12]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[13]	3GPP TS 32.156: "Telecommunication management; Fixed Mobile Convergence (FMC) Model repertoire".

[14]	3GPP TS 32.160: "Management and orchestration; Management service template".

[15]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[16]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

[17]	3GPP TS 38.401: "NG-RAN; Architecture description".

[18]	3GPP TS 28.541: " Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[19]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[X]	3GPP TS 29.520: "5G System; Network Data Analytics Services; Stage 3".

2nd Change

7.4.X	AIMLInferenceName <<choice>>

7.4.X.1	Definition

This <<choice>> represents the type of inference that the ML model supports.

7.4.X.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| CHOICE_1.1 mDAType | M | T | T | F | T |
| CHOICE_2.1 nwdafAnalyticsType | M | T | T | F | T |
| CHOICE_3.1 ngRanInferenceType | M | T | T | F | T |
| CHOICE_4.1 vSExtensionType | O | T | T | F | T |

7.4.X.3	Attribute constraints

| Name | Definition |
| --- | --- |
| CHOICE_1.1 mDAType | This attribute shall be supported, when the MnS producer supports management of AI/ML inference for MDA |
| CHOICE_2.1 nwdafAnalyticsType | This attribute shall be supported, when the MnS producer supports management of AI/ML inference for NWDAF |
| CHOICE_3.1 ngRanInferenceType | This attribute shall be supported, when the MnS producer supports management of AIML inference for NG-RAN |
| CHOICE_4.1 vSExtensionType | This attribute shall be supported, when the MnS producer supports a management activity for an ML model with inference name is vendor's specific extensions |

7.4.X.4	Notifications

The notifications specified for the IOC using this <<datatype>> for its attribute(s), shall be applicable.

3rd Change

7.4a	Enumerations

7.4a.1	NgRanInferenceType <<enumeration>>

Table 7.4a.1-1: <<enumeration>> NgRanInferenceType

| Enumeration value | Description |
| --- | --- |
| "NG_RAN_NETWORK_ENERGY_SAVING" | Indicates that the MDA type for the Network Energy Saving defined in TS 38.300 [16] |
| "NG_RAN_LOAD_BALANCING" | Indicates that the MDA type for Load Balancing defined in TS 38.300 [16] |
| "NG_RAN_ MOBILITY_OPTIMIZATION" | Indicates that the MDA type for the Mobility Optimization defined in TS 38.300 [16] |

4th Change

7.5.1	Attribute properties

Table 7.5.1-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| mLModelId | It identifies the ML model. It is unique in each MnS producer. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| candidateTrainingDataSource | It provides the address(es) of the candidate training data source provided by MnS consumer. The detailed training data format is vendor specific. allowedValues: N/A. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| aIMLInferenceName | It indicates the type of inference that the ML model supports. allowedValues: see clause 7.4.X | type: AIMLInferenceName multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mDAType | It indicates the type of inference that the ML model for MDA supports. The detailed definition and corresponding allowed values for mDAType see TS 28.104 [2]. | type: MDAType (TS 28.104 [2]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nwdafAnalyticsType | It indicates the type of inference that the ML model for NWDAF supports. The detailed definition and corresponding allowed values for nwdafAnalyticsID see NwdafEvent in TS 29.520 [X]. | type: NwdafEvent (TS 29.520 [X]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ngRanInferenceType | It indicates the type of inference that the ML model for NG-RAN supports. The detailed definition and corresponding allowed values for ngRanInferenceType see clause 7.4a.1 | type: NgRanInferenceType multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| vSExtensionType | It indicates the type of inference that is vendor's specific extension. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| areConsumerTrainingDataUsed | It indicates whether the consumer provided training data have been used for the ML model training. allowedValues: ALL, PARTIALLY, NONE. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| usedConsumerTrainingData | It provides the address(es) where lists of the consumer-provided training data are located, which have been used for the ML model training. allowedValues: N/A. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| trainingRequestRef | It is the DN(s) of the related MLTrainingRequest MOI(s). allowedValues: DN. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| trainingProcessRef | It is the DN(s) of the related MLTrainingProcess MOI(s) that produced the MLTrainingReport. allowedValues: DN. | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| trainingReportRef | It is the DN of the MLTrainingReport MOI that represents the reports of the ML model training. allowedValues: DN. | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| lastTrainingRef | It is the DN of the MLTrainingReport MOI that represents the reports for the last training of the ML model. allowedValues: DN. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| modelConfidenceIndication | It indicates the average confidence value (in unit of percentage) that the ML model would perform for inference on the data with the same distribution as training data. Essentially, this is a measure of degree of the convergence of the trained ML model. allowedValues: { 0..100 }. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| trainingRequestSource | It describes the model that requested to instantiate the MLTrainingRequest MOI. This attribute can be of type String or DN. | type: <<CHOICE>> multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLTrainingRequest.requestStatus | It describes the status of a particular ML model training request. allowedValues: NOT_STARTED, IN_PROGRESS, CANCELLING, SUSPENDED, FINISHED, and CANCELLED. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLTrainingProcessId | It identifies the training process. It is unique in each instantiated process in the MnS producer. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| priority | It indicates the priority of the training process. The priority may be used by the ML model training to schedule the training processes. Lower value indicates a higher priority. allowedValues: { 0..65535 }. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| terminationConditions | It indicates the conditions to be considered by the ML training MnS producer to terminate a specific training process. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| progressStatus | It indicates the status of the process. allowedValues: N/A. | type: ProcessMonitor multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLUpdateProcess.cancelProcess | It allows the ML update MnS consumer to cancel the ML update process. Setting this attribute to "TRUE" cancels the ML update process. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLUpdateProcess.suspendProcess | It allows the ML update MnS consumer to suspend the ML update process. Setting this attribute to "TRUE" suspends the ML update process. The process can be resumed by setting this attribute to “FALSE” when it is suspended. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| mLModelVersion | It indicates the version number of the ML model. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| performanceRequirements | It indicates the expected performance for a trained ML model when performing on the training data. allowedValues: N/A. | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| modelPerformanceTraining | It indicates the performance score of the ML model when performing on the training data. allowedValues: N/A. | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| MLTrainingProcess.progressStatus.progressStateInfo | It provides the following specialization for the “progressStateInfo“ attribute of the “ProcessMonitor“ data type for the “MLTrainingProcess.progressStatus“. When the ML model training is in progress, and the " mLTrainingProcess.progressStatus.status " is equal to "RUNNING", it provides the more detailed progress information. allowedValues for " mLTrainingProcess.progressStatus.status " = "RUNNING": - “COLLECTING_DATA” - “PREPARING_TRAINING_DATA” - “TRAINING” + DN of the MLModel being trained The allowed values for " mLTrainingProcess.progressStatus.status " = "CANCELLING" are vendor specific. The allowed values for " mLTrainingProcess.progressStatus.status " = "NOT_STARTED" are vendor specific. | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| inferenceOutputName | It indicates the name of an inference output of an ML model. allowedValues: the name of the MDA output IEs (see 3GPP TS 28.104 [2]), name of analytics output IEs of NWDAF (see TS 23.288 [3]), RAN inference output IE name(s), and vendor's specific extensions. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| performanceMetric | It indicates the performance metric used to evaluate the performance of an ML model, e.g. "accuracy", "precision", "F1 score", etc. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| performanceScore | It indicates the performance score (in unit of percentage) of an ML model when performing inference on a specific data set (Note). The performance metrics may be different for different kinds of ML models depending on the nature of the model. For instance, for numeric prediction, the metric may be accuracy; for classification, the metric may be a combination of precision and recall, like the "F1 score". allowedValues: { 0..100 }. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLTrainingRequest.cancelRequest | It allows the ML training MnS consumer to cancel the ML model training request. Setting this attribute to "TRUE" cancels the ML model training request. The request can be resumed by setting this attribute to "FALSE" when it is suspended. Cancellation is possible when the requestStatus is the "NOT_STARTED", " IN_PROGRESS", and "SUSPENDED" state. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLTrainingRequest.suspendRequest | It allows the ML training MnS consumer to suspend the ML model training request. Setting this attribute to "TRUE" suspends the ML model training process. Suspension is possible when the requestStatus is not the "FINISHED" state. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLTrainingProcess.cancelProcess | It allows the ML training MnS consumer to cancel the ML model training process. Setting this attribute to “TRUE“ cancels the ML model training process. Cancellation is possible when the “mLTrainingProcess.progressStatus.status“ is not the “FINISHED“ state. Setting the attribute to “FALSE“ has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLTrainingProcess.suspendProcess | It allows the ML training MnS consumer to suspend the ML model training process. Setting this attribute to "TRUE" suspends the ML model training process. The process can be resumed by setting this attribute to “FALSE” when it is suspended. Suspension is possible when the " mLTrainingProcess.progressStatus.status" is not the "FINISHED", "CANCELLING" or "CANCELLED" state. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| inferenceModelRef | It describes the target entities that will use the ML model for inference. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| dataProviderRef | It describes the entities that have provided or should provide data needed by the ML model e.g. for training or inference | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| areNewTrainingDataUsed | It indicates whether the other new training data have been used for the ML model training. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| trainingDataQualityScore | It indicates numerical value that represents the dependability/quality of a given observation and measurement type. The lowest value indicates the lowest level of dependability of the data, i.e. that the data is not usable at all. allowedValues: { 0..100 }. | type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| decisionConfidenceScore | It is the numerical value that represents the dependability/quality of a given decision generated by the AI/ML inference function. The lowest value indicates the lowest level of dependability of the decisions, i.e. that the data is not usable at all. allowedValues: { 0..100 }. | type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| expectedRuntimeContext | This describes the context where an MLModel is expected to be applied. allowedValues: N/A | type: MLContext multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| trainingContext | This specifies the context under which the MLModel has been trained. allowedValues: N/A | type: MLContext multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| runTimeContext | This specifies the context where the MLmodel or model is being applied. allowedValues: N/A | type: MLContext multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLTrainingRequest.mLModelRef | It identifies the DN of the MLModel requested to be trained. allowedValues: DN | type: DN multiplicity: 0..1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| MLTrainingReport.mLModelGeneratedRef | It identifies the DN of the MLModel generated by the ML training. allowedValues: DN | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLModelRepositoryRef | It identifies the DN of the MLModelRepository. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLRepositoryId | It indicates the unique ID of the ML repository. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| modelPerformanceValidation | It indicates the performance score of the ML model when performing on the validation data. allowedValues: N/A | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| dataRatioTrainingAndValidation | It indicates the ratio (in terms of quantity of data samples) of the training data and validation data used during the training and validation process. It is represented by the percentage of the validation data samples in the total training data set (including both training data samples and validation data samples). The value is an integer reflecting the rounded number of percent * 100. allowedValues: { 0 .. 100 }. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLTestingRequest.requestStatus | It describes the status of a particular ML testing request. allowedValues: NOT_STARTED, IN_PROGRESS, CANCELLING, SUSPENDED, FINISHED, and CANCELLED. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLTestingRequest.cancelRequest | It allows the ML testing MnS consumer to cancel the ML testing request. Setting this attribute to "TRUE" cancels the ML testing request. Cancellation is possible when the requestStatus is the "NOT_STARTED", " IN_PROGRESS", and "SUSPENDED" state. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLTestingRequest.suspendRequest | It allows the ML testing MnS consumer to suspend the ML testing request. Setting this attribute to "TRUE" suspends the ML testing request. The request can be resumed by setting this attribute to “FALSE” when it is suspended. Suspension is possible when the requestStatus is not the "FINISHED" state. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLTestingRequest.mLModelRef | It identifies the DN of the MLModel requested to be tested. AllowedValues: DN | type: DN Multiplicity: 0..1 isOrdered: Falso isUnique: True defaultValue: None isNullable: True |
| modelPerformanceTesting | It indicates the performance score of the ML model when performing on the testing data. allowedValues: N/A. | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mLTestingResult | It provides the address where the testing result (including the inference result for each testing data example) is provided. The detailed testing result format is vendor specific. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| testingRequestRef | It identifies the DN of the MLTestingRequest MOI. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| supportedPerformanceIndicators | This parameter lists specific PerformanceIndicator(s) of an ML model. allowedValues: N/A. | type: SupportedPerfIndicator multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| performanceIndicatorName | It indicates the identifier of the specific performance indicator. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| isSupportedForTraining | It indicates whether the specific performance indicator is supported a performance metric of ML model training for the ML model. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| isSupportedForTesting | It indicates whether the specific performance indicator is supported a performance metric of ML model testing for the ML model. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| mLUpdateProcessRef | It is the DN of the mLUpdateProcess MOI that represents the process of updating an ML model. allowedValues: DN. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLUpdateRequestRefList | It is the list of DN of the MLUpdateRequest MOI that represents an ML update request. allowedValues: DN. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mLUpdateReportRef | It is the DN of the MLUpdateReport MOI that represents an ML update report. allowedValues: DN. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLUpdateReportingPeriod | It specifies the time duration upon which the MnS consumer expects the ML update is reported. | type: TimeWindow multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| availMLCapabilityReport | It represents the available ML capabilities. allowedValues: N/A. | type: AvailMLCapabilityReport multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| updatedMLCapability | It represents the updated ML capabilities. allowedValues: N/A. | type: AvailMLCapabilityReport multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| availMLCapabilityReportID | It identifies the available ML capability report. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| newCapabilityVersionId | It indicates the specific version of AI/ML capabilities to be applied for the update. It is typically the one indicated by the MLCapabilityVersionID in a newCapabilityVersion | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mlCapabilityVersionId | It indicates the version of ML capabilities that is available for the update. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| performanceGainThreshold | It defines the minimum performance gain as a percentage that shall be achieved with the capability update, i.e., the difference in the performances between the existing capabilities and the new capabilities should be at least performanceGainThreshold otherwise the new capabilities should not be applied. Allowed value: float between 0.0 and 100.0 | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| expectedPerformanceGains | It indicates the expected performance gain if/when the AI/ML capabilities of the respective network function are updated with/to the specific set of newly available AI/ML capabilities. | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| updateTimeDeadline | It indicates the maximum as stated in the MLUpdate request that should be taken to complete the update | type: TimeWindow multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLUpdateReport.mLModelModelRef | It indicates the DN of MLModel instances that can be updated. | type: DN multiplicity: 1 .. * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| MLUpdateRequest.requestStatus | It describes the status of a particular ML update request. allowedValues: NOT_STARTED, IN_PROGRESS, CANCELLING, SUSPENDED, FINISHED, and CANCELLED. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLUpdateRequest.cancelRequest | It allows the MnS consumer to cancel the ML update request. Setting this attribute to "TRUE" cancels the ML update request. Cancellation is possible when the requestStatus is the "NOT_STARTED", " IN_PROGRESS", and "SUSPENDED" state. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLUpdateRequest.suspendRequest | It allows the MnS consumer to suspend the ML update request. Setting this attribute to "TRUE" suspends the ML update request. The request can be resumed by setting this attribute to “FALSE” when it is suspended. Suspension is possible when the requestStatus is not the "FINISHED" state. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| memberMLModelRefList | It identifies the list of member ML models within a level of an ML model coordination group. allowedValues: DN list | type: DN multiplicity: 2..* isOrdered: True isUnique: True defaultValue: None isNullable: False |
| MLTrainingRequest.mLModelCoordinationGroupRef | It identifies the DN of the MLModelCoordinationGroup requested to be trained. allowedValues: DN | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLTrainingReport.mLModelCoordinationGroupGeneratedRef | It identifies the DN of the MLModelCoordinationGroup generated by ML training. AllowedValues: DN | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| MLTestingRequest.mLModelCoordinationGroupRef | It identifies the DN of the MLEntityCoordinationGroup requested to be tested. AllowedValues: DN | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| retrainingEventsMonitorRef | It indicates the DN of the ThresholdMonitor MOI that indicates the performance measurements and its corresponding thresholds to be used by MnS producer to initiate the re-training of the MLModel. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sourceTrainedMLModelRef | It identifies the DN of the source trained MLModel whose copy has been loaded from the ML model repository to the inference function. allowedValues: DN | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| MLModelLoadingRequest.requestStatus | It describes the status of a particular ML model loading request. allowedValues: NOT_STARTED, IN_PROGRESS, CANCELLING, SUSPENDED, FINISHED, and CANCELLED. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLModelLoadingRequest.cancelRequest | It allows the MnS consumer to cancel the ML model loading request. Setting this attribute to "TRUE" cancels the ML model loading. Cancellation is possible when the requestStatus is the "NOT_STARTED", " IN_PROGRESS", and "SUSPENDED" state. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLModelLoadingRequest.suspendRequest | It allows the MnS consumer to suspend the ML model loading request. Setting this attribute to "TRUE" suspends the ML model loading request. The request can be resumed by setting this attribute to “FALSE” when it is suspended. Suspension is possible when the requestStatus is not the "FINISHED" state. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| mLModelToLoadRef | It identifies the DN of a trained MLModel requested to be loaded to the target inference function(s). | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| policyForLoading | It provides the policy for controlling ML model loading triggered by the MnS producer. This policy contains two thresholds in the thresholdList attribute. The first threshold is related to the ML model to be loaded, and the second threshold is related to the existing ML model being used for inference. | type: AIMLManagementPolicy multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| thresholdList | It provides the list of threshold. | type: ThresholdInfo multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| MLModelLoadingProcess.progressStatus.progressStateInfo | It provides the following specialization for the "progressStateInfo" attribute of the "ProcessMonitor" data type for the "MLModelLoadingProcess.progressStatus". When the ML model loading is in progress, and the " MLModelLoadingProcess.progressStatus.status " is equal to "RUNNING", it provides the more detailed progress information. allowedValues for " MLModelLoadingProcess.progressStatus.status " = "RUNNING": The allowed values for " MLModelLoadingProcess.progressStatus.status " = "CANCELLING" are vendor specific. The allowed values for " MLModelLoadingProcess.progressStatus.status " = "NOT_STARTED" are vendor specific. | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLModelLoadingProcess.cancelProcess | It allows the MnS consumer to cancel the ML model loading process. Setting this attribute to "TRUE" cancels the process. Cancellation is possible when the "MLModelLoadingProcess.progressStatus.status" is not the "FINISHED" state. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLModelLoadingProcess.suspendProcess | It allows the MnS consumer to suspend the ML model loading process. Setting this attribute to "TRUE" suspends the process. The process can be resumed by setting this attribute to "FALSE" when it is suspended. Suspension is possible when the "MLModelLoadingProcess.progressStatus.status" is not the "FINISHED", "CANCELLING" or "CANCELLED" state. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| mLModelLoadingRequestRef | It identifies the DN of the associated MLModelLoadingRequest. allowedValues: DN. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| mLModelLoadingPolicyRef | It identifies the DN of the associated MLModelLoadingPolicyRef. allowedValues: DN. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| loadedMLModelRef | It identifies the DN of the MLModel that has been loaded to the inference function. allowedValues: DN | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| activationStatus | It describes the activation status. allowedValues: ACTIVATED, DEACTIVATED. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| AIMLManagementPolicy.managedActivationScope | It provides a list of sub scopes for which ML inference is activated as triggered by a policy on the MnS producer. For example, the sub scopes may be a list of cells or of geographical areas. The list is an ordered list indicating the inference is activated for the first sub scope and gradually extended to the next sub scope if the policy evaluates to true. allowedValues: N/A | type: ManagedActivationScope multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| AIMLInferenceFunction.managedActivationScope | It provides a list of sub scopes for which ML inference is activated as triggered by a policy on the MnS producer. For example, the sub scopes may be a list of cells or of geographical areas. The list is an ordered list indicating the inference is activated for the first sub scope and gradually extended to the next sub scope if the policy evaluates to true. allowedValues: N/A | type: AIMLManagementPolicy multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ManagedActivationScope.dNList | It indicates the list of DN, the list is an ordered list indicating the inference is activated for the first sub scope and gradually extended to the next sub scope. allowedValues: N/A | type: DN multiplicity: * isOrdered: True isUnique: True defaultValue: None isNullable: False |
| ManagedActivationScope.timeWindow | It indicates the list of time window; the list is an ordered list indicating the inference is activated for the first sub scope and gradually extended to the next sub scope. allowedValues: N/A | type: TimeWindow multiplicity: * isOrdered: True isUnique: True defaultValue: None isNullable: False |
| ManagedActivationScope.geoPolygon | It indicates the list of GeoArea, the list is an ordered list indicating the inference is activated for the first sub scope and gradually extended to the next sub scope. allowedValues: N/A | type: GeoArea multiplicity: * isOrdered: True isUnique: True defaultValue: None isNullable: False |
| usedByFunctionRefList | It provides the DNs of the functions supported by the AIMLInferenceFunction. allowedValues: N/A | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| inferenceOutputId | It identifies an inference output within an AIMLinferenceReport. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| inferenceOutputs | It indicates the Outputs that have been derived by the AIMLInferenceFunction instance from a specific ML model. Each ML model, inferenceOutputs may be a set of values. allowedValues: N/A. | type: InferenceOutput multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| inferencePerformance | It indicates the performance score of the ML model during Inference. allowedValues: N/A. | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| inferenceOutputTime | It indicates the time at which the inference output is generated. allowedValues: N/A | type: DateTime multiplicity: * isOrdered: True isUnique: True defaultValue: None isNullable: False |
| outputResult | It indicates the result of an inference. | type: AttributeValuePair multiplicity: * isOrdered: False isUnique: True defaultValue: Null isNullable: False |
| mLCapabilitiesInfoList | It indicates information about what an ML model can generate inference for. allowedValues: N/A. | type: MLCapabilityInfo multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| capabilityName | It indicates the name of a capability for which an ML model can generate inference. The capability is defined by Mns producer which can be traffic analysis capability, coverage analysis capability, mobility analysis capability or vendor specific extensions. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLCapabilityParameters | It indicates a set of optional parameters that apply for an aIMLInferenceName and capabilityName. allowedValues: N/A | type: AttributeValuePair multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| aIMLInferenceReportRefList | It indicates a list of DN of the MLInferenceReport MOI that represents an ML inference report. allowedValues: DN. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |  |
| mLModelRefList | It identifies the list of MLModel DN. allowedValues: DN list | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |  |
| NOTE: When the performanceScore is to indicate the performance score for ML model training, the data set is the training data set. When the performanceScore is to indicate the performance score for ML validation, the data set is the validation data set. When the performanceScore is to indicate the performance score for ML model testing, the data set is the testing data set. |  |  |

5th Change

*** OpenAPI/TS28105_AiMlNrm.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: AI/ML NRM

version: 18.5.0

description: >-

OAS 3.0.1 specification of the AI/ML NRM

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.105; AI/ML Management

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.105/

paths: {}

components:

schemas:

#-------- Definition of types-----------------------------------------------------

MLContext:

type: object

properties:

inferenceModelRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

dataProviderRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

RequestStatus:

type: string

enum:

- NOT_STARTED

- IN_PROGRESS

- SUSPENDED

- FINISHED

- CANCELLED

- CANCELLING

ModelPerformance:

type: object

properties:

inferenceOutputName:

type: string

performanceMetric:

type: string

performanceScore:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Float'

decisionConfidenceScore:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Float'

ProcessMonitor:

description: >-

This data type is the "ProcessMonitor" data type defined in “genericNrm.yaml”

with specialisations for usage in TS 28.105.

type: object

properties:

status:

type: string

progressPercentage:

type: integer

minimum: 0

maximum: 100

progressStateInfo:

type: string

resultStateInfo:

type: string

AIMLManagementPolicy:

description: >-

This data type represents the properties of a policy for AI/ML management.

type: object

properties:

thresholdList:

type: array

items:

$ref: 'TS28623_ThresholdMonitorNrm.yaml#/components/schemas/ThresholdInfo'

managedActivationScope:

$ref: '#/components/schemas/ManagedActivationScope'

SupportedPerfIndicator:

type: object

properties:

performanceIndicatorName:

type: string

isSupportedForTraining:

type: boolean

isSupportedForTesting:

type: boolean

ManagedActivationScope:

oneOf:

- type: object

properties:

dNList:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

- type: object

properties:

timeWindow:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeWindow'

- type: object

properties:

geoPolygon:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/GeoArea'

MLCapabilityInfo:

type: object

properties:

aIMLInferenceName:

$ref: '#/components/schemas/AIMLInferenceName'

capabilityName:

type: string

mLCapabilityParameters:

description: A map (list of key-value pairs) for an aIMLInferenceName and capabilityName

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

AvailMLCapabilityReport:

type: object

properties:

availMLCapabilityReportID:

type: string

mLCapabilityVersionId:

type: array

items:

type: string

expectedPerformanceGains:

type: array

items:

$ref: '#/components/schemas/ModelPerformance'

mLModelRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

InferenceOutput:

type: object

properties:

inferenceOutputId:

type: array

items:

type: string

aIMLInferenceName:

$ref: '#/components/schemas/AIMLInferenceName'

inferenceOutputTime:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

# FIXME, isOrder/isUnique both as True

inferencePerformance:

$ref: '#/components/schemas/ModelPerformance'

outputResult:

description: A map (list of key-value pairs) for Inference result name and it's value

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

AIMLInferenceName:

oneOf:

- $ref: 'TS28104_MdaNrm.yaml#/components/schemas/MDAType'

- $ref: '#/components/schemas/NwdafAnalyticsType'

- $ref: '#/components/schemas/NgRanInferenceType'

- $ref: '#/components/schemas/VSExtensionType'

NwdafAnalyticsType:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NwdafEvent'

NgRanInferenceType:

type: string

enum:

- NG_RAN_NETWORK_ENERGY_SAVING

- NG_RAN_LOAD_BALANCING

- NG_RAN_ MOBILITY_OPTIMIZATION

VSExtensionType:

type: string

#-------- Definition of types for name-containments ------

SubNetwork-ncO-AiMlNrm:

type: object

properties:

MLTrainingFunction:

$ref: '#/components/schemas/MLTrainingFunction-Multiple'

MLTestingFunction:

$ref: '#/components/schemas/MLTestingFunction-Multiple'

MLModelRepository:

$ref: '#/components/schemas/MLModelRepository-Multiple'

MLUpdateFunction:

$ref: '#/components/schemas/MLUpdateFunction-Multiple'

AIMLInferenceFunction:

$ref: '#/components/schemas/AIMLInferenceFunction-Multiple'

AIMLInferenceEmulationFunction:

$ref: '#/components/schemas/AIMLInferenceEmulationFunction-Multiple'

ManagedElement-ncO-AiMlNrm:

type: object

properties:

MLTrainingFunction:

$ref: '#/components/schemas/MLTrainingFunction-Multiple'

MLTestingFunction:

$ref: '#/components/schemas/MLTestingFunction-Multiple'

MLModelRepository:

$ref: '#/components/schemas/MLModelRepository-Multiple'

MLUpdateFunction:

$ref: '#/components/schemas/MLUpdateFunction-Multiple'

AIMLInferenceFunction:

$ref: '#/components/schemas/AIMLInferenceFunction-Multiple'

AIMLInferenceEmulationFunction:

$ref: '#/components/schemas/AIMLInferenceEmulationFunction-Multiple'

#-------- Definition of concrete IOCs --------------------------------------------

MLTrainingFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

mLModelRepositoryRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

MLTrainingRequest:

$ref: '#/components/schemas/MLTrainingRequest-Multiple'

MLTrainingProcess:

$ref: '#/components/schemas/MLTrainingProcess-Multiple'

MLTrainingReport:

$ref: '#/components/schemas/MLTrainingReport-Multiple'

ThresholdMonitors:

$ref: 'TS28623_ThresholdMonitorNrm.yaml#/components/schemas/ThresholdMonitor-Multiple'

MLTestingRequest:

$ref: '#/components/schemas/MLTestingRequest-Multiple'

MLTestingReport:

$ref: '#/components/schemas/MLTestingReport-Multiple'

MLTrainingRequest-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

aIMLInferenceName:

$ref: '#/components/schemas/AIMLInferenceName'

candidateTrainingDataSource:

type: array

items:

type: string

trainingDataQualityScore:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Float'

trainingRequestSource:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

requestStatus:

$ref: '#/components/schemas/RequestStatus'

expectedRuntimeContext:

$ref: '#/components/schemas/MLContext'

performanceRequirements:

type: array

items:

$ref: '#/components/schemas/ModelPerformance'

cancelRequest:

type: boolean

suspendRequest:

type: boolean

mLModelRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

mLModelCoordinationGroupRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

MLTrainingProcess-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

priority:

type: integer

terminationConditions:

type: string

progressStatus:

$ref: '#/components/schemas/ProcessMonitor'

cancelProcess:

type: boolean

suspendProcess:

type: boolean

trainingRequestRef: ## Figure 7.3a.1.1.1-1 has no such pointer

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

trainingReportRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

mLModelGeneratedRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

mLModelRef:  ## Figure 7.3a.1.1.1-1 is 1-0..1 mapping, hence should be single

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

MLTrainingReport-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

usedConsumerTrainingData:

type: array

items:

type: string

modelConfidenceIndication:

type: integer

modelPerformanceTraining:

type: array

items:

$ref: '#/components/schemas/ModelPerformance'

modelPerformanceValidation:

type: array

items:

$ref: '#/components/schemas/ModelPerformance'

dataRatioTrainingAndValidation:

type: integer

areNewTrainingDataUsed:

type: boolean

trainingRequestRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

trainingProcessRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

lastTrainingRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

mLModelGeneratedRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

mLModelCoordinationGroupGeneratedRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

mLModelRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

MLTestingFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:  ##FIXME pointer to MLModelCoordinationGroup missing

mLModelRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

MLTestingRequest:

$ref: '#/components/schemas/MLTestingRequest-Multiple'

MLTestingReport:

$ref: '#/components/schemas/MLTestingReport-Multiple'

MLTestingRequest-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

requestStatus:

$ref: '#/components/schemas/RequestStatus'

cancelRequest:

type: boolean

suspendRequest:

type: boolean

mLModelRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

mLModelCoordinationGroupRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

MLTestingReport-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

modelPerformanceTesting:

type: array

items:

$ref: '#/components/schemas/ModelPerformance'

mLTestingResult:

type: string

testingRequestRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

MLModelLoadingRequest-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

requestStatus:

$ref: '#/components/schemas/RequestStatus'

cancelRequest:

type: boolean

suspendRequest:

type: boolean

mLModelToLoadRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

MLModelLoadingPolicy-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

aIMLInferenceName:

$ref: '#/components/schemas/AIMLInferenceName'

policyForLoading:

$ref: '#/components/schemas/AIMLManagementPolicy'

mLModelRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

MLModelLoadingProcess-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

progressStatus:

$ref: '#/components/schemas/ProcessMonitor'

cancelProcess:

type: boolean

suspendProcess:

type: boolean

mLModelLoadingRequestRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

mLModelLoadingPolicyRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

loadedMLModelRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

MLModel-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

mLModelId:

type: string

aIMLInferenceName:

$ref: '#/components/schemas/AIMLInferenceName'

mLModelVersion:

type: string

expectedRunTimeContext:

$ref: '#/components/schemas/MLContext'

trainingContext:

$ref: '#/components/schemas/MLContext'

runTimeContext:

$ref: '#/components/schemas/MLContext'

supportedPerformanceIndicators:

$ref: '#/components/schemas/SupportedPerfIndicator'

mLCapabilitiesInfoList:

type: array

items:

$ref: '#/components/schemas/MLCapabilityInfo'

retrainingEventsMonitorRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

sourceTrainedMLModelRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

aIMLInferenceReportRefList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

usedByFunctionRefList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

MLModelRepository-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

MLModel:

$ref: '#/components/schemas/MLModel-Multiple'

MLModelCoordinationGroup:

$ref: '#/components/schemas/MLModelCoordinationGroup-Multiple'

MLModelCoordinationGroup-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

memberMLModelRefList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

## 7.3a.4.1 IOC

MLUpdateFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

availMLCapabilityReport:

$ref: '#/components/schemas/AvailMLCapabilityReport'

mLModelRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

MLUpdateRequest:

$ref: '#/components/schemas/MLUpdateRequest-Multiple'

MLUpdateProcess:

$ref: '#/components/schemas/MLUpdateProcess-Multiple'

MLUpdateReport:

$ref: '#/components/schemas/MLUpdateReport-Multiple'

MLUpdateRequest-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

performanceGainThreshold:

type: array

items:

$ref: '#/components/schemas/ModelPerformance'

newCapabilityVersionId:

type: array

items:

type: string

updateTimeDeadline:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeWindow'

requestStatus:

$ref: '#/components/schemas/RequestStatus'

mLUpdateReportingPeriod:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeWindow'

cancelRequest:

type: boolean

suspendRequest:

type: boolean

mLUpdateProcessRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

mLModelRefList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

MLUpdateProcess-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

progressStatus:

$ref: '#/components/schemas/ProcessMonitor'

cancelProcess:

type: boolean

suspendProcess:

type: boolean

mLModelRefList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

mLUpdateRequestRefList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

mLUpdateReportRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

MLUpdateReport-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

updatedMLCapability:

$ref: '#/components/schemas/AvailMLCapabilityReport'

mLModelRefList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

mLUpdateProcessRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

AIMLInferenceFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

activationStatus:

type: string

enum:

- ACTIVATED

- DEACTIVATED

managedActivationScope:

$ref: '#/components/schemas/AIMLManagementPolicy'

usedByFunctionRefList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

mLModelRefList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

AIMLInferenceReport:

$ref: '#/components/schemas/AIMLInferenceReport-Multiple'

MLModelLoadingRequest:

$ref: '#/components/schemas/MLModelLoadingRequest-Multiple'

MLModelLoadingProcess:

$ref: '#/components/schemas/MLModelLoadingProcess-Multiple'

MLModelLoadingPolicy:

$ref: '#/components/schemas/MLModelLoadingPolicy-Multiple'

MLModel:

$ref: '#/components/schemas/MLModel-Multiple'

AIMLInferenceReport-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

inferenceOutputs:

type: array

items:

$ref: '#/components/schemas/InferenceOutput'

minItems: 1

mLModelRefList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

AIMLInferenceEmulationFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

AIMLInferenceReport:

$ref: '#/components/schemas/AIMLInferenceReport-Multiple'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-ncO'

#-------- Definition of JSON arrays for name-contained IOCs ----------------------

MLTrainingFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/MLTrainingFunction-Single'

MLTrainingRequest-Multiple:

type: array

items:

$ref: '#/components/schemas/MLTrainingRequest-Single'

MLTrainingProcess-Multiple:

type: array

items:

$ref: '#/components/schemas/MLTrainingProcess-Single'

MLTrainingReport-Multiple:

type: array

items:

$ref: '#/components/schemas/MLTrainingReport-Single'

MLModel-Multiple:

type: array

items:

$ref: '#/components/schemas/MLModel-Single'

MLModelRepository-Multiple:

type: array

items:

$ref: '#/components/schemas/MLModelRepository-Single'

MLModelCoordinationGroup-Multiple:

type: array

items:

$ref: '#/components/schemas/MLModelCoordinationGroup-Single'

MLTestingFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/MLTestingFunction-Single'

MLTestingRequest-Multiple:

type: array

items:

$ref: '#/components/schemas/MLTestingRequest-Single'

MLTestingReport-Multiple:

type: array

items:

$ref: '#/components/schemas/MLTestingRequest-Single'

MLModelLoadingRequest-Multiple:

type: array

items:

$ref: '#/components/schemas/MLModelLoadingRequest-Single'

MLModelLoadingProcess-Multiple:

type: array

items:

$ref: '#/components/schemas/MLModelLoadingProcess-Single'

MLModelLoadingPolicy-Multiple:

type: array

items:

$ref: '#/components/schemas/MLModelLoadingPolicy-Single'

MLUpdateFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/MLUpdateFunction-Single'

MLUpdateRequest-Multiple:

type: array

items:

$ref: '#/components/schemas/MLUpdateRequest-Single'

MLUpdateProcess-Multiple:

type: array

items:

$ref: '#/components/schemas/MLUpdateProcess-Single'

MLUpdateReport-Multiple:

type: array

items:

$ref: '#/components/schemas/MLUpdateReport-Single'

AIMLInferenceFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/AIMLInferenceFunction-Single'

AIMLInferenceReport-Multiple:

type: array

items:

$ref: '#/components/schemas/AIMLInferenceReport-Single'

AIMLInferenceEmulationFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/AIMLInferenceEmulationFunction-Single'

#-------- Definitions in TS 28.104 for TS 28.532 ---------------------------------

resources-AiMlNrm:

oneOf:

- $ref: '#/components/schemas/MLTrainingFunction-Single'

- $ref: '#/components/schemas/MLTrainingRequest-Single'

- $ref: '#/components/schemas/MLTrainingProcess-Single'

- $ref: '#/components/schemas/MLTrainingReport-Single'

- $ref: '#/components/schemas/MLModel-Single'

- $ref: '#/components/schemas/MLModelRepository-Single'

- $ref: '#/components/schemas/MLModelCoordinationGroup-Single'

- $ref: '#/components/schemas/MLTestingFunction-Single'

- $ref: '#/components/schemas/MLTestingRequest-Single'

- $ref: '#/components/schemas/MLTestingReport-Single'

- $ref: '#/components/schemas/MLModelLoadingRequest-Single'

- $ref: '#/components/schemas/MLModelLoadingProcess-Single'

- $ref: '#/components/schemas/MLModelLoadingPolicy-Single'

- $ref: '#/components/schemas/MLUpdateFunction-Single'

- $ref: '#/components/schemas/MLUpdateRequest-Single'

- $ref: '#/components/schemas/MLUpdateProcess-Single'

- $ref: '#/components/schemas/MLUpdateReport-Single'

- $ref: '#/components/schemas/AIMLInferenceFunction-Single'

- $ref: '#/components/schemas/AIMLInferenceReport-Single'

- $ref: '#/components/schemas/AIMLInferenceEmulationFunction-Single'

<CODE ENDS>

End of Changes