3GPP TSG-SA5 Meeting #158	S5-246401

Orlando, USA, 18 - 22 November 2024

# Source:	ZTE CORPORATION

# Title:	pCR TR 28.880 Potential solution for alternative option to obtain energy consumption of VNF/VNFC

# Document for:	Approval

Agenda Item:	6.19.20

1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

2	References

[1]		3GPP TR 28.880 1.1.0: Study on energy efficiency and energy saving aspects of 5G networks and services

3	Rationale

This contribution is a potential solution and evaluation for the use case Alternative option to obtain energy consumption of VNF/VNFC. This potential solution describes how to estimate the containerized VNF/VNFC.

4	Detailed proposal

This document proposes the following changes in TR 28.880 [1].

1st Change

5.2	Use case #2: Alternative option to obtain energy consumption of VNF/VNFC

5.2.1	Description

For Energy Consumption (EC) estimated measurement, VNF and VM-based VNFCs are supported as defined in clauses 6.7.3.1.2 and 6.7.3.1.3 of 3GPP TS 28.554 [2], respectively.. The way to calculate the estimated VM-based VNF/VNFC energy consumption requires obtaining the sum of the vCPU mean usage of all virtual compute resource instances running on the same NFVI Node during the same observation period, all separately provided by NFV MANO (see clause 7.1.2 of ETSI GS NFV-IFA 027 [6]).

The issue is that there is no easy way to obtain all virtual compute resource instances information running on the same NFVI Node and measure the vCPU, vMemory or vDisk mean usage for all of them. The measurement method as described in clause 7.1.2 of ETSI GS NFV-IFA 027 [6] does not support setting the measurement scope on the same NFVI Node. Some other mechanism to know all the virtual compute resource instances information running on the same NFVI Node is required.

In addition, method for obtaining energy consumption of VNF/VNFC in practical implementation is more complicated than the estimation method currently defined in in clauses 6.7.3.1.2 and 6.7.3.1.3 of 3GPP TS 28.554 [2] which is simply based on vCPU, vMemory, vDisk mean usage or I/O traffic volume. Diverse power measurement models and tools for obtaining the EC are possible depending on the operator deployment scenarios.

Alternative options for obtaining energy consumption of VNF/VNFC in addition to the existing estimation methods defined in clauses 6.7.3.1.2 and 6.7.3.1.3 of 3GPP TS 28.554 [2] are needed.

As described in clause 7.1.18 and clause 7.4.12 of ETSI GS NFV-IFA 027 [6], the actual energy consumption of virtual compute and OS container workload is provided. As an enhancement, this use case is to study:

a)	potential enhancements to existing Energy Consumption (EC) of VM-based VNF/VNFCs estimation method; and

b)	potential extension to Energy Consumption (EC) estimation of containerized VNF/VNFCs.

2nd Change

5.2.3	Potential solutions

5.2.3.2	Potential solution #2: <Estimate energy consumption of containerized VNF/VNFC>

5.2.3.2.1	Introduction

In this potential solution, various metrics are proposed to estimate the energy consumption of containerized VNF/VNFCs, along with a detailed calculation method. Given that the energy consumption of VM-based VNF/VNFCs is already provided in clause 6.7.3.1 of 3GPP TS 28.554[2]

5.2.3.1.2	Description

The estimated energy consumption of the containerized VNF is calculated by summing up the estimated energy consumption of each containerized VNFC that constitutes it.

ECcontainerized VNF, estimated=containerized VNFCECcontainerized VNFC, estimated

The estimated energy consumption of containerized VNFC is obtained by taking the estimated energy consumption of the OS container workload instance on which the VNFC runs.

ECcontainerized VNFC, estimated=ECOS container workload, estimated

The energy consumption of a OS container workload instance X is estimated as a proportion of the energy consumption of the NFVI node on which the OS container workload instance X runs. This proportion can be obtained by considering following four metrics:

a)	Estimated OS container workload energy consumption based on mean vCPU usage：

The proportion is obtained by dividing the vCPU mean usage of the OS container workload instance X, by the sum of the vCPU mean usage of all OS container workload instances running on the same NFVI Node as X, as defined by the equation below:

ECOS container workload, estimated, VCpuUsageMean=VCpuUsageMeanOS container workloadVCpuUsageMean∗ECNFVINode, measured

where:

- VCpuUsageMean is the mean virtual CPU usage measurement for the OS container workload instance during the observation period, provided by NFV MANO(see clause 7.4.2 of ETSI GS NFV-IFA 027 V5.2.1[6]);

- OS container workloadVCpuUsageMean is the sum of the mean virtual CPU usage of all OS container workload instances running on the same NFVI Node during the same observation period, all separately provided by NFV MANO;

- ECNFVINode, measured is the energy consumption of the NFVI node on which the OS container workload instance runs, measured during the same observation period, as per ETSI ES 202 336-12 [32].

b)	Estimated OS container workload energy consumption based on mean vMemory usage：

The proportion is obtained by dividing the vMemory mean usage of the OS container workload instance X, by the sum of the vMemory mean usage of all OS container workload instances running on the same NFVI Node as X, as defined by the equation below:

ECOS container workload, estimated, VMemoryUsageMean=VMemoryUsageMeanOS container workloadVMemoryUsageMean∗ECNFVINode, measured

where:

- VMemoryUsageMean is the mean virtual memory usage measurement for the OS container workload instance during the observation period, provided by NFV MANO(see clause 7.4.4 of ETSI GS NFV-IFA 027 V5.2.1[6]);

- OS container workloadVMemoryUsageMean is the sum of the mean virtual memory usage of all OS container workload instances running on the same NFVI Node during the same observation period, all separately provided by NFV MANO;

- ECNFVINode, measured is the energy consumption of the NFVI node on which the OS container workload instance runs, measured during the same observation period, as per ETSI ES 202 336-12 [32].

c)	Estimated OS container workload energy consumption based on mean vStorage Resource usage：

The proportion is obtained by dividing the vStorage Resource mean usage of the OS container workload instance X, by the sum of the vStorage Resource mean usage of all OS container workload instances running on the same NFVI Node as X, as defined by the equation below:

ECOS container workload, estimated, VDiskUsageMean

=VStorageResourceUsageMeanOS container workloadVStorageResourceUsageMean∗ECNFVINode, measured

where:

- VStorageResourceUsageMean is the mean virtual storage resource usage measurement for the OS container workload instance during the observation period, provided by NFV MANO(see clause 7.4.10 of ETSI GS NFV-IFA 027 V5.2.1[6]);

- OS container workloadVStorageResourceUsageMean is the sum of the mean virtual storage resource usage of all OS container workload instances running on the same NFVI Node during the same observation period, all separately provided by NFV MANO;

- ECNFVINode, measured is the energy consumption of the NFVI node on which the OS container workload instance runs, measured during the same observation period, as per ETSI ES 202 336-12 [32].

d)	Estimated OS container workload energy consumption based on I/O traffic volume：

The proportion is obtained by dividing the I/O traffic volume of the OS container workload instance X, by the sum of the I/O traffic volume of all OS container workload instances running on the same NFVI Node as X, as defined by the equation below:

ECOS container workload, estimated, IOTrafficVolume=IOTrafficVolumeOS container workloadIOTrafficVolume∗ECNFVINode, measured

where:

- IOTrafficVolume is the sum of the incoming and outgoing traffic volumes of the OS container workload instance during the observation period, provided by NFV MANO;

- Incoming traffic volume is obtained by measuring the number of incoming bytes on OS container workload(VNetByteIncoming - cf. clause 7.4.6 of [6]) during the observation period;

- Outgoing traffic volume is obtained by measuring the number of outgoing bytes on OS container  workload(VNetByteOutgoing - cf. clause 7.4.7 of [6]) during the observation period.

IOTrafficVolume=VNetByteIncoming+VNetByteOutcoming

- OS container workloadIOTrafficVolume is the sum of the  incoming and outgoing traffic volumes of all OS container workload instances running on the same NFVI Node during the same observation period, all separately provided by NFV MANO (see clause 7.4.6 and 7.4.7 of [6]);

- ECNFVINode, measured is the energy consumption of the NFVI node on which the OS container workload instance runs, measured during the same observation period, as per ETSI ES 202 336-12 [32].

3rd Change

5.2.4	Evaluation of potential solutions

The potential solution #1 provides different options to obtain the energy consumption of VNF/VNFC, the potential solution #2 addresses the current gap in providing specific estimation methods for energy consumption of containerized VNF/VNFC, by considering mean virtual CPU usage, mean memory usage, mean storage resource usage and I/O traffic of the OS container workload.

End of Changes