3GPP TSG-SA5 Meeting #158	S5-246515

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.623 | CR | 0474 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: compr ehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.623 Remove unneeded fileLocation attribute (stage 3) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson España S.A. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The attribute fileLocation needs to be updated, since type and description are wrong. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update fileLocation attribute in stage 3 (yang) |  |  |  |  |  |  |  |  |
| - a |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage-3 misaligned with stage-2. fileLocation attribute description is wrong |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Forge |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | x |  | O&M Specifications |  |  |  | TS 28.622 CR 0504 |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1469 at commit 4d5f840f699c8337b1d1a045d075572296a45761 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF CHANGE 1 ***

*** yang-models/_3gpp-common-filemanagement.yang ***

<CODE BEGINS>

module _3gpp-common-filemanagement {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-common-filemanagement";

prefix "filemgmt3gpp";

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-yang-extensions { prefix yext3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the FileDownloadJob Information Object

Class (IOC) that is part of the Generic Network Resource Model (NRM).

Copyright 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.623 Generic Network Resource Model (NRM)";

revision 2024-11-07 { reference CR-0474 ; }

revision 2023-09-18 { reference CR-0271 ; }

revision 2023-02-14 { reference CR-0234; }

revision 2022-10-24 { reference CR-0196;  }

revision 2022-02-10 { reference "Initial revision, S5-221757"; }

grouping FileDownloadProcessMonitor {

description "Provides specialisations of the ProcessMonitor datatype.";

uses types3gpp:ProcessMonitorGrp {

refine resultStateInfo {

description "If status is FAILED resultStateInfo will be one of

the following or empty.";

must '../status != "FAILED"

or . = "UNKNOWN"

or . = "NO_STORAGE"

or . = "LOW_MEMORY"

or . = "NO_CONNECTION_TO_REMOTE_SERVER"

or . = "FILE_NOT_AVAILABLE"

or . = "DNS_CANNOT_BE_RESOLVED"

or . = "TIMER_EXPIRED"

or . = "OTHER"

or . = "NULL" ';

}

refine status {

description "Status must not be 'PARTIALLY_FAILED'.";

must '. != "PARTIALLY_FAILED" ';

}

}

}

grouping FileDownloadJobGrp {

description "Represents the FileDownloadJob IOC.";

reference "3GPP TS 28.622";

uses top3gpp:Top_Grp;

leaf fileLocation {

type inet:uri;

yext3gpp:notNotifyable;

yext3gpp:inVariant;

description "Location of the file incl. the file transfer protocol,

and the file name for the case the file content cannot be retrieved

by reading the 'fileContent' attribute.

The allowed file transfer protocols are:

- sftp

- ftpes

- https

Examples:

'sftp://companyA.com/datastore/fileName.xml',

'https://companyA.com/ManagedElement=1/Files=1/File=1'

";

}

leaf notificationRecipientAddress {

description "Address of the notification recipient.";

type string;

yext3gpp:notNotifyable;

yext3gpp:inVariant;

}

leaf cancelJob {

description "Setting this attribute to TRUE cancels the file download job.

As specified in the definition of ProcessMonitor, cancellation is

possible in the NOT_STARTED and RUNNING state. Setting the attribute

to FALSE has no observable result.";

type boolean;

mandatory true;

}

list jobMonitor {

key "id";

description "Provides monitoring for the file download job.";

uses FileDownloadProcessMonitor;

}

}

grouping FileDownloadJobSubtree {

description "Helps augmenting FileDownloadJob into multiple places.";

list FileDownloadJob {

description "Specifies the FileDownloadJob IOC, see 3GPP TS 28.622.";

key "id";

uses top3gpp:Top_Grp;

container attributes {

uses FileDownloadJobGrp;

}

}

}

augment "/subnet3gpp:SubNetwork" {

uses FileDownloadJobSubtree;

}

augment "/me3gpp:ManagedElement" {

uses FileDownloadJobSubtree;

}

}

<CODE ENDS>

*** END OF CHANGE 1 ***