3GPP TSG-SA5 Meeting #158	S5-246565

Orlando, USA, 18 - 22 November 2024

# Source:	Huawei

# Title:	Solution on exposure of management services through the CAPIF framework

# Document for:	Approval

Agenda Item:	6.19.21

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]		3GPP TR 28.879 v1.1.0: Study on OAM for service management and exposure to external consumers

# 3	Rationale

This contribution proposes to add solution on exposure of management services through the CAPIF framework in TR 28.879 [1].

# 4	Detailed proposal

This document proposes the following changes in TR 28.879 [1].

1st Change

#### 5.1.0.3	Potential solutions

##### 5.1.0.3.1	Potential solution #1: EGMF implements the service specific aspect compliant with the CAPIF architecture

5.1.0.3.1.1	Introduction

The potential solution assumes that the 3GPP management system supports the capability to expose management services to external MnS consumers through the EGMF implementing the service specific aspect compliant with the CAPIF architecture.

5.1.0.3.1.2	Description

Figure 5.1.0.3.1.2-1 illustrates the deployment model where the EGMF implements the service specific aspect compliant with the CAPIF architecture.

Figure 5.1.0.3.1.2-1: EGMF implements the service specific aspect compliant with the CAPIF architecture

Operators can deploy the CAPIF core function along with the EGMF in 3GPP 5GS.

The EGMF can implement the functionalities of the API provider domain functions.

According to the CAPIF architecture, CAPIF-2 and CAPIF-2e consist of framework aspects and service specific aspects. The service specific aspects are out of scope of CAPIF. 3GPP SA5 MnS can implement the service specific aspects of CAPIF-2 and CAPIF-2e, and can provide the service APIs exposed by EGMF (AEF) to the SA5 MnS consumer (API invoker).

The EGMF can implement the CAPIF-3, CAPIF-4 CAPIF-5 reference point/interface to the CAPIF core function.

#### 5.1.0.4	Evaluation of potential solutions

The proposed solution satisfies the requirements PREQ-FS_MExpo-EGMF-01 and PREQ-FS_MExpo-EGMF-02. This is a feasible candidate as input to the normative phase.

End of change