3GPP TSG-SA5 Meeting #158	S5-246609

Orlando, USA, 18 - 22 November 2024

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.290 | CR | 0233 | rev | 2 | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 32.290 Correction on CHF as NF consumer detected failure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18, B2B_CH, CHRACHF |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The application level failure handling is now defined to support the CTF detected failure handling, with three enumeration values that specify how CTF handles the service in case of request times out to CHF. However, in terms of consumer CHF detected failure handling, the consumer CHF cannot decide directly whether the service shall continue or not, the above three values cannot be used to specify how consumer CHF handles failure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Clarify the application level failure handling specified for CTF detected failure handling. 2. Specify the application level failure handling when consumer CHF detected failure handling for inter-CHF communication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The failure handling mechanism is incomplete to support the failure handling for inter-CHF communication. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.1.1, 5.5.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 32.291 ... CR 0594... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-245589 (SA5#157). |  |  |  |  |  |  |  |  |

First change

5.5.1.1	CTF detected failure

The failure handling determines what to do if the sending of charging data request to the CHF, including retry handling as described in clause 5.5.2, without response in a period of time (request times out).

In the case of the NF consumer (CTF) towards CHF request times out, NF consumer (CTF) uses application level failure handling. Failure handling may be received from the CHF previously or may be locally configured. The value received from the CHF in the charging data response will always override any already existing value. The application level failure handling for NF consumer (CTF), as defined in TS 32.291 [58] clause 6.1.3.11, may include the following values:

-	TERMINATE: the service shall only be granted for as long as there is a connection between NF consumer and the CHF.

-	CONTINUE: the NF consumer should re-send and continue the request to an alternative server in the case of transport temporary failures, provided that a failover procedure is supported in the CHF and the NF consumer, and that an alternative server is available.  Otherwise, the service SHOULD be granted, and Charging Information may be stored, even if charging data request can't be delivered.

-	RETRY_AND_TERMINATE: the NF consumer should re-send the request to an alternative server in the case of transport temporary failures at the NF Consumer, provided that a failover procedure is supported in the CHF and NF consumer, and that an alternative server is available. Otherwise, the service should not be granted when the charging data request can't be delivered.

NOTE: In case of a RETRY, the NF Consumer should re-send the Charging Data Message Request to an alternative server in the case of transport temporary failures. Therefor this scenario can be used in case of a CONTINUE value. RETRY_AND_TERMINATE indicates the failure handling should lead to TERIMINATE after RETRY.

In case the CHF is determined not reachable, the CTF uses application level failure handling and may store Charging Data Request(s) or charging information. Once the connection to a CHF is established by the NF consumer (CTF), the CHF may receive the Charging Data Request(s) or charging information that were previously stored by NF consumer (CTF).

In case there is an application level error response from the CHF, NF consumer (CTF)  action will depend on the type of Application Error, as defined in TS 32.291 [58] clause 6.1.7.3 and clause 6.2.6.3.

For protocol level errors, refer to applicable protocol failure handling mechanisms as described in TS 32.291  [58] clause 6.1.7.2.

Next change

5.5.1.3	CHF as NF Consumer detected failure

When a Charging Data Request [Initial/Update/Termination] received by the consumer CHF for inter CHFs communication, in the case of the consumer CHF towards producer CHF request times out and without alternative producer CHF, consumer CHF uses application level failure handling. Failure handling may be received from the producer CHF previously or may be locally configured. The value received from the producer CHF in the charging data response will always override any already existing value. The application level failure handling for consumer CHF, as defined in TS 32.291  [58] clause 6.1.3.11, may include the following values:

-	CH_RELEASE: the consumer CHF should respond an error code (i.e. 504 Gateway Timeout) that can instruct NF consumer (CTF) to release the charging session.

-	CH_CONTINUE: the consumer CHF should keep the session continue with NF consumer (CTF), which is implementation specific.

End of change