3GPP TSG-SA5 Meeting #158 	S5-246627

Orlando, USA, 18 - 22 November 2024

# Source:	Huawei

# Title:	pCR TR 28.874 Correct the description of MO and MT

# Document for:	Approval

Agenda Item:	6.19.15

1	Decision/action requested

The group is asked to discuss and agree on the proposal.

2	References

[1]		3GPP TR 28.874: Study on management aspects of NTN – Phase 2

[2]		3GPP TR 23.700-29: Study on integration of satellite components in the 5G architecture; Phase 3

3	Rationale

According to the description in TR 23.700-29 [2], the MO data is initiated in UE while MT data is transferred to the UE:

“The MO data is stored in the MME-onboard when the service link is available and the feeder link is unavailable, and transferred to the ground when the feeder link becomes available. The MT data is stored in the MME-ground or in S-GW when the feeder link is unavailable and transferred to the MME-onboard when the feeder link becomes available. The MT data is stored in the MME-onboard when the feeder link is available and service link is unavailable, and transferred to the UE when service link becomes available. All types of data traffic (e.g. IP, etc.) can be supported and transferred using the existing user plane and control plane procedures defined in EPS.”

The description of MO/MT data flow direction is wrong.

4	Detailed proposal

This document proposes the following changes in TR 28.874 [1].

1st Change

5.3.1.1	Description

The use case and requirements of store and forward functionality in defined in [5]. The Store and Forward Satellite (S&F) operation in a 5G system with satellite access is intended to provide some level of communication service for UEs under satellite coverage with intermittent/temporary satellite connectivity (e.g. when the satellite is not connected via a feeder link or via ISL to the ground network) for delay-tolerant communication service. The concept of "S&F" service is widely used in the fields of delay-tolerant networking and disruption-tolerant networking.

The management of the S&F functionality need to be defined. The limitations on the size/amount of data that can be sent from the UE to the AF (Application Function, e.g. IoT devices) and vice versa need to be configured. Forwarding priority for the stored data to the ground station or to the UE and data retention period for the exchanged data should be configured. The acknowledgment can be provided for the received messages. The acknowledge policy may dictate that the acknowledgment should not be provided. Whether to acknowledgement of the received data by the satellite could be issued possibly with the additional information about the store and forward including (not limited to) estimated time to deliver the messages need to be configured.

The S&F functionality requires to store the messages, in case of MT[12] (mobile terminated), until the satellite coverage is available and the UE is connected to the network. This is when the stored messages are sent to the UE. Same goes for MO[12] (mobile originated) messages where UE messages are stored until the connection is established with the ground network (gateway) and messages can be delivered to the appropriate AF. Since the stored messages have to be read by the network entity at run time the format and the composition, needed to enforce the S&F delivery policies, of the stored message need to be defined.

End of changes