3GPP TSG-SA5 Meeting #158	S5-246724

Orlando, Florida, USA 18 - 22 November 2024

# Source:	Huawei

# Title:	Rel-19 pCR 28.866 Editorial improvements

# Document for:	Approval

Agenda Item:	6.19.2

# 1	Decision/action requested

For approval

# 2	References

[1] 	3GPP TR 28.866: “Study on Management Data Analytics (MDA) – Phase 3”.

# 3	Rationale

Editorial improvements to TR 28.866 [1].

1st change: Expand abbreviation EE to make the text more consistent. Fix error in grammar. Remove extra word.

2nd change: Insert missing space.

3rd change: Fix incorrect use case number.

4th change: Fix format of list.

# 4	Detailed proposal

It is proposed to make the following changes to TR 28.866 [1].

1st change

#### 5.1.2.1	Description

Energy efficiency can be achieved by moving certain cells to energy saving state (see TS 28.104 [2]). When moved to energy saving state, such cells or beams will have different impacts to the neighbour cells or beams depending on how much the coverage of those cells overlap.

RAN3 has a use case to configure unified mapping rule for a group of cells. This implies that a mapping rule related to energy efficiency is the same across a group of cells for a given area. It is desirable to use MDA analytics to get a group of cells that should be managed together for energy saving, computed, or learned depending on their proximity and likely impact on one another.

Then, for the cells in the group, it is desirable to use MDA analytics to get the sequence in which the beams can be activated/de-activated in order to maximize the throughput of the users while minimizing the energy consumption. Note that the sequence in which activation of the beams happen are necessarily not in the same sequence as deactivation. The sequence of activation/de-activation of the beams do matter in real time scenarios. For instance, there are 4 beams in a cell serving the same geo area at different directions. To maximize the energy efficiency, if it has been decided to switch two beams off for energy saving purposes, in general the sequence of the deactivation of the beams can be random or all the identified beams can be deactivated together. In both these cases, the sharing of load among the other beams and the handover performances between the beams would not be optimal. If all the identified beams are deactivated at the same time, the remaining beams would have to share the load of the beams those are deactivated at the same time. This would cause call drops and throughput impacts. Alternatively, if a random sequence is followed, the impact and the performance of the beams sharing the load of the deactivated beams would be random as well. To avoid this scenario, it is recommended to follow a sequence in which the beams are activated/deactivated.

RAN3 has identified the need for configuring a unified mapping rule among gNBs. These set of gNBs to which unified mapping rule to be applied form a group of cells that need to be managed together for energy efficiency purposes as shown in the below figure.

Figure 5.1.2.1-1: Example of Energy Cost exchange between gNBs.

The enabling data of this use case may include Mobility Management related measurements (clause 5.1.1.6 from TS 28.552 [5]) including intra and inter gNB handovers. These also include handover measurements per beam (in clauses 5.1.1.6.1.12 and 5.1.1.6.1.13 from TS 28.552 [5]) and intra-NR cell SSB beam switch measurements (in clause 5.1.1.21 from TS 28.552 [5]).

2nd change

#### 5.1.2.3	Potential solutions

This potential solution proposes to extend the existing energy saving analytics for RAN to include the datatype that may contain the groups and the cells in each of the groups. RAN3 has already identified the need for the groups of cells that can be managed together and this recommendation of energy saving analytics proposes the optimal number of groups and the cells in each of those groups. The cells belonging to each group can be identified by the datatype NCGI (clause 5.3.212 from [13]).

The potential solution also proposes to extend the cell energy saving analytics for gNB to indicate the sequence in which the beams may be activated or de-activated. This recommendation can be proposed to gNB which can decide to implement the recommendation based on the local policies. The beams can be identified with the “beamIndex” (clause 4.3.40 from [13]).

3rd change

### 5.6.4	Use case 4: NF failure evaluation

4th change

#### 5.8.1.3	Potential solutions

The analytics output for MDA assisted control plane congestion analysis may be enhanced with the following attributes.

- rootCause. This attribute may indicate the root cause of control plane congestion issue, such as network failure, signalling overload, etc.

- predictedDuration. This attribute may indicate the predicted time duration of control plane congestion issue.

End of changes