3GPP TSG-SA5 Meeting #158 	S5-246726

Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

# Source:	ZTE Corporation

# Title:	add potential solution for MEC deployed on the satellite

# Document for:	Approval

Agenda Item:	6.19.15

# 1	Decision/action requested

The group is asked to discuss and approval.

# 2	References

# 3	Rationale

This contribution proposes to add potential solution for MEC deployed on the satellite.

# 4	Detailed proposal

Start of change

### 5.4.2	Use case #2: MEC deployed on the satellite

#### 5.4.2.1	Description

Mobile Edge Computing (MEC) migrates computing, storage, and service capabilities to the mobile network edge, enabling 5G networks to meet edge application service requirements such as high bandwidth, low latency, and high reliability.

In order to solve the problems of satellite link transmission delay and reduce the backhaul bandwidth, MEC server can be deployed on the satellite, so that the satellite has the capabilities of calculation and content distribution, and can process the service requests from the ground mobile users, which can not only effectively reduce service transmission delay of the satellite-to-earth link, but also effectively save the service data transmission bandwidth of the satellite-to-earth link. MEC deployment in the satellite is suitable for services with low mobility requirements but high latency requirements, in addition, for remote users without the support of ground network communication facilities can directly offload data to the satellite for processing.

The scenarios about edge computing via UPF deployed on the satellite is described in clauses 5.43 of TS 23.501 [1]. When the UE is accessing gNB deployed in satellite, the AMF selects special SMF which supports specific DNAI, the SMF can select the UPF deployed on the GEO satellite and enable PDU Session Establishment with PSA UPF on the satellite during the PDU Session Establishment procedure as described in TS 23.502 [2].

The 3GPP management system should manage the MEC deployed on the satellite, e.g. the deployment of the edge components and provisioning of the edge components, and also need to configure necessary flow diversion policy and user plane selection information, e.g. specific DNAI related with the UPF deployed on the satellite for the 5GC elements, e.g. AMF, SMF, which is used to find and select the UPF deployed on the satellite to complete the PDU session establishment.

Figure 5.4.2.1-1: MEC deployed on the satellite

#### 5.4.2.2	Potential requirements

REQ _MECOS_CON-1 The 3GPP management system should be able to manage the MEC deployed on the satellite, e.g. the deployment and provisioning of the edge components.

REQ _MECOS_CON-2 The 3GPP management system should be able to configure the user plane selection information and flow diversion policy for the 5GC elements, e.g. SMF to support PDU session establishment procedure using the UPF deployed on the satellite.

#### 5.4.2.3	Potential solution: Configuration of UPF and EASDF to indicate deployment on satellite

For REQ _MECOS_CON-1，this solution proposes to add new attributes or IOCs that required to be presented when the UPF is on-board satellite. It may contain the following attributes:

- Explicit indication of whether the UPF or EASDF is deployed on a satellite or on the ground.

-      Satellite identification information representing the satellite on which the UPF or EASDF is deployed.

For REQ _MECOS_CON-2，the existing UPF selection solution for deployment on the satellite can be reused.

#### 5.4.2.4	Evaluation of potential solution

For REQ _MECOS_CON-1, the potential solution described in clause 5.4.2.3 adopts the NRM-based approach, which enhances the existing NRM fragment with new conditional attributes indicating whether this functions are on board the satellite and related deployment information, the change is lightweight and it is backward compatible, therefore it is a feasible solution.

For REQ _MECOS_CON-2，the existing NRM approach can be reused.

End of change