3GPP TSG-SA5 Meeting #158 	S5-246780

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

# Source:	China Telecom

# Title:	pCR TR 28.874 Update S&F potential solution#2 for addressing the Editor’s note

# Document for:	Approval

Agenda Item:	6.19.15

1	Decision/action requested

The group is asked to discuss and agree on the proposal.

2	References

[1]		3GPP TR 28.874: Study on management aspects of NTN – Phase 2

3	Rationale

This contribution proposes to update the S&F potential solution#2 with more clear influence on NRM and add the procedure flow for addressing the EN.

4	Detailed proposal

This document proposes the following changes in TR 28.874 [1].

1st Change

5.3.2.3.2	Potential solution #<2>: Configuration of the MME to indicates whether it support S&F operation or not

This is a candidate solution to address the REQ-SNF-REQ-1.

This solution proposes to add new attributes or IOCs that required to be presented when the MME supports S&F operation. It may contain the following attributes:

- New attribute called "snFSupportInd " can be defined in the MMEFunction IOC to indicate whether this function supports S&F operation or not.

- New attribute called "snFEphemerisInfo" may be defined in the MMEFunction IOC or in a new IOC representing configuration information for S&F operation (e.g., S&FConfigInfo defined in clause 5.3.1.3.1) to support MME determining the wait-timer and list of satellite ID to be provided to UE. The data type of this new attribute represents the S&F satellite ephemeris related information, e.g. snFSatelliteId a new attribute representing the identification of the satellite that supports S&F operation and locationInfo as defined in TS 28.541 [13] clause 5.3.188.

NOTE:	This solution needs to align with RAN WG3 and SA WG2.

Figure 5.3.2.3.2-1: procedure of S&F operation relation configuration

- Operator decides for a MME to support S&F operation based on some local policies and service contracts.

- MnS consumer requests to create MMEFunction and/or the new IOC representing configuration information for S&F operation instance, the snFSupportInd and snFEphemerisInfo are added as necessary attributes.

Provisioning MnS producer sends instance creation results to MnS consumer.

5.3.2.4	Evaluation of potential solutions

The possible solutions described in clause 5.3.2.3 adopt the NRM-based approach, which enhance the existing NRM fragment with new conditional attributes indicating whether these functions support S&F operation or not, and the information needed by eNB supported S&F operation to broadcast, and the information needed by MME supported S&F operation to determine specific information for UE, the change is lightweight and it is backward compatible, therefore it is a feasible solution.

End of changes