3GPP TSG-SA5 Meeting #158	S5-246804

Orlando, FL, U.S.A., 18 - 22 November 2024

# Source:	Ericsson

# Title:	Withdraw incorrect contribution on Sub optimal Handover Metrics

# Document for:	Approval

Agenda Item:	6.19.12

# 1	Decision/action requested

Approve this contribution.

# 2	References

[1]	S5-245521, Rel-19 CR TS 28.552 Sub optimal Handover Metrics

[2]	3GPP TS 32.401, Telecommunication management; Performance Management (PM); Concept and requirements

[3]	3GPP TS 38.331, NR; Radio Resource Control (RRC); Protocol specification

# 3	Rationale

## 3.1	Main flaw

During SA5#157, Hyderabad, contribution S5-245521, Rel-19 CR TS 28.552 Sub optimal Handover Metrics [1] was agreed. This was unfortunate, as the contribution is incorrect and should not be implemented.

The contribution in question proposes a novel method for collecting PM measurements. The core of the idea is to update a counter after the end of its granularity period. Unfortunately, this novel method is flawed and not implementable.

The main obstacle is that, at the end of a granularity interval, the result is computed and transferred to a collection entity. This means that, at the end of a granularity period, the measurement cannot be changed. The measurement may even have left the node where it was collected and sent to a Trace Collection Entity (TCE). However, [1] is based on the assumption that a measurement can be updated after the end of the granularity period.

The fact that the contribution cannot be implemented follows from the description of PM measurements, in TS 32.401 [2] (our highlighting):

#### 5.4.1.4 Granularity period The granularity period is the time between the initiation of two successive gatherings of measurement data. Valid values for the granularity period are 5 minutes, 15 minutes, 30 minutes, 1 hour. The minimum granularity period is 5 minutes in most cases, but for some measurements it may only make sense to collect data in a larger granularity period. The granularity period shall be synchronised on the full hour, but its value is not required to be changeable during the lifetime of the job. #### 5.4.1.5 Measurement reporting Each measurement job running on an NE produces scheduled measurement reports (measurement records) at the end of each granularity period, and contains the information as requested by the system operator. This information consists of: - an identification of the measurement job that generated the report; - an identification of the involved measurement type(s) and the measured network resource(s) (e.g. NodeB); - a time stamp, referring to the end of the granularity period; - for each measurement type, the result value(s) and an indication of the validity of the result value(s); - an indication if the scan is not complete, and the reason why the scan could not be completed. The exact layout of the measurement reports (measurement records) generated by the NEs may be vendor specific. Multiple measurement reports shall be collated, based on reporting period, into a measurement result file for transfer, e.g from the NE or the EM. The file format of this aggregated file is specified by Performance Measurement File Format Definition 3GPP TS 32.432 [29]. Clause 5.5.2 specifies how these reports can be transferred to the destination EM and/or NM. #### 5.4.1.6 Illustration of the measurement scheduling principles The diagram below gives an example of a NE which runs a measurement job, with a 15 minute granularity period, that has a recording interval start and end time, respectively, of 12:00 and 14:00. Figure 2 - At 12:00 the measurement job starts collecting data for its defined measurements. - At 12:15, and every 15 minutes during the Recording Interval, the results for the measurements will be computed from the data gathered over the previous 15 minutes, and measurement reporting occurs as specified in clause 5.3.1.4. - Beginning at 12:15, the results for the expired granularity periods may be sent to a destination OS. - At 14:00 the measurement job activity is terminated for this recording interval.

… and …

### 5.5.2 Transfer of measurement results During the recording intervals specified for a measurement job, scheduled measurement reports are generated at the end of each granularity period if the measurement job is not suspended. These reports can be transferred to the EM in either of two ways: 1) immediate notifications: - the reports are automatically forwarded to the EM at the end of the granularity period. 2) deferred retrieval: - the reports are stored locally in the NE, where they can be retrieved when required.

Contribution [1] assumes that the counter for a certain granularity period can be incremented up to 48 hours after the end of the granularity period, as the UE stores RLF reports or SHRs up to 48 hours. See TS 38.331 [3] clause 5.3.10.5 and 5.7.10.6. This is not possible for the current SA5 specified PM architecture.

## 3.2	Generalising the idea in the contribution

Contribution [1] rises the more generic question of how to collect measurements for the cases where an event that updates a counter is not known to the NE until after the granularity period ends. Is there a need for SA5 to specify such a solution?

- Contribution [1] counts Radio Link Failure (RLF) Reports and Successful Handover Reports (SHR). It attempts to collect information about “old” RLE Reports and SHRs. It is clear that this can be useful for optimising mobility.

- Are there more examples than RLF Reports and SHRs for which counting “old” events would be useful?

- Does SA5 need to develop an implementable solution that is implementable for counting “old” events?

We propose that SA5 discuss this topic.

## 3.3	Further considerations

Contribution [1] further contains several flaws that need to be addressed:

- 5.1.1.6.x item a): The three quoted text snippets do not exist in 38.300. Remove.

- A.x: Avoid exotic words like “lacuna”.

- A.x: This clause does not describe a use case, but looks like a motivation for the CR.

- A.x: A sentence like “We propose to address this lacuna in this invention by proposing measurements for these scenarios and their associated counting rules” suggests that this clause is merely a copy-paste from an invention disclosure. Needs rewording.

# 4	Detailed proposal

1. SA5 to withdraw contribution [1].

2. SA5 to discuss whether an implementable solution for counting “old” events is needed.