3GPP TSG-SA5 Meeting #158	S5-246810

# Orlando, USA, 18-22 November 2024

# Source:	Ericsson

# Title:	Rel-19 pCR 28.874 Update solution evaluations

# Document for:	Approval

Agenda Item:	6.19.15

# 1	Decision/action requested

Approve the pCR

# 2	References

[1]				3GPP TR 28.874-110 Study on management aspects of NTN – Phase 2

# 3	Rationale

This pCR adds evaluations of pros/cons for Use Case #1 / Potential solution #<4> and Use Case #2 / Potential solution #<3>.

# 4	Detailed proposal

Beginning of changes

#### 5.1.1.4	Evaluation of potential solutions

There are 2 potential solutions support the REQ-NTN-REGCON-1. It is proposed to evaluate them based on the following principles.

Principle 1: When regenerative mode is considered, the potential solution supports gNB/eNB and ground nodes to know in advance when the connections will be lost and need to be reconnected, without dependence of a working management interface. So that the N2/S1 connection setup/resume and disconnect/suspend can be triggered.

Both of the potential Solution #<1> and the potential Solution #<2> can support Principle 1. Moreover, both of them can be configured based on the pre-obtained information about the connection between the satellite and the gateway. Therefore, neither of them has the risk of N2/S1 connection update failure due to the configuration delay.

Principle 2: Less configuration complexity.

Solution #<1> proposes to change the attribute of the existing EP_RP IOC. It requires that the existing standard definition of the FarEndEntity attribute changes from a DN type attribute allowing only one value, to the FarEndEntityList attribute which is a list attribute and each list element of this attribute consists of a FarEndEntity and a timeWindow describing the valid time of the connection.

Because of this change, the associations represented by EP_RP IOC may need to change from one-to-one association to one-to-many association. Considering that many IOCs inherit from the EP_RP IOC, such change may lead to impact on other IOCs.

Solution #<2> adds a new attribute or IOC in the existing interface instances. The new attribute or IOC is a list describing the available time window for the instance.

Potential solution#<1>:

- Pros:

  - Supports gNB/eNB and ground nodes to know in advance when the connections will be lost and need to be reconnected, without dependence of a working management interface. So that the N2/S1 connection setup/resume and disconnect/suspend can be triggered.

  - Since the association between the local address and the remote address is changed, the complexity and overhead when configuring all association time windows is reduced, as only one instance per association needs to be updated.

  - No changes to EP_RP and not modifying any existing attributes, minimizing the impact on inheritance relationships and reducing unexpected backward compatibility issues.

  - In case of large satellite constellation, the number of EP instances will be the same as in terrestrial network case.

- Cons:

  - Each MOI can represent different logical entities in different time windows which can potentially increase the complexity.

Potential solution#<2>:

- Pros:

  - Supports gNB/eNB and ground nodes to know in advance when the connections will be lost and need to be reconnected, without dependence of a working management interface. So that the N2/S1 connection setup/resume and disconnect/suspend can be triggered.

  - No changes to EP_RP, minimizing the impact on inheritance relationships and reducing unexpected backward compatibility issues.

- Cons:

  - When the satellite constellation is large, many instances need to be created and updated, resulting in more overhead cost and potential misalignment needs to be handled.

Potential solution #<3.1>

- Pros:

  - Minimizes the complexity and overhead when configuring all association time windows, as only one instance per association needs to be updated. Also minimizes feeder link load as well as satellite CPU load and memory usage.

  - No backward compatibility issues as there are no modifications of any existing attributes.

- Cons:

  - Each MOI can represent different logical entities in different time windows which can potentially increase the complexity.

Potential solution #<3.2>

- Pros:

  - No backward compatibility issues.

- Cons:

  - As in gNB in space are non-geo synchronized, each space gNB needs to serve all the quasi-earth fixed cells on the entire earth, and the association updates need to be made with a period of approximately every minute, this solution has the drawback of managing a huge number of instances for all the connections (hundreds or even thousands) with the high system load for creation and updates, and related risk of delays and inconsistency in the creation/updates due to loss of feeder link between the management system and satellites, or alternatively a huge overhead and memory cost if all instances should be created in advance. In the latter case there is also a risk for inconsistent configuration in case some of all the sub-operations cannot be successfully executed for various reasons, causing a "PARTIALLY_FAILED" response.

Potential solution #<4>

- Pros:

  - No backward compatibility issues.

- Cons:

  - Creates dependencies between two separate IOCs: NTNFunction and EP_NgC/EP_N2/EP_RP_EPS, etc, which increases the complexity of root cause analysis.

  - The availability time window of a certain NgC/N2 is depending on the “line of sight” between the ground station and the satellite, while the availability time window of a certain quasi-earth-fixed NRCell is depending on the “line of sight” between the earth-fixed footprint and the satellite. These two types of availability time windows are not synchronized to each other and will not provide benefit in terms of decreased number of time window instances.

  - Some cons mentioned in Potential solution #<2> and Potential solution #<3.2> remain, e.g.:

    - When the satellite constellation is large, many instances need to be created, resulting in more overhead cost.

    - As in gNB in space are non-geo synchronized, each space gNB needs to serve all the quasi-earth fixed cells on the entire earth, this solution a huge overhead and memory cost if all instances should be created in advance.

  - Increased number of instances compared to Potential solution #<2> and Potential solution #<3.2>, as each gNB in space firstly needs to create all the IOC instances required as in Potential solution #<2> and Potential solution #<3.2>, and in addition the NTNTimeBasedConfig IOC instances described in this solution.

  - It is unclear if the Managed Entity containing NTNFunction IOC is located on ground or on the satellite. As in current version of TS 28.541 figure 4.2.1.1-21, the NTNFunction IOC contains a number of EphemerisInfoSet IOC, where each EphemerisInfoSet can be associated to one or several GNBCUCPFunction. This gives a strong indication that the Managed Entity containing NTNFunction IOC is located on ground. This violates the potential requirements on capability to configure the managed entity onboard the satellite with an unreliable connection between space and earth.

Next change

#### 5.1.2.4	Evaluation of potential solutions

Potential solution #<1>

- Pros:

  - Minimizes the complexity and overhead when configuring all association time windows, as only one attribute per association needs to be updated. Also minimizes feeder link load as well as satellite CPU load and memory usage.

  - No backward compatibility issues as there are no modifications of any existing attributes.

- Cons:

  - Each MOI can represent different logical entities in different time windows which can potentially increase the complexity.

Potential solution #<2>

- Pros:

  - No backward compatibility issues as there are no modifications of any existing attributes.

- Cons:

  - As the association updates need to be made with a period of approximately every minute, this solution has the drawback of managing a huge number of instances for all the connections (hundreds or even thousands) with the high system load to create and update them frequently, and related risk of delays and inconsistency in the creation/updates due to loss of feeder link between the management system and satellites, or alternatively a huge overhead and memory cost if all instances should be created in advance. In the latter case there is also a risk for inconsistent configuration in case some of all the sub-operations cannot be successfully executed for various reasons, causing a "PARTIALLY_FAILED" response.

Potential solution #<3>

- Pros:

  - No backward compatibility issues.

- Cons:

  - Creates dependencies between two separate IOCs: NTNFunction and NRSectorCarrier/SectorEquipmentFunction etc, which increases the complexity of root cause analysis.

  - Some cons mentioned in Potential solution #<2> remain, e.g.:

    - As the association updates need to be made with a period of approximately every minute, this solution has the drawback of managing a huge number of instances for all the connections (hundreds or even thousands), a huge overhead and memory cost if all instances should be created in advance.

  - Increased number of instances compared to Potential solution #<2>, as each gNB in space firstly needs to create all the IOC instances required as in Potential solution #<2>, and in addition the NTNTimeBasedConfig IOC instances described in this solution.

  - It is unclear if the Managed Entity containing NTNFunction IOC is located on ground or on the satellite. As in current version of TS 28.541 figure 4.2.1.1-21, the NTNFunction IOC contains a number of EphemerisInfoSet IOC, where each EphemerisInfoSet can be associated to one or several GNBCUCPFunction. This gives a strong indication that the Managed Entity containing NTNFunction IOC is located on ground. This violates the potential requirements on capability to configure the managed entity onboard the satellite with an unreliable connection between space and earth.

End of changes