3GPP TSG-SA5 Meeting #158	S5-246824

Orlando, USA, 18 - 22 November 2024

# Source:	Ericsson

# Title:	Usage during request for quota

# Document for:	Discussion

Agenda Item:	7.3

# 1	Decision/action requested

The group is asked to discuss and approve the proposal.

# 2	References

[1]	3GPP TS 32.290 5G system; Services, operations and procedures of charging using Service Based Interface (SBI)

[3]	3GPP TS 32.291 5G system, charging service; Stage 3

[4]	3GPP TS 32.298 Charging Data Record (CDR) parameter description

[5] 	3GPP TS 32.299 Diameter charging applications

# 3	Rationale

## 3.1	Background

The usage during request for quota is not described in TS 32.290 [1]. One way to interpret the current specification could be from the Diameter based charging TS 32.299 [5], where the difference between the granted units and the quota threshold is allowed to be used while waiting for the charging data response.

Figure 3.1: Flow session based charging using quota threshold

1)	Request for service delivery: A request for session-based service is received in the NF (CTF).

2)	Charging Data request [Initial, Requested Unit]: The NF (CTF) sends an initial request to the CHF for the service authorization based on account balance, and to be granted units.

3)	In case of non-blocking mode service may be authorized and units may be used.

4)	Charging Data response [Granted Unit, Quota Threshold]: The CHF authorize the service with a granted units and threshold to NF (CTF).

5)	If any units were used in step3, this is deducted from the granted units and the units left will be used up to the threshold.

6)	Charging Data request [Update, Unit Used, Requested Units]: When the threshold is met the NF (CTF) sends an update request to the CHF for reporting used units (which includes the units used in step 3 or step 7) with related charging information, and to be granted units.

7)	If any granted units were not used, i.e. the difference between threshold and granted units, then these may be used.

8)	Charging Data response [Granted Unit, Quota Threshold]: The CHF responds with a granted units and threshold to NF (CTF).

9)	If any units were used in step 7, this is deducted from the granted units and the units left will be used up to the threshold.

10)	Request for end of service delivery: A request for termination of the session-based service is received in the NF (CTF)

11)	Charging Data request [Termination, Unit Used]: The NF (CTF) sends a termination request to the CHF for the service for reporting used units (which includes the units used in step 7) with related charging information.

12)	Charging Data response [ ]: The CHF responds with an acknowledgement of the termination to NF (CTF).

## 3.1	Evaluation

The issue with the current way of working is twofold one it’s not really specified in TS 32.290 [1], and there is no knowledge of the units that are used while waiting for the charging data response.

In some cases, the units used while waiting for the charging data response is not required since these are included in the used units reported in the next request. However, if all units are used while waiting for the charging data response then the service delivery may be paused or stopped, which will have an impact on the service experience. This means that there is be a need for control of the difference between the “Quota Threshold” and the “Granted Units”, which defines the number of units that may be used while waiting for the charging data response. This to avoid that if the difference between “Quota Threshold” and the “Granted Units” is larger than needed there will be unnecessary requests or reservations of units, if the between “Quota Threshold” and the “Granted Units” is smaller than needed then the service delivery may be impacted.

# 4	Detailed proposal

The current way of working needs to be specified, and there is a need to have some information about the usage while waiting for the charging data response. It would be preferred to have both the units used and the units that could not be delivered (if know) while waiting for the charging data response.

This could be done as an addition to the common data structure of the Charging Data Request in TS 32.290 [1] table 7.1, and without changing what is reported in the “Used Units” i.e., the “Used Units” would still contain the units used while waiting for the charging data response as well as the units used until the “Quota Threshold” is reached.

Table 4.3.1: Addition to common data structure of Charging Data Request

| Information Element | Converged Charging Category | Description |
| --- | --- | --- |
| Used Unit During Request | OC | This field contains the number of units measured between the trigger of charging data request and response. |
| Time | OC | This field holds the used time. |
| Total Volume | OC | This field holds the used volume in both uplink and downlink directions. |
| Uplink Volume | OC | This field holds the used volume in uplink direction. |
| Downlink Volume | OC | This field holds the used volume in downlink direction. |
| Service Specific Unit | OC | This field holds the used service specific units. |
| Halted Unit During Request | OC | This field contains the units that could not be delivered between the trigger of charging data request and response. |
| Time | OC | This field holds the used time. |
| Total Volume | OC | This field holds the used volume in both uplink and downlink directions. |
| Uplink Volume | OC | This field holds the used volume in uplink direction. |
| Downlink Volume | OC | This field holds the used volume in downlink direction. |
| Service Specific Unit | OC | This field holds the used service specific units. |

This would give the CHF the possibility to adjust the difference between the “Quota Threshold” and the “Granted Units” to better match the usage of the service, but wouldn’t impact the current way of working.

Figure 4.1: Flow session based charging using quota threshold

1)	Request for service delivery: A request for session-based service is received in the NF (CTF).

2)	Charging Data request [Initial, Requested Unit]: The NF (CTF) sends an initial request to the CHF for the service authorization based on account balance, and to be granted units.

3)	In case of non-blocking mode service may be authorized and units may be used.

4)	Charging Data response [Granted Unit, Quota Threshold]: The CHF authorize the service with a granted units and threshold to NF (CTF).

5)	If any units were used in step3, this is deducted from the granted units and the units left will be used up to the threshold.

6)	Charging Data request [Update, Unit Used, Requested Units, Used Unit During Request, Halted Unit During Request]: When the threshold is met the NF (CTF) sends an update request to the CHF for reporting used units (including units used in step 3 or 7) in the Used Units with related charging information, and to be granted units. The Used Unit During Request will be the units used in either step 3 or 7 and Halted Unit During Request will contain any units that couldn’t be delivered in either step 3 or 7.

7)	If any granted units were not used, i.e. the difference between threshold and granted units, then these may be used.

8)	Charging Data response [Granted Unit, Quota Threshold]: The CHF responds with a granted units and threshold to NF (CTF).

9)	If any units were used in step 7, this is deducted from the granted units and the units left will be used up to the threshold.

10)	Request for end of service delivery: A request for termination of the session-based service is received in the NF (CTF)

11)	Charging Data request [Termination, Unit Used, Used Unit During Request, Halted Unit During Request]: The NF (CTF) sends a termination request to the CHF for the service for reporting used units (including the units used in step 7) with related charging information. The Used Unit During Request will be the units used in step 7 and Halted Unit During Request will contain any units that couldn’t be delivered in step 7.

12)	Charging Data response [ ]: The CHF responds with an acknowledgement of the termination to NF (CTF).