3GPP TSG-SA5 Meeting #158	S5-246838

Orlando, USA, 18 - 22 November 2024

# Source:	Ericsson

# Title:	New solution for topic 2

# Document for:	Approval

Agenda Item:	7.5.2

# 1	Decision/action requested

The group is asked to discuss and approve the proposal.

# 2	References

[1]	3GPP TS 28.849 Study on charging Aspects for Common API Framework (CAPIF) phase2

# 3	Rationale

New solution for topic 2 reusing Exposure function Northbound Application Program Interfaces (APIs) charging.

# 4	Detailed proposal

This document proposes the following changes in TR 28.849 [1].

First change

#### 6.1.5.5	Solution #2.x: Use of Exposure function Northbound Application Program Interfaces (APIs) charging

##### 6.1.5.5.1	General description

This solution covers key issue #2.1, and requirement REQ-CH_CAPIF_NEF-01. It reuses the current exposure function northbound APIs charging, TS 32.254 [3], with some adaptations and extensions.

##### 6.1.5.5.2	Architecture description

For architecture see TS 32.254 [3] clause 4.4 where CAPIF Core Function (CCF) is an alternative to NEF and the NEF/SCEF could be considered API Providers.

##### 6.1.5.5.3	Procedures description

For flows see TS 32.254 [3] clause 5.4.2 where CCF is an alternative to NEF.

For the API Provider charging the following is required:

-	API Provider would be considered the tenant, i.e. the API Provider Id would be stored in the Tenant Identifier

-	New triggers for service API publish, unpublish, retrieve, update

-	The API Target Network Function would be CCF

For the API Invoker charging the following is required:

-	API Invoker would be considered the tenant, i.e. the API Invoker id would be stored in the Tenant Identifier

-	New triggers for onboarding and offboarding

-	The API Target Network Function would be CCF

End of change