3GPP TSG-SA5 Meeting #158	S5-246888

Orlando, USA, 18 - 22 November 2024

# Source:	Nokia

# Title:	Rel-19 pCR TS 28.abc Add network resource model to manage signalling monitoring

# Document for:	Approval

Agenda Item:	6.19.22 MonStra

# 1	Decision/action requested

The group is requested to discuss and approve the pCR below.

# 2	References

[1]	3GPP TS 28.abc V0.1.0 Signalling traffic monitoring management

# 3	Rationale

The 3GPP management system controls the network by a service-based and model-driven architecture with an object-oriented model.

In order to integrate the control of sending copied signalling messages into the 3GPP management system, the network resource model needs to be enhaced by corresponding informatin object classes (IOC).

# 4	Detailed proposal

The following changes are proposed to TS 28.abc [1].

Begin of modifications

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[a]	3GPP TS 28.532: “Management and orchestration; Generic Management services”.

[b]	3GPP TS 28.622: “Generic Network Resource Model (NRM)”.

[c]	3GPP TS 28.541: “5G Network Resource Model (NRM)”.

[d]	3GPP TS 28.625: “State Management Data Definition”.

Next modification

# 7	Model

## 7.1 	Imported and associated information entities

### 7.1.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.532 [a], notification, notifyMOICreation | notifyMOICreation |
| TS 28.532 [a], notification, notifyMOIDeletion | notifyMOIDeletion |
| TS 28.532 [a], notification, notifyMOIAttributeValueChanges | notifyMOIAttributeValueChanges |
| TS 28.532 [a], notification, notifyMOIChanges | notifyMOIChanges |
| TS 28.622 [b], IOC, Top | Top |
| TS 28.622 [b], IOC, ManagedElement | ManagedElement |
| TS 28.622 [b], IOC, SubNetwork | SubNetwork |
| TS 28.622 [b], abstract class, EP_RP | EP_RP |
| TS 28.541 [c], IOC, clause 5.3: All IOC that represent network functions of 5GC. | e.g. AMFFunction, SMFFunction |
| TS 28.541 [c], IOC, clause 5.3: All IOC that represent service-based interfaces of network functions of 5GC. | e.g. EP_N4, EP_N11 |
| TS 28.625 [d], attribute administrativeState | administrativeState |
| TS 28.625 [d], attribute operationalState | operationalState |

Note: It is not useful to copy all classes that represent network functions or corresponding interfaces from [c, clause 5] “Information Model definitions for 5GC NRM” into the above table and to keep this table consistent with [c]. Therefore the table is meant to import from [c, clause 5.3] all classes representing NF or their corresponding service-based interfaces.

## 7.2	Class diagrams

### 7.2.1	Relationships

This clause depicts the set of classes (e.g. IOCs) implemented by Management of Signalling Monitoring. This clause provides the overview of the relationships of relevant classes in UML. Subsequent clauses provide more detailed specification of various aspects of these classes.

Figure 7.2.1-1 Signalling Monitoring control NRM fragment

### 7.2.2	Inheritance

This clause depicts the inheritance relationship.

Figure 7.2.2-1 Signalling Monitoring control NRM fragment

## 7.3	Class definitions

### 7.3.1	SignallingMonitoringControl <<IOC>>

#### 7.3.1.1	Definition

An instance of IOC SignallingMonitoringControl represents the capabilities of the corresponding network function to send copies of signalling messages towards the given collecting entity.

The attribute interfaceRefList contains a list of references to interfaces objects (e.g. EP_N4, EP_N11) to control for which of its signalling interfaces the containing NF shall send copies of the corresponding messages. Since one NF might support multiple interfaces to multiple NF of the same kind (e.g. one AMF might have multiple EP_N11 to connect to mutiple SMFs), the monitoring control needs a possibility to manage these individually.

The attribute signallingMonitoringCollectionUri specifies the streaming target.for the copied messages.

To activate sending copies of signalling messages to the collecting entity, the MnS consumer sets the administrativeState to “UNLOCKED”; to stop sending the MnS consumer sets the administrativeState to “LOCKED”.

The MnS producer uses the attribute operationalState to indicate whether it is able to send copied messages to the collecting entity (“ENABLED”) or whether it is not able to send (“DISABLED”), e.g. due to some malfunction or network error.

#### 7.3.1.2	Attributes

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| administrativeState | O | T | T | F | T |
| operationalState | O | T | F | F | T |
| interfaceRefList | O | T | T | F | T |
| signallingMonitoringCollectionUri | M | T | T | F | T |

#### 7.3.1.3	Attribute constraints

Editor Note: TBC

#### 7.3.1.4	Notifications

The common notifications defined in clause 7.5 are valid for this IOC, without exceptions or additions.

Editor Note: TBC

## 7.4	Attribute definitions

### 7.4.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| interfaceRefList | List of DN as references to interfaces (EP_xxx) as defined by TS 28.541 [c] (currently clauses 5.3.19 – 5.3.48) and derived from EP_RP 28.622 [10] (clause 4.3.11). An empty list indicates that all interfaces of the corresponding NF shall be reported. Default value is the Empty list, i.e. the corresponding NF shall send copies from all signalling interfaces it supports. | type: DN multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: Empty list isNullable: True |
| signallingMonitoringCollectionUri | It specifies the Uniform Resource Identifier (URI) of the entity that collects the copied signalling messages, i.e. where the NF shall send the copied signalling to.. | type: URI multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| administrativeState | Administrative state of a managed object instance [d]. The administrative state describes the permission to use or prohibition against using the object instance. The administrative state is set by the MnS consumer. allowedValues: LOCKED, UNLOCKED. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: LOCKED isNullable: False |
| operationalState | Operational state of manged object instance [d]. The operational state describes if an object instance is operable ("ENABLED") or inoperable ("DISABLED"). This state is set by the object instance or the MnS producer and is hence READ-ONLY. allowedValues: ENABLED, DISABLED. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: DISABLED isNullable: False |

## 7.5	Common notifications

### 7.5.1	Configuration notifications

This clause presents a list of notifications, defined in [7], that a MnS consumer can receive.

| Name | S | Notes |
| --- | --- | --- |
| notifyMOICreation | O |  |
| notifyMOIDeletion | O |  |
| notifyMOIAttributeValueChanges | O |  |
| notifyMOIChanges | O |  |

End of modifications