3GPP TSG-SA5 Meeting #159	S5-250059

Sofia-Antipolis, US, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.567 | CR |  | rev |  | Current version: | 0.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS28.567 Conditional control of CCLs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CCLM |  |  |  |  |  | Date: |  |  | 2025-02-13 |
| Category: |  |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add use case, requirements and solution description for Conditional control of CCLs of CCLs |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clauses to be introduced |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | use case for Conditional control of CCLs of CCLs will not be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5 (new clause 5.1 added) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

# 5	Closed Control Loop management capabilities

## 5.2	Conditional control of CCLs

### 5.2.1	Description

#### 5.2.1.1	Conditionally triggered CCL

As a default, the CCL enables a consumer to request the initiation of a CCL with the need to achieve a certain goal. The CCL is expected to monitor the network to see if there have been some goal breaches. If there is, the consumer is notified, and the appropriate actions can be taken to mitigate the breach by the consumer. The consumer may also decide to update the existing CCL or create a new one to mitigate the breach. A CCL is always instantiated, updated and deleted on an explicit request from the consumer.

Considering the autonomous nature of CCL, it is beneficial to study possible improvements to CCL management including automated instantiation, update and deletion of a CCL based on information provided by the consumer that could be used by the system to trigger CCL management. The existing CCL mechanism places a burden on the consumer to monitor the network and decide whether to instantiate a CCL, update a CCL, or delete a CCL. A possible improvement may be to allow the consumer to define trigger conditions for automated instantiation, update and deletion of a CCL.

The MnS consumer may want to request for a CCL to be dynamically instantiated when certain conditions are met. For example, the MnS consumer may want that for a CCL of a stated type or that matches a set of stated characteristics (e.g. goal) to be instantiated under conditions A and another with variations in goals to be instantiated under other conditions. The MnS consumer should be enabled to define those conditions so that the CCL is instantiated when the stated conditions are met.

The ConditionMonitor[11] is utilized to define triggering conditions for CCL management.

#### 5.2.1.2	Conditional execution of CCLs network changes

For the CCLs that have been instantiated, the MnS consumer may want to request for a CCL to be triggered to execute when certain conditions are met, e.g. when the performance on a certain threshold is crossed, or when the confidence is the decision is above a stated threshold. The consumer does not need to be aware of all decisions, but by providing conditions under which decisions may be activated or not, it is able to have supervision over the CCL without having to continuously track the decisions. The MnS consumer should be enabled to define those conditions for executing the CCL and that the CCL is triggered to execute when the stated conditions are met. Otherwise, the consumer should be enabled to define alternative actions, e.g. to notify the consumer of the decision that is not executed.

By supporting this, the execution can be affected by producer based on consumer's conditions or requirements.

### 5.2.2	Requirements

REQ-TRI-FUN-01: The 3GPP management system should enable authorized consumers to provide information that can be used to trigger CCL instantiation.

REQ-TRI-FUN-02: The 3GPP management system should enable authorized consumers to provide information that can be used to trigger CCL update.

REQ-TRI-FUN-03: The 3GPP management system should enable authorized consumers to provide information that can be used to trigger CCL deletion.

### 5.2.3	Solution Description

TBD

End Change