3GPP TSG-SA5 Meeting #159	S5-250064

Sofia-Antipolis, US, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.567 | CR |  | rev |  | Current version: | 0.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS28.567 CCL indirect target conflict handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CCLM |  |  |  |  |  | Date: |  |  | 2025-02-13 |
| Category: |  |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add use case on CCL indirect target conflict handling |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clauses on the use case to be introduced |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6 (new clause added) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

# 6	Closed Control Loops Coordination Capabilities

## 6.A	 Closed Control Loop Conflict management

### 6.A.1	Description

#### 6.A.1.5	CCL Indirect target conflict handling

##### 6.A.1.5.1	Introduction

Two (or more) CCLs configuring different control parameter may all influence the same metric. If the two CL desire different values for the metric, the CCLs are in conflict for the metric resulting into a metric-value conflict. In effect the actions of the two CCLs are in conflict but indirectly since they are conflicting for the same control parameter but their impacts are conflict on the desired value of the metric or target. Such conflicts are thus called Indirect target conflicts.

The Indirect target conflicts should be avoided and if they occur, they should be detected and resolved.

##### 5.6.3.5.2	Detecting potential and actual indirect targets conflicts

Two CCLs (CCL1 and CCL2) may optimize 2 target metrics Y1 and Y2, e.g. one intending to ensure "HO failure is < 2 %" while the other wants "SINR > 10dB". Due to coupling between Y1 and Y2, actions to optimize these by CCLs may lead to correlated oscillations/degradations in Y1 or Y2. The correlated oscillations indicate a potential conflict, but the CCLs may not see the oscillations in the metric that is not of their interest. The coordinator CCL may analyse the behavior of Y1 and Y2 to see if there are correlated oscillations as result of actions by CCL1 and CCL2 which then indicates potential conflict between CCL1 and CCL2. When the oscillations are observed, the CCL coordination MnS producer should be able to inform the related MnS Consumer(s) (i.e. CCL1 and CCL2) about the detected potential conflict represented by the correlated oscillations.

For detected potential conflict the CCL coordination service producer needs to confirm that it is an actual harmful conflict. This can be determined based on the severity of degradation in the performance metrics of the related CCLs. The threshold to determine the severity may be defined by the MnS consumer (e.g. the operator or coordinator CCL). If the degree of degradation is higher than the threshold then it is a confirmed conflict that requires resolution. Otherwise, no action is needed.

##### 6.A.1.5.3	Avoiding potential indirect target conflicts

For any CCL, large and frequent changes to network parameters may affect network stability since they increase the probability of occurrence of conflicts, i.e. avoiding making unnecessary configuration changes to the managed objects guarantees network stability and minimize the probability of conflicts between CCLs. This may then imply that executing large changes, e.g. to quickly improve the performance, in case of a poor decision, may also result in significant degradation. So, it is preferred to take small smooth changes in the case where the impact is not so clear, and only make the large changes when the CCL is sure that the impact is positive.

For any planned action, the CCL sends to the coordinator CCL the planned change, its claimed/predicted performance improvement and reliability/confidence in that action/decision. The coordinator CCL evaluates the claimed performance improvement and reliability/confidence to determine if the action should be allowed or not. This ensures to avoid counter-productive actions - if the CCL demands to make large changes, it proves high reliability/ confidence and significant improvement in performance. The criteria applied by the coordinator CCL to match acceptance/rejection of a planned action to the reliability and performance may be implementation-specific or defined by the operator.

The coordinator CCL then sends the decision and possibly the failed criteria to the CCL - to either be executed or to be used to compute better decisions. It is assumed that based on feedback on the quality of its decisions, the CCL updates its decision-making engine and repeats the decision evaluation process. Then if the CCL has consistently made good large action-decisions, the coordinator CCL can consider the CCL as trusted to make such large decisions. The coordinator CCL informs the CCL that the CCL has consistently made good decisions and achieved its ultimate trust.

##### 6.A.1.5.4	Resolving indirect targets conflicts

For a detected indirect targets conflict, the coordinator CCL can trigger one or more CCLs to respond to the detected potential conflict. If the CCLs that has been requested to resolve potential conflict is unable to resolve that conflict, the CCL should inform the CCL coordination MnS producer about the failure to resolve the problem.

End Change