3GPP TSG-SA5 Meeting #159	S5-250065

Sofia-Antipolis, US, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.567 | CR |  | rev |  | Current version: | 0.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS28.567 Conditional control of CCLs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CCLM |  |  |  |  |  | Date: |  |  | 2025-02-13 |
| Category: |  |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add use case, requirements and solution description for Conditional control of CCLs of CCLs |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clauses to be introduced |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | use case for Conditional control of CCLs of CCLs will not be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5 (new clause 5.1 added) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

# 6	Closed Control Loops Coordination Capabilities

## 6.B	CCL-impact assessment and resolution

### 6.B.1	Description

#### 6.B.1.1	Consumers feedback on CCL actions

In fully automated control loops, the CCL re-configures a particular NF to meet its stated goals without the involvement of any other entity. The actions executed by the CCL have different levels of satisfaction for the different consumers. Without a reliable means to gauge consumer Execution Satisfaction, the CCL lacks the feedback to fine-tune and optimize functionality, and so is unable to improve the overall performance. To be able to gauge the satisfaction, even as the CCL continuously executes actions onto the network, the consumer should be able to receive information about the provisioning operations executed by the CCL. This information includes operation performed, MOIs updated, etc. The CCL does not break its execution when it provides information to the MnS consumer or to wait for feedback from the MnS consumer. The feedback from an MnS consumer does not break the loop.

Based on some local policies, the consumer may prefer that a particular NF is not updated as part of the Execution step of CCL. The consumer should be enabled to revoke the changes made to a NF. Consumer may also update the CCL to ensure that a particular NF is never updated in future. The existing attribute aCCLDisallowedList can be used, as appropriate.

Alternatively, the consumer may want to provide feedback enabling the CCL to apply an alternative approach to achieve the objectives. The consumer should be able to provide its feedback on the execution indicating how satisfied the consumer is with the CCL actions.

EXAMPLE:	The consumer feedback may grade the usefulness of the executed action on a fixed scale say from 0 (indicating a terrible and never to be re-used action) to 10 (indicating a very good action for the interests of the consumer).

NOTE:	The actions that need to be provided to the consumer are decided by the producer.

#### 6.B.1.2	Impact on known/bounded impact-scope

For some Closed Control Loops, the expected impact of the action may be known to the Closed Control Loop or coordination functionality governing the CCL. The scope affected by these actions is derived from the (candidate) actions executed by the CCL (or their descriptions). A CCL coordination functionality may wish to evaluate the known impact scope and needs to rely on information from MnS producers of other Closed Control Loops to:

1)	determine if there are unwanted outcomes;

2)	diagnose if the executed action(s) is/are responsible for those outcomes, especially for the case where multiple Closed Control Loops have concurrently taken actions; and

3)	determine what needs to be done to undo the degradation and to avoid it in future.

#### 6.B.1.3	Impact on unknown impact-scope

For some CCLs, the impact-scope affected by the actions of a CCL A may not be known a priori. For example, for a CCL A that adjusts transmit power of a cell (e.g. to minimize interference), the exact neighbour cells and related CCLs acting on those cells that would be affected by any transmit power decrease or increase cannot be explicitly enumerated. Any negative effects cannot be easily anticipated, and most may not be easily resolvable by simple if-then-else rules. Instead, the MnS producer of a CCL A should interact with MnS producers of the other CLLs or with a coordination functionality to identify actions that lead to negative outcomes and flag them accordingly. Thereby, after the CCL A takes an action on the network:

1)	MnS producer of CCL A or the coordination functionality notifies all other CCLs or MnS producers of all other CCLs that an action has been executed that may affect those CCLs. The action is expected to have impact is a specified time, called the impact-time, which depends on the use case. Fr example the impact of load balancing action can be evaluated in a few seconds while the impact of a handover decision can take several minutes or hours. The notification should include the length of this impact time which indicates the time at which an observed impacts should be reported.

2)	After a preset monitoring period equivalent to the notified impact time, the MnS producers of the impacted CCLs report (directly or through the coordination functionality) the impact that CCL A had, (i.e. the impact that CCL A's action had had to their performance metrics or goals. The impact may be reported an index say in the range [0,10] where 0 implies an unacceptable action and 10 implies a good action.

3)	MnS producer of CCL A or the coordination function derives an appropriate remediation, e.g. by reconfiguring the candidate actions of the acting CCL (i.e. CCL A) or by undoing the action.

NOTE:	That this clause needs further clarification.

#### 6.B.1.4	Detection of indirect targets conflicts from impact on unknown/unbounded impact-scope

For impacts on an unknown impact-scope, the CCL that took action or its CCL coordination entity cannot determine the impact and has to collect that form the affected entities. So:

-	The CCL that took action or its CCL coordination entity should be enabled to notify the affected MnS consumers (e.g. other CCLs) of:

1)	the fact that an action has been taken that may affect them;

2)	that they need to provide feedback on how impact there has been; and

3)	when they need to provide that feedback.

-	To enable the affecting CCL to determine how much impact it had on the other CCL, the affected CCLs can provide their evaluation of the impacts as an index that indicates the degree to which the action was good or bad to their objectives.

### 6.B.2	Requirements

REQ-FED-FUN-01: The 3GPP management system should enable consumer to provide its feedback on the action(s) taken by CCL.

REQ-FED-FUN-02: The 3GPP management system should enable consumer to request for revocation of the action(s) taken by the CCL.

REQ-FED-FUN-03: The 3GPP management system should have a capability enabling consumer to receive information (e.g. operation performed, MOIs updated) about the action(s) taken by the CCL.

NOTE:	The CCL does not break its execution when it provides information to the MnS consumer or to wait for feedback from the MnS consumer. The feedback from an MnS consumer should not break the loop.

REQ-CCL-IMPACT-1: The CCL MnS producer should support a capability enabling an MnS consumer to receive information on the impacts of the CCL on a particular impact-scope and the actions that caused such impacts.

NOTE 1:	The MnS consumer may for example be another CCL or a CCL impact coordination function.

NOTE 2:	The information enables the MnS consumer to determine if there are unwanted outcomes resulting from actions of the CCL and to propose what needs to be done to undo the degradation.

REQ-CCL- IMPACT-2: The CCL MnS producer should support a capability enabling an MnS consumer to notify the MnS producer of the actions of another CCL that may affect the MnS producer's CCL.

NOTE 3:	The MnS consumer could for example be a CCL impact coordination function.

NOTE 4:	The MnS producer represents the CCL which may be potentially impacted when a CCL A executes an action that may affect the goals or metrics of the MnS producer's CCL.

REQ-CCL- IMPACT-3: The CCL MnS producer should support a capability to report to an MnS consumer what the impact that the action had to the goals of the MnS producer's CCL.

NOTE 5:	The MnS producer represents impacted CCL or MnF or a coordination function representing the impacted CCL or MnF.

NOTE 6:	MnS consumer may for example be a coordination function or an acting CCL that took an action that has impacted the MnS producer's metrics.

NOTE 7:	The use of metrics for this requirement is For Further Study.

REQ-CCL- IMPACT-4: The CCL MnS producer should support a capability enabling an MnS consumer to propose to MnS producer the appropriate remediation against the noted impact, e.g. the reconfiguration of the candidate actions of the acting CCL.

NOTE 8:	MnS consumer may be the CCL impact coordination function or another CCL or management function.

NOTE 9:	The MnS producer may be the acting CCL or the impacted CCL.

### 6.B.3	Solution Description

When a CCL (or its CCL coordination entity) does not have full knowledge of the scope it impacts (for impacts on an unknown impact-scope), the CCL collects the impact from any entities that have been affected by the CCL’s actions, which are jointly called CCL-action-impacted entities. The CCL that has taken action (called the actor-CCL) or its CCL coordination entity notifies the affected MnS consumers (e.g. other CCLs) or other management functions of the action and the need to evaluate and report about it. Accordingly, the CCL contains a data type representing information on this executed action, the information including:

-	the fact that an action has been taken that may affect them;

-	that they need to provide feedback on how impact there has been; and

-	the time when the affected entities should provide that feedback.

The CCL contains an attribute, called CCL-action-impact time, representing the time at which the affected entities should provide that feedback for an action that is taken by the CCL. Any entity which may be affected by the CCL actions (e.g. e.g. the CCL coordination entity or other CCLs) subscribes to be notified of changes to the CCL-action-impact time. Then, subsequent to the action, the CCL updates the CCL-action-impact time so that notifications are sent to the subscribed entities. Receiving the values of the CCL-action-impact time indicates to the subscribed entity that an action has been taken by the CCL and if the entity is affected, it should provide its feedback on effect in a time not exceeding the CCL-action-impact time.

After evaluating the impacts of the action, the CCL-action-impacted entity computes it observed impact in form of an index, called the Action Quality Indicator (AQI), that describes and quantifies the observed impact, i.e. it indicates the degree to which the action was good or bad to their objectives. the Action Quality Indicator is an integer in the range [0,10] where "0" indicates that the action was completely unacceptable and should never be reused in that context while "10" indicates that the action had very good outcomes for the reporting CCL-action-impacted entity (e.g. the affected CCL). The CCL-action-impacted entity then send the AQI to the actor-CCL within the CCL-action-impact time that was notified by the actor-CCL.

NOTE 1: An index is used instead sending specific metrics measured by each CCL-action-impacted entity because specific metrics would require the actor-CCL to understand all the different metrics in exactly the same way as the CCL-action-impacted entities do, which is not guaranteed to always be true.

NOTE 2:	The AQI is specific to each CCL and to each scenario thar the CCL evaluates - since it is used to check how good or bad an action was for that CCL. Accordingly, its computation cannot be standardized, only its range can be standardized.

The Action Quality Indicator is delivered to the actor-CCL in one of two ways:

-	CCL-action-impacted entity writes the AQI into an equivalent attribute called “reported AQIs” on the actor-CCL, i.e., the actor-CCL contains an attribute for the Action Quality Indicator to which each CCL-action-impacted entity writes its computed AQI. The reported AQIs attribute is a list to which each affected entity appends a value

-	The CCL-action-impacted entity contains an attribute for an observed AQI in which it writes the computed AQI. This can then be notified to the actor-CCL which subsequently reads it to obtain the AQI for that CCL-action-impacted entity.

To enable the actor-CCL or its CCL coordination entity to determine how much impact it had on all the other CCLs together, it aggregates the impact based on the reported Action Quality Indicators from the respective CCL-action-impacted entities. Accordingly, the actor-CCL or its CCL coordination entity contains an attribute for the aggregate AQI that is computed from form the AQIs reported by the multiple affected CCLs and used by the actor-CCL to decide an appropriate action to minimise the impact. If it is computed by the CCL coordination entity, the coordination entity notifies it to the actor-CCL and may also propose a response action, e.g. to reverse the action that was taken.

Figure 6.B.3-1: CCL-impact assessment and actual indirect targets conflicts resolution

End Change