3GPP TSG-SA5 Meeting #159	S5-250066

Sofia-Antipolis, US, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.567 | CR |  | rev |  | Current version: | 0.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS28.567 Conditional control of CCLs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CCLM |  |  |  |  |  | Date: |  |  | 2025-02-13 |
| Category: |  |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add use case, requirements and solution description for Conditional control of CCLs of CCLs |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clauses to be introduced |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | use case for Conditional control of CCLs of CCLs will not be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5 (new clause 5.1 added) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[X]	3GPP TS 28.872: "Study on Management of planned configurations".

next change

# 6	Closed Control Loops Coordination Capabilities

## 6.C	 CCL decision escalation

### 6.C.1	Description

CCLs will make decisions in different contexts (states, status, conditions, etc.) of the network and not all decisions are equally effective. In some cases, the CCL may need to provide feedback indication the challenges faced on reaching a definite decision. The feedback may include a request to escalate its decision making to another entity.

A CCL can be fully or partially autonomous derivation or execution of its decisions. The degree to which the CCL independently executes decisions or escalates them, should be flexibly configurable by the MnS consumer as a confidence threshold. The confidence threshold could be configured based on the sensitivity of the operations under its control, the trust level in the decisions of the CCL and the necessity to consider a bigger picture at times. Then, based on how much confidence the CCL has in its decisions, the CCL can escalate a decision or situation to an escalation recipient which has this bigger picture (say has wider scope), can execute a different(larger) set of actions or has better capabilities, e.g. a larger and more capable ML model.

NOTE 1:	The computation of confidence within the CCL is up to implementation as it depends on the CCL's purpose and the scenario that the CCL is addressing.

The escalation recipient CCL enables the escalator CCL to request for escalation for a given network context or state with e.g. information about the escalator CCL preferences and observed constraints when driving decisions. Based on its evaluations, the escalation recipient CCL should provide to the escalator CCL a report that holds the outcomes that the CCL (acting as an escalation recipient) has derived for a given escalation request.

NOTE 2:	How this relates with existing Assurance closed loop execution supervision use case in TS 28.535 [2] is FFS.

Figure 6.C.1-1: End-to-end flow of CCL decision escalation

### 6.C.2	Requirements

REQ- CCL-ESC-1: The CCL MnS producer should have a capability to enabling an authorized consumer to configure the degree of autonomy of the CCL as characterization of the conditions under which the ACCL should escalate a decision.

REQ- CCL-ESC-2: The CCL MnS producer should have a capability to enabling an authorized consumer to configure the entity to which a decision should be escalated.

REQ-CCL-ESC-3: The CCL MnS producer (acting as an escalation recipient CCL) should have a capability to enabling an authorized MnS consumer (e.g. an escalator CCL) to request escalation of a decision or escalation of decision-making for a given network context or state to the CCL associated with the CCL MnS producer.

REQ-CCL-ESC-4: The 3GPP management system (or the CCL Mns producer) should have a capability enabling an MnS consumer to provide information related to its previous decisions, decision constraints, preferences, … as input to be used in resolving escalations sent towards the CCL associated with the CCL MnS producer.

REQ-CCL-ESC-5: The CCL MnS producer (acting as an escalation recipient CCL) should have a capability to provide to an authorized MnS consumer (e.g. an escalator CCL) a report that holds the outcomes that the CCL (acting as an escalation recipient) has derived for a given escalation request.

### 6.C.3	Solution Description

To enable escalation, there has to be entities to which decision can be escalated, called escalation recipients. These are mainly closed control loops but other decision makers, e.g. AIML inference functions could be used as escalation recipients. The CCL which wishes to escalate a decision is named an escalator CCL.

To enable escalation, each CCL contains an attribute identifying an entity acting as an escalation recipient to which a decision is escalated. The CCL also contains an attribute for defining the condition that triggers the escalation. For example, the CCL may trigger escalation when its level of confidence in the derived decision is below some threshold, in which case the confidence threshold is the condition for triggering the escalation. The confidence threshold attribute enables the CCL to autonomously make decisions for each situation and context based on its computed confidence level in the given situation. If the confidence level is lower than the confidence threshold the decision is escalated otherwise the decision is executed.

When a CCL requires an escalation, it instantiates a request for escalation on the escalation recipient. Accordingly, the escalation recipient contains an object representing the escalation request and that holds all the information related to the request for escalation, including:

-	An attribute for the proposed CM change which describes the configuration management changes that has been proposed by the escalator CCL.

-	An attribute for the decision constraints indicating the constraints observed by the escalating CCL in making the decision(s). The constraints may be the type context as defined in 3GPP TS 28.312 [5] or conditions expressed using JEX/XPATH.

-	An attribute for the escalation reason  can provide an optional description of the reason behind the escalation to provide more context and further clarification.

Based on the information in the escalation request, the capabilities of the escalation recipient as well as its observations of the evaluated network state, the escalation recipient derives an outcomes for the request which is written into an attribute on the escalation recipient called escalation outcome. The escalation outcome is delivered to the escalator CCL in one of two ways:

-	It is written into an equivalent attribute on the escalator CCL, i.e., the escalator CCL contains an attribute for the escalation outcome to which the escalation recipient writes its computed escalation outcome.

-	The escalation recipient contains an attribute for an escalation outcomes report in which it writes the derived outcomes for each corresponding escalation request. This can then be notified to the escalator CCL which subsequently reads it to obtain the recommendations.

The escalation outcome indicates whether the escalator CCL should take any action and what that action is. Accordingly, it contains an ENUM attribute to indicate what should be done by the escalator CCL, with the values:

-	"DONOTHING"- indicating that the escalator CCL does not need to take any action, i.e. the escalation recipient is addressing the scenario.

-	"APPLYACTION"- indicating that the escalator CCL should apply a specific set of actions proposed by the escalation recipient. The action is written into a proposed-actions attribute of the escalation outcome, which is the type plan according to TS 28.872[X].

-	"APPLYGUIDANCE"- indicating that the escalator CCL should compute a new CM change based on the guidance from the escalation recipient. The guidance is written into the proposed-actions attribute.

End Change