3GPP TSG-SA5 Meeting #159	S5-250069

Sofia-Antipolis, US, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.567 | CR |  | rev |  | Current version: | 0.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS28.567 CCL coordination solution description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CCLM |  |  |  |  |  | Date: |  |  | 2025-02-13 |
| Category: |  |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add solution description for CCL conflict handling |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clauses to be introduced |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NRM features may be hard to understand |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6 (new clause added) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

# 6	Closed Control Loops Coordination Capabilities

### 6.A.3	Solution Description

#### 6.A.3.1	Alternative CCL coordination Approaches for conflicts handling

To address the conflicts, coordination interactions are required either among CCLs or between the CCLs and one or more higher hierarchy coordination functions to avoid or detect and resolve the conflicts.

The coordination of CCLs could be accomplished via one of three approaches illustrated by Figure 6.A.3.1-1:

-	Distributed coordination with distributed execution (Figure 6.A.3.1-1 a), where the CCLs directly coordinate with one another, and each manages execution of its decisions.

-	Hierarchical coordination with distributed execution (Figure 6.A.3.1-1 b), where the CCLs coordinate through a separate coordination layer, say via a CCL coordination entity, but each manages execution of its coordinated decisions.

-	Hierarchical coordination and execution (Figure 6.A.3.1-1 c), where the CCLs coordinate through a separate coordination layer, say via a coordination entity that besides coordination also manages execution of the coordinated decisions.

Figure 6.A.3.1-1: Closed Control Loop Coordination approaches

Distributed coordination can lead to too many exchanges between the CCLs which may unnecessarily clog the system. On the other hand, "Hierarchical coordination and execution" implies that too much responsibility is concentrated in a single CCL. A desired behavior is that the individual CCLs are responsible for their own decision execution, so it is recommended that to follow the "hierarchical coordination with distributed execution" approach. In this approach, the CCLs are responsible for making their decisions and executing actions, but they coordinate with the CCL coordinator before, during or after execution.

#### 6.A.3.2	Hierarchical CCL-coordination-interactions for conflicts handling

To address the conflicts, coordination interactions are required between the CCLs and one or more higher hierarchy coordination functions to avoid or detect and resolve the conflicts among goals and their targets, control scopes or actions of the CCLs. The 3GPP management system includes at least one entity called the Coordination entity that undertakes the role of CCL coordination. The Coordination entity can be implemented as a CCL, an AIML inference engine or any other functionality that is found appropriate. The coordination entity may support coordination for conflict management for Goal- and Goal-targets- Conflicts; scope conflicts, CCL-Trigger-time and CCL-action-execution-time conflicts, Direct actions conflicts as well as indirect targets conflicts. Accordingly, the coordination of CCLs includes the management services with the following capabilities:

-	Capabilities to align targets among the goals of individual CCLs sharing a given scope.

-	Capabilities to identify different types of conflicts between CCLs such as parameters conflict, metrics conflict, or any others.

-	Capabilities to address the different interactions between CCLs with adequate mechanisms, such as conflict resolution mechanisms.

-	Capabilities to identify before the execution of a proposed action of CCL that such an action could cause undesired effects to other CCLs or to managed entities (e.g. pre-execution and post-execution coordination, concurrency coordination, etc.).

-	Capabilities to evaluate the impact and effectiveness of CCLs actions after their execution (e.g. impact assessment).

The coordination of CCLs could be required at different execution points of the CCL translating into different CCL coordination use cases with corresponding CCL coordination services required at those points as illustrated by example Figure 6.A.3.2-1. The coordination of CCLs could be achieved via direct interaction among the CCLs or via a third-party entity, say called the CCLs coordination Function (or simply CCL Coordinator).

NOTE:	The terms at the top indicate general naming of the groupings of coordination interactions at the different execution points during the execution of the CCL. Action-space coordination implies coordinating the sets of actions that the different CCL can apply. Concurrency control implies coordinating the times at which different CCLs can execute actions. Action-impact assessment indicates interactions and processes on the evaluation of the impacts of the different CCLs.

Figure 6.A.3.2-1: Exemplary Closed Control Loop Coordination interaction points

To support conflict handling, the coordination entity contains one or more of the 5 capabilities below, each represented by a data type and corresponding CCL coordination purpose attribute:

-	CCL Goal coordination purpose

-	CCL Scope coordination purpose

-	CCL Trigger and Execution Time coordination purpose

-	CCL Direct Actions coordination purpose

-	CCL Indirect Targets coordination purpose

The coordination purpose attributes contain the information and data needed or used by the coordination entity for interacting with the CCL when handling conflicts.

End Change