3GPP TSG-SA5 Meeting #159	S5-250093

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Nokia

# Title:	Rel-19 pCR TS 28.572 Add clause Asynchronous configuration updates

# Document for:	Approval

Agenda Item:	6.19.9.1 - Management of planned configurations

# 1	Decision/action requested

The group is requested to discuss and approve the pCR below.

# 2	References

[1]		3GPP TS 28.572: "Management of planned configurations"

# 3	Rationale

None.

# 4	Detailed proposal

The following changes are proposed for TS 28.572[1].

Begin of modifications

## 6.6	Asynchronous configuration updates

Updating the current configuration may take some time, more time than an update operation based on a synchronous request response pattern may be able to cope with, where the request provides the desired updates and the response information about the outcome of the requested updates. In a synchronous request-response pattern, the request and response occur almost at the same time.

This problem is addressed by so-called asynchronous update operations, where the request response pattern just provides the desired updates to the MnS producer. The response does not contain information about the outcome but a confirmation of reception of the desired updates. The information about the outcome of the update operations is provided asynchronously, i.e. later when the information is available.

## Management of planned configuration features as a special case also an asynchronous update operation. It is not mandatory to create a planned configuration before validation or activation. A new configuration can be provided also together with the validation or activation request. This is conceptually equivalent to an asynchronous update operation.

End of modifications