3GPP TSG-SA5 Meeting #159	S5-250170

Sophia-Antipolis, France, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.561 | CR |  | rev | - | Current version: | 0.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.561 Add New Use Case and Requirements on Life-cycle Management of NDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NDT |  |  |  |  |  | Date: |  |  | 2024-01-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add a generic use case and requirements for NDT life-cycle management. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a generic use case and requirements for NDT life-cycle management. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Use case and requirements for NDT life-cycle management is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5(new), 5.1(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | No impact on stage 3 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First Changes

5	Specification Level Requirements

5.1	Use cases

5.1.1	Life-cycle Management of NDT

5.1.1.1	Description

The lifecycle management of Network Digital Twins (NDT) refers to the comprehensive management process that ensures NDT can accurately reflect the status of physical entities and support efficient simulation. The lifecycle of NDT can be divided into three main phases: creation, configuration, and running. The NDT Management and Service (MnS) Consumer has distinct management requirements for NDT instances across these phases. Below is a detailed description of NDT functionalities in each phase:

- NDT Creation Phase. The creation phase marks the beginning of the digital twin lifecycle, with the primary task of instantiating the digital twin. During this phase, the NDT MnS Consumer requests the creation of an NDT instance based on scenario-specific requirements and the capabilities of the NDT producer. For example, the consumer may request the NDT producer to create a new NDT from scratch. However, if pre-existing NDT components are available, the consumer should have the option to update existing components or orchestrate existing NDT components to generate a new NDT.

- The Configuration Phase aims to refine and adjust the created NDT to accurately reflect the status of the physical entity and adapt it to specific application scenarios. During this phase, the consumer may request data collection from the network and synchronization of the state with the NDT. This ensures that the digital twin is properly aligned with the physical entity and ready for subsequent simulation.

- NDT Running Phase. The running phase is the core of the NDT lifecycle, where it supports the simulation of network behavior and provides scenario-specific output based on the revious NDT configuration. During this phase, the NDT continuously interacts with the physical network, enabling real-time monitoring, predictive analytics, and optimization. The NDT MnS Consumer can leverage the digital twin to enhance network performance.

By managing these three stages effectively, NDT can achieve comprehensive lifecycle management from creation to running, providing robust support for the intelligent management and optimization of the network.

5.1.1.2	Requirements

REQ-NDT_LCM-1: The MnS Producer should have the capability allowing the consumer to perform life-cycle management on the NDT instance.

REQ-NDT_LCM-2: The MnS Producer should have the capability allowing the consumer to create, delete, orchestrate, update, and query the NDT in the NDT creation phase.

REQ-NDT_LCM -3: The MnS Producer should have the capability allowing the consumer to configure the NDT in the NDT configuration phase.

REQ-NDT_ LCM-4: The MnS Producer should have the capability allowing the consumer to trigger, suspend, resume, and teriminate the NDT in the NDT running phase.

End of First Changes