3GPP TSG-SA5 Meeting #159	S5-250171

Sophia-Antipolis, France, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.561 | CR |  | rev | - | Current version: | 0.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.561 Add Use Case and Requirements of Network Failure and Risk Prediction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NDT |  |  |  |  |  | Date: |  |  | 2024-01-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Use Case and Requirements of Network Failure and Risk Prediction has been discussed and agreed (in TR 28.915). This CR is to normatively propose the use case and requirements for the mentioned use case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add New Use Case and Requirements to support Network Failure and Risk Prediction |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The proposed use case is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5(new), 5.1(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | No impact on stage 3 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First Changes

5	Specification Level Requirements

5.1	Use cases

5.1.1	Network failure and risk prediction

5.1.1.1	Description

Each operations for network optimization and maintenance on mobile network may cause potential network failures and risks, especially high-risk operations, such as potentially dangerous configuration modification, policy modification, software version upgrade, and board switching, which may cause network congestion and network breakdown. To avoid any impact on the physical network, we cannot carry out the potential high-risk network operations in the physical network directly without concerning any consequences, and we cannot use the physical network to evaluate possible network optimization strategy and solution directly. Therefore, it is the better way that these network operations and possible network optimization solutions can be simulated and evaluated using network digital twin.

Using NDT, high-risk operations can identify whether these operations may cause potential network failures and risks by performing necessary digital twin related operations, e.g. simulation, verification and evaluation. The NDT can also optimize, verify and evaluate possible network policies and solutions for the further risk avoidance. After simulating and evaluating by the NDT, the results of high-risk operations prediction and evaluation should be notified back to 3GPP network system.

In addition, SLA degradation and failure of single node in mobile network can also be predicted using the NDT. When it is predicted that the network resources in the network domain are not enough to maintain the SLA or hardware resources failure at some time in the future, the NDT should warn 3GPP management system to take actions for network failure and risk avoidance.

Another scenario of network slice risk prediction is described in clause 5.2 [1]. Using the NDT to predict risks, the ZSM framework can identify risks of specific service or network slice profile parameters not being met due to changing traffic and network conditions (e.g. a MD not being able to provide the network slice latency it committed for ), and the NDT supports the ZSM framework to take actions before these risks materialize and therefore before the committed SLA/SLS are broken.

Therefore, 3GPP network system has needs to use network digital twin to predict and evaluate potential network failures and risks based on operator's requirements, such as predict possible network failures and risks posed by the high-risk operation. 3GPP management system can also use the NDT to evaluate and verify possible network policies and solutions to minimize the impact of high-risk operations.

5.1.1.2	Requirements

REQ-NDT_Failurerisk-1: The NDT should have the capability allowing the consumer to request evaluation of the impact of  high-risk operations.

REQ-NDT_ Failurerisk-2: The NDT should have the capability to provide the results of a simulation, including evaluation of impact of high-risk operations.

REQ-NDT_ Failurerisk-3: NDT should have a capability enabling the MnS consumer to configure the network scenario to be modelled for evaluating a Network failure.

REQ-NDT_ Failurerisk-4: NDT should have a capability enabling the MnS consumer to configure the parameters of the NDT instance of the objects to be modelled for evaluating a Network failure.

End of First Changes