3GPP TSG-SA5 Meeting #159	S5-250177

Sophia-Antipolis, France, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | draftCR | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Input to DraftCR Rel-19 TS 28.105 Add New Use Case and Requirements of Management of Federated Learning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT_Ph2 |  |  |  |  |  | Date: |  |  | 2024-01-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Use Case and Requirement of Management of Federated Learning has been discussed and agreed (in TR 28.858). This CR is to normatively propose the use case, requirements, and solution. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add New Use Case and Requirements to support Management of Federated Learning |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The proposed use case is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2b.2.x(new), 6.2b.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | No impact on stage 3. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First Changes

6.2b.2.x 	Management of Federated Learning

6.2b.2.x.1 	Description

Federated Learning (FL) is a distributed machine learning approach that allows multiple FL clients to collaboratively train an ML model on local datasets contained in each FL client without explicitly exchanging data samples.

FL is supported by a group of FL clients and FL server wherein FL client keeps the data localized and private and trains the ML model directly on the local nodes (client) where the data is generated or stored.

Federated learning can be categorized into two main types: Horizontal federated learning (HFL) and Vertical Federated Learning (VFL), based on the nature of the data distribution and the way the model training is orchestrated among participants. For HFL, the process typically includes FL Client discovery and selection, and ML model distribution and aggregation.

Figure 5.1.10.1-1: ML model distribution and aggregation for HFL

NOTE:	 A prior agreement needs to exist between the FL server and the FL clients to exchange ML models.

6.2b.2.x.2	Use cases

6.2b.2.x.2.1	Management of different roles in Federated Learning

For FL, an ML model is collaboratively trained by a group of FL clients (e.g. MTLF in NWDAFs) including one acting as FL server and the others acting as FL clients. Federated Learning training allows multiple FL clients to collaboratively train an ML model on local datasets, it means that the local training of each FL client needs to start and complete almost at the same time to ensure the performance of ML model aggregation performing in FL server.

For managing the FL, the ML training MnS consumer needs to know the FL clients and FL server involved in the FL, so that the consumer understands the impact of each one of them and can manage it correspondingly.

When receiving an ML Training request, the ML Training MnS Producer should evaluate whether FL process needs to be started according to the training requirements (e.g. minimal number of FL Clients, minimal number of total iterations, minimal number of data samples and available time of the FL Clients) provided by the ML training consumer. Based on the received requirements, the FL server may select appropriate FL Clients.

To evaluate the performance of FL, the consumer needs to know the performance of the final global ML model running on the local training data set of participating FL clients. For instance, if an FL server cannot generate a global ML model with satisfied performance for the FL clients, the consumer may interact with the MnS ML training producer to optimize the FL for future training, e.g. updating the criteria for selecting FL clients.

In addition, the consumer needs to get the information about the contribution of each FL client to the FL process, for instance, number of iterations the FL client participated in the FL, number of data examples the FL client used, training duration the FL Client performed.

Maintaining a Federation Learning process in FL execution phase is helpful to make the FL training efficient, which may include reselection, addition, or removal of FL Client(s). The consumer needs to get the information of candidate FL clients and contribution of each FL client to maintain the FL process.

End of First Changes

Start of Second Changes

6.2b.3	Requirements for ML model training

Table 6.2b.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-ML_TRAIN-FUN-01 | The ML training MnS producer shall have a capability allowing an authorized ML training MnS consumer to request ML model training. | ML model training requested by consumer (clause 6.2b.2.1) |
| REQ- ML_TRAIN-FUN-02 | The ML training MnS producer shall have a capability allowing the authorized ML training MnS consumer to specify the data sources containing the candidate training data for ML model training. | ML model training requested by consumer (clause 6.2b.2.1) |
| REQ- ML_TRAIN-FUN-03 | The ML training MnS producer shall have a capability allowing the authorized ML training MnS consumer to specify the AI/ML inference name of the ML model to be trained. | ML model training requested by consumer (clause 6.2b.2.1) |
| REQ- ML_TRAIN-FUN-04 | The ML training MnS producer shall have a capability to provide the training result to the ML training MnS consumer. | ML model training requested by consumer (clause 6.2b.2.1), ML model training initiated by producer (clause 6.2b.2.2) |
| REQ- ML_TRAIN-FUN-05 | The ML training MnS producer shall have a capability allowing an authorized ML training MnS consumer to configure the thresholds of the performance measurements and/or KPIs to trigger the re-training of an ML model. (See Note) | ML model training initiated by producer (clause 6.2b.2.2) |
| REQ- ML_TRAIN-FUN-06 | The ML training MnS producer shall have a capability to provide the version number of the ML model when it is generated by ML model re-training to the authorized ML training MnS consumer. | ML model training requested by consumer (clause 6.2b.2.1), ML model training initiated by producer (clause 6.2b.2.2) |
| REQ- ML_TRAIN-FUN-07 | The ML training MnS producer shall have a capability allowing an authorized ML training MnS consumer to manage the training process, including starting, suspending, or resuming the training process, and configuring the ML context for ML model training. | ML model training requested by consumer (clause 6.2b.2.1), ML model training initiated by producer (clause 6.2b.2.2), ML model joint training (clause 6.2b.2.6) |
| REQ- ML_TRAIN-FUN-08 | The ML training MnS producer should have a capability to provide the grouping of ML models to an authorized ML training MnS consumer to enable coordinated inference. | ML model joint training (clause 6.2b.1.2.6) |
| REQ- ML_TRAIN-FUN-09 | The ML training MnS producer should have a capability to allow an authorized ML training MnS consumer to request joint training of a group of ML models. | ML model joint training (clause 6.2b.2.6) |
| REQ- ML_TRAIN-FUN-10 | The ML training MnS producer should have a capability to jointly train a group of ML models and provide the training results to an authorized consumer. | ML model joint training (clause 6.2b.2.6) |
| REQ-ML_SELECT-01 | 3GPP management system shall have a capability to enable an authorized ML training MnS consumer to discover the properties of available ML models including the contexts under which each of the models were trained. | ML model and ML model selection (clause 6.2b.2.3) |
| REQ-ML_SELECT-02 | 3GPP management system shall have a capability to enable an authorized ML training MnS consumer to select an ML model to be used for inference. | ML models and ML model selection (clause 6.2b.2.3) |
| REQ-ML_SELECT-03 | 3GPP management system shall have a capability to enable an authorized ML training MnS consumer to request for information and be informed about the available alternative ML models of differing complexity and performance. | ML model and ML model selection (clause 6.2b.2.3) |
| REQ-ML_SELECT-04 | The 3GPP management system shall have a capability to provide a selected ML model to the authorized ML training MnS consumer. | ML model and ML model selection (clause 6.2b.2.3) |
| REQ-ML_TRAIN- MGT-01 | The ML training MnS producer shall have a capability allowing an authorized consumer to manage and configure one or more requests for the specific ML model training, e.g. to modify the request or to delete the request. | ML model training requested by consumer (clause 6.2b.2.1), Managing ML model Training Processes (clause 6.2b.2.4) |
| REQ-ML_TRAIN- MGT-02 | The ML training MnS producer shall have a capability allowing an authorized ML training MnS consumer to manage and configure one or more training processes, e.g. to start, suspend or restart the training. | ML model training requested by consumer (clause 6.2b.2.1), Managing ML model training processes (clause 6.2b.2.4) |
| REQ-ML_TRAIN- MGT-03 | 3GPP management system shall have a capability to enable an authorized ML training MnS consumer (e.g. the function/model different from the function that generated a request for ML model training) to request for a report on the outcomes of a specific training instance. | Managing ML model training processes (clause 6.2b.2.4) |
| REQ-ML_TRAIN- MGT-04 | 3GPP management system shall have a capability to enable an authorized ML training MnS consumer to define the reporting characteristics related to a specific training request or training instance. | Managing ML model training processes (clause 6.2b.2.4) |
| REQ-ML_TRAIN- MGT-05 | 3GPP management system shall have a capability to enable the ML training function to report to any authorized ML training MnS consumer about specific ML model training process and/or report about the outcomes of any such ML model training process. | Managing ML model training processes (clause 6.2b.2.4) |
| REQ-ML_ERROR-01 | The 3GPP management system shall enable an authorized consumer of data services (e.g. an ML training function) to request from a producer of data services a Value Quality Score of the data, which is the numerical value that represents the dependability/quality of a given observation and measurement type. | Handling errors in data and ML decisions (clause 6.2b.2.5) |
| REQ-ML_ERROR-02 | The 3GPP management system shall enable an authorized consumer of AI/ML decisions (e.g. a controller) to request ML decision confidence score which is the numerical value that represents the dependability/quality of a given decision generated by an AI/ML inference function. | Handling errors in data and ML decisions (clause 6.2b.2.5) |
| REQ-ML_ERROR-03 | The 3GPP management system shall have a capability to enable an authorized consumer to provide to the ML Training MnS producer, a training data quality score, which is the numerical value that represents the dependability/quality of a given observation and measurement type. | Handling errors in data and ML decisions (clause 6.2b.2.5) |
| REQ-ML_ERROR-04 | The 3GPP management system shall enable a producer of ML decisions (e.g. an AI/ML inference function) to provide to an authorized consumer of ML decisions (e.g. a controller) an AI/ML decision confidence score which is the numerical value that represents the dependability/quality of a given decision generated by the AI/ML inference function. | Handling errors in data and ML decisions (clause 6.2b.2.5) |
| REQ-ML_VLD-01 | The ML training MnS producer should have a capability to validate the ML models during the ML model training process and report the performance of the ML models on both the training data and validation data to the authorized consumer. | ML model validation performance reporting (clause 6.2b.2.7) |
| REQ-ML_VLD-02 | The ML training MnS producer should have a capability to report the ratio (in terms of quantity of data samples) of the training data and validation data used during the ML model training and validation process. | ML model validation performance reporting (clause 6.2b.2.7) |
| REQ-TRAIN_EFF-01 | The 3GPP management system should have the capability to allow an authorized consumer to configure an ML training function to report the effectiveness of data used for model training. | Training data effectiveness reporting (clause 6.2b.2.8) |
| REQ-ML_TRAIN_PM-1 | The ML Training MnS producer should have a capability to allow an authorized consumer to get the capabilities about what kind of ML models the ML training function is able to train. | Performance indicator selection for ML model training (clause 6.2b.2.9.2) |
| REQ-ML_TRAIN_PM-2 | The ML Training MnS producer should have a capability to allow an authorized consumer to query what performance indicators are supported by the ML model training for each ML model. | Performance indicator selection for ML model training (clause 6.2b.2.9.2) |
| REQ-ML_TRAIN_PM-3 | The ML Training MnS producer should have a capability to allow an authorized consumer to select the performance indicators from those supported by the ML training function for reporting the training performance for each ML model. | Performance indicator selection for ML model training (clause 6.2b.2.9.2) |
| REQ-ML_TRAIN_PM-4 | The ML Training MnS producer should have a capability to allow an authorized consumer to provide the performance requirements for the ML model training using the selected the performance indicators from those supported by the ML training function. | Performance indicator selection for ML model training (clause 6.2b.2.9.2) |
| REQ-FL_MGMT-01 | The ML training MnS producer should have a capability allowing an authorized consumer to discover the FL roles (FL server or FL client) in Federated Learning | Management of different roles in Federated Learning (clause 6.2b.2.x.2.1) |
| REQ-FL_MGMT-02 | The ML training MnS producer should have a capability allowing an authorized consumer to provide FL training requirements to the MnS Producer. | Management of different roles in Federated Learning (clause 6.2b.2.x.2.1) |
| REQ-FL_MGMT-03 | The ML training MnS producer should have a capability allowing an authorized consumer to provide requirements for selecting FL clients in Federated Learning to the MnS Producer. | Management of different roles in Federated Learning (clause 6.2b.2.x.2.1) |
| REQ-FL_MGMT-04 | The ML training MnS producer should have a capability allowing an authorized consumer to get the performance of ML model on each participating FL client. | Management of different roles in Federated Learning (clause 6.2b.2.x.2.1) |
| REQ-FL_MGMT-05 | The ML training MnS producer should have a capability to report the information about the contribution of each FL client to the FL process to MnS consumer. | Management of different roles in Federated Learning (clause 6.2b.2.x.2.1) |
| REQ-FL_MGMT-06 | The ML training MnS producer should have a capability to report the candidate FL Clients for the FL process. | Management of different roles in Federated Learning (clause 6.2b.2.x.2.1) |
| REQ-FL_MGMT-07 | The ML training MnS producer should have a capability allowing an authorized consumer to reselect, add, or remove the FL Clients in the FL Process according to the candidate FL clients. | Management of different roles in Federated Learning (clause 6.2b.2.x.2.1) |
| NOTE: The performance measurements and KPIs are specific to each type (i.e., the inference type that the ML model supports) of ML model. |  |  |

End of Second Changes