3GPP TSG-SA5 (Telecom Management)	S5-110540

Meeting SA5#75, 24 – 28 Jan 2011; Sorrento, ITALY 	revision of S5-110526

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.421 | CR | 0029 |  rev | 3 |  Current version: | 10.1.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Add requirements for security and privacy of UE-based Network Performance Measurements / MDT as required by SA3 (Security) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Telecom Italia, China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | OAM-PM-UE |  |  |  |  |  |  | Date:  |  | 28/01/2011 |
| Category:  | C |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) |  |
| Reason for change:  |  | SA3 analyzed the solutions specified for UE based network performance measurement / MDT and indicated that machanisms shall be envisaged to ensure security and user privacy |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Introduce relevant requirements at business and specification level |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | UE privacy and security requirements will not be fulfilled |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

First change

# 6	Requirements for managing UE based network performance measurements

## 6.1		Business Level Requirements

REQ-MDT-CON-1	The Operator shall be able to collect measurements for Network Performance Management purposes from UEs attached to their network.

REQ-MDT-CON-2 The collected measurements shall be made available in a centralised entity.

REQ-MDT-CON-3	Operator shall be able to select specific set of subscribers for the measurement collection based on IMSI.

REQ-MDT-CON-4 	Operator shall be able to select specific set of devices for the measurement collection based on IMEI(SV).

REQ-MDT-CON-5	Operator shall be able to select specific set of devices for the measurement collection based on geographical area.

REQ-MDT-CON-6	Operator shall be able to select specific set of devices for the measurement collection based on device capabilities.

REQ-MDT-CON-7	Operator shall be able to select specific set of subscribers based on IMSI and a geographical areas for the measurement collection.

REQ-MDT-CON-8	Operator shall be able to select specific set of devices based on IMEI(SV) and a geographical area for the measurement collection.

REQ-MDT-CON-9	Operator shall be able to configure set of device capabilities and a geographical area for the measurement collection.

REQ-MDT-CON-10	Operator shall be able to select specific set of subscribers based on IMSI and a set of device capabilities and a geographical area for the measurement collection.

REQ-MDT-CON-11	Operator shall be able to select specific set of device IMEI(SV) and capabilities and a geographical area for the measurement collection.

REQ-MDT-CON-12	Operator shall be able to select set of subscribers based on IMSI and a set of device capabilities for the measurement collection

REQ-MDT-CON-13	Operator shall be able to select specific set of device IMEI(SV) and capabilities for the measurement collection.

REQ-MDT-CON-14 The operator shall be able to configure the duration of the measurement collection

REQ-MDT-CON-15	The operator shall be able to configure the UE measurement types and events for collection

REQ-MDT-CON-16	The operator shall be able to configure the type of UE measurement reporting and log formats i.e.  raw measurement results or type of  measurement aggregation (e.g. statistical aggregation of raw measurement results, sampling of raw measurement results, etc.)

REQ-MDT-CON-17 The management of UE based network performance measurements shall be independent from the management of SON functionalities

REQ-MDT-CON-XX The management of UE based network performance measurements shall allow the network operator to control whether or not it is possible to link a measurement result and related information (e.g. location information) to a real user ID such as an IMSI/IMEI.

End of changes