3GPP TSG-SA5 (Telecom Management)	S5-111417

SA5#76, 28 Feb - 4 Mar 2011; San Diego, USA	revision of S5-111211

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.297 | CR | 0010 |  rev | 1 |  Current version: | 9.0.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Release considerations beyond Release 9 for CDR headers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Alcatel-Lucent |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | CH10 |  |  |  |  |  |  | Date:  |  | 04/03/2011 |
| Category:  | C |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change:  |  | - “Release Identifier “ cannot be identified beyond Rel-10 (only 3 bits). - As the CDR header is used to identify the CDR ASN.1 dictionary, and the ASN.1 syntax is defined in 32.298, it is seen more accurate to use 32.298 version identifier. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | - New extension field for indicating Rel-10 in “Release Identifier”, allowing future releases to be also considered. - Version identifier is for 32.298, and TS number is for the domain CDRs |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | CDRs file cannot be transferred for Rel-10 |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 6.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | x | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

First change

### 6.1.2	CDR header format

The exact format of the CDR header is given in table 6.1.2.

Table 6.1.2: Format of CDR Header

| Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| 1..2 | CDR length |  |  |  |  |  |  |  |
| 3 | Release Identifier |  |  | Version Identifier |  |  |  |  |
| 4 | Data Record Format |  |  | TS number |  |  |  |  |
| 5 | Release Identifier extension |  |  |  |  |  |  |  |

The following subclauses specify the contents and encoding of the CDR header fields.

#### 6.1.2.1	CDR length

This two octet field contains a binary value that specifies the length of the subsequent CDR, excluding the 5 header octets. The value of "65535" (all bits set to "1")in the CDR length field implies that it is reserved for future extensions and shall therefore not be used.

#### 6.1.2.2	Release Identifier

This three bit field contains a binary value that identifies the 3GPP Release of the Technical Specification indicated in "TS number" (see clause 6.1.2.5). Its value is set / used according to the following rules:

Table 6.1.2.2: Release Identifier

| Release Identifier (3 bits) | 3GPP Release | Indicates 3GPP "TS number" for the release | Comment |
| --- | --- | --- | --- |
| "0" | Rel-99 | 32.005 or 32.015 | this parameter shall be ignored |
| "1" | Rel-4 | 32.205, 32.215 or 32.235 |  |
| "2" | Rel-5 | 32.205, 32.215, 32.225 or 32.235 |  |
| "3" | Rel-6 | 32.250, 32.251, 32.252, 32.260, 32.270, 32.271, 32.272 or 32.273 |  |
| "4" | Rel-7 | 32.250, 32.251, 32.252, 32.260, 32.270, 32.271, 32.272 or 32.273 |  |
| "5" | Rel-8 | 32.250, 32.251, 32.252, 32.260, 32.270, 32.271, 32.272 , 32.273, 32.274 or 32.275 |  |
| "6" | Rel-9 | 32.250, 32.251, 32.252, 32.260, 32.270, 32.271, 32.272 , 32.273, 32.274 or 32.275 |  |
| "7" | Beyond Rel-9 | The 3GPP Release for “TS number” is included in “Release Identifier extension” field | this parameter is used as an indicator |

#### 6.1.2.3	Version Identifier

This five bit field contains a binary value that identifies the version of the TS 32.298 [51]. Its value corresponds to the middle digit of the version number of that TS, as indicated on the TS cover sheet.

#### 6.1.2.4	Data Record Format

This three bit field contains a binary value that identifies the CDR encoding according to the following table:

- "1" signifies the use of Basic Encoding Rules (BER);

- "2" signifies the use of unaligned basic Packed Encoding Rules (PER);

- "3" signifies the use of aligned basic Packed Encoding Rules (PER);

- "4" signifies the use of XML Encoding Rules (XER).

#### 6.1.2.5	TS number

This five bit field contains a binary value that identifies the number of the TS associated to the domain CDRs encoding according to table 6.1.2.5:

Table 6.1.2.5: "TS number" Identifier

| "TS number" Identifier (5 bits) | TS number |
| --- | --- |
| 0 | 32.005 |
| 1 | 32.015 |
| 2 | 32.205 |
| 3 | 32.215 |
| 4 | 32.225 |
| 5 | 32.235 |
| 6 | 32.250 |
| 7 | 32.251 |
| 8 | 32.252 |
| 9 | 32.260 |
| 10 | 32.270 |
| 11 | 32.271 |
| 12 | 32.272 |
| 13 | 32.273 |
| 14 | 32.275 |
| 15 | 32.274 |
| NOTE: 16-31 are for future use. |  |

#### 6.1.2.6	Release Identifier extension

This eight bit field contains a binary value that identifies the 3GPP Release of the TS 32.298 [51], for Releases beyond Rel-9. Its value is set / used according to the following rules:

Table 6.1.2.6: Release Identifier extension

| Release Identifier (8 bits) | 3GPP Release | Comment |  |
| --- | --- | --- | --- |
| "0" | Rel-10 |  |  |
| "1" | Rel-11 |  |  |
|  |  |  | NOTE: Additional "Release Identifier extension" value incremented by "1" indicates subsequent releases |

End of changes