3GPP TSG-SA5 (Telecom Management)	S5-111422

SA5#76, 28 Feb - 4 Mar 2011; San Diego, USA	revision of S5-111107

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.299 | CR | 0345 |  rev | 1 |  Current version: | 10.0.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Addition of IARI in IMS charging information, alignment with TS 22.115 and TS 23.228 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | CH10 |  |  |  |  |  |  | Date:  |  | 04/03/2011 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change:  |  | According to TS 22.115, service identification shall be available in IMS charging information. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | IMS Application Reference ID (IARI) is added on Rf from P-CSCF, plus the CDRs from these functions. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | IMS charging doesn’t fulfill stage 1 requirements. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 7.2, 7.2.74A (new), 7.2.77 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  | X |  | O&M Specifications |  |  | 32.260, 32.298 |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

First change

## 7.2	3GPP specific AVPs

For the purpose of offline charging additional AVPs are used in ACR / ACA and for online charging additional AVPs are used in CCR / CCA. All 3GPP specific AVPs mentioned are relevant for both offline and online charging unless specifically excluded. The information is summarized in the following table along with the AVP flag rules.

The 3GPP Charging Application uses the value 10415 (3GPP) as Vendor-Id.

Detailed descriptions of AVPs that are used specifically for 3GPP charging are provided in the subclauses below the table. However, for AVPs that are just borrowed from other applications only the reference (e.g. TS 29.229 [204]), is provided in the following table and the detailed description is not repeated.

Where 3GPP RADIUS VSAs are re-used, they shall be translated to Diameter AVPs as described in RFC 4005 [407] with the exception that the 'M' flag shall be set and the ''P' flag may be set.

Table 7.2: 3GPP specific AVPs

| AVP Name | AVP Code | Used in |  |  |  | Value Type | AVP Flag rules |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ACR | ACA | CCR | CCA |  | Must | May | Should not | Must not | May Encr. |
| 3GPP-Charging-Characteristics | 13 | X | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-Charging-Id | 2 | X | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-GGSN-MCC-MNC | 9 | X | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-IMSI | 1 | - | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-IMSI-MCC-MNC | 8 | X | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-MS-TimeZone | 23 | X | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-NSAPI | 10 | X | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-PDP-Type | 3 | X | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-RAT-Type | 21 | X | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-Selection-Mode | 12 | X | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-Session-Stop-Indicator | 11 | - | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-SGSN-MCC-MNC | 18 | X | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-User-Location-Info | 22 | X | - | X | - | refer [207] |  |  |  |  |  |
| Access-Network-Charging-Identifier-Value | 503 | X | - | X | - | refer [214] |  |  |  |  |  |
| Access-Network-Information | 1263 | X | - | X | - | OctetString | V,M | P |  |  | N |
| Account-Expiration | 2309 | - | - | - | X | Time | V,M | P |  |  | N |
| Accumulated-Cost | 2052 | - | - | - | X | Grouped | V,M | P |  |  | N |
| Adaptations | 1217 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Additional-Content-Information | 1207 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Additional-Type-Information | 1205 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Address-Data | 897 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Address-Domain | 898 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Addressee-Type | 1208 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Address-Type | 899 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| AF-Charging-Identifier | 505 | - | - | X | - | refer [214] |  |  |  |  |  |
| AF-Correlation-Information | 1276 | X | - | X | - | Grouped | V,M | P |  |  | N |
| Allocation-Retention-Priority | 1034 | X | - | X | - | refer [215] |  |  |  |  |  |
| Alternate-Charged-Party-Address | 1280 | X | - | - | - | UTF8string | V.M | P |  |  | N |
| AoC-Cost-Information | 2053 | - | - | X | X | Grouped | V,M | P |  |  | N |
| AoC-Format | 2310 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| AoC-Information | 2054 | - | - | - | X | Grouped | V.M | P |  |  | N |
| AoC-Request-Type | 2055 | - | - | X | - | Enumerated | V.M | P |  |  | N |
| AoC-Service | 2311 | - | - | X | - | Grouped | V,M | P |  |  | N |
| AoC-Service-Obligatory-Type | 2312 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| AoC-Service-Type | 2313 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| AoC-Subscription-Information | 2314 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Application-provided-called-party-address | 837 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Application-Server | 836 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Application-Server-ID | 2101 | X | - | X | - | refer[223] |  |  |  |  |  |
| Application-Server-Information | 850 | X | - | X | - | Grouped | V,M | P |  |  | N |
| Application-Service-Type | 2102 | X | - | X | - | refer[223] |  |  |  |  |  |
| Application-Session-ID | 2103 | X | - | X | - | refer[223] |  |  |  |  |  |
| Applic-ID | 1218 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Associated-Party-Address | 2035 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Associated-URI | 856 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Authorised-QoS | 849 | X | - | - | - | UTF8String | V,M | P |  |  | N |
| Aux-Applic-Info | 1219 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Base-Time-Interval | 1265 | - | - | - | X | Unsigned32 | V,M | P |  |  | N |
| Bearer-Service | 854 | X | - | - | - | OctetString | V,M | P |  |  | N |
| Called-Asserted-Identity | 1250 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Called-Party-Address | 832 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Calling-Party-Address | 831 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Carrier-Select-Routing-Information | 2023 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Cause-Code | 861 | X | - | X | - | Integer32 | V,M | P |  |  | N |
| CG-Address | 846 | X | - | X | - | Address | V,M | P |  |  | Y |
| Change-Condition | 2037 | X | - | - | - | Integer32 | V,M | P |  |  | N |
| Change-Time | 2038 | X | - | - | - | Time | V,M | P |  |  | N |
| Charged-Party | 857 | X | - | - | - | UTF8String | V,M | P |  |  | N |
| Charging-Characteristics-Selection-Mode | 2066 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| Charging-Rule-Base-Name | 1004 | X | - | X | - | refer [215] |  |  |  |  |  |
| Class-Identifier | 1214 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Client-Address | 2018 | - | - | X | - | Address | V,M | P |  |  | N |
| CN-IP-Multicast-Distribution | 921 | X | - | - | - | refer [207] |  |  |  |  |  |
| Content-Class | 1220 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Content-Disposition | 828 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Content-ID | 2116 | X | - | X | - | refer[223] |  |  |  |  |  |
| Content-Provider-ID | 2117 | X | - | X | - | refer[223] |  |  |  |  |  |
| Content-Length | 827 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| Content-Size | 1206 | - | - | X | - | Unsigned32 | V,M | P |  |  | N |
| CSG-Access-Mode | 2317 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| CSG-Id | 1437 | X | - | X | - | refer[219] |  |  |  |  |  |
| CSG-Membership-Indication | 2318 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| Content-Type | 826 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Current-Tariff | 2056 | - | - | X | X | Grouped | V,M | P |  |  | N |
| CUG-Information | 2304 | X | - | X | - | OctetString | V,M | P |  |  | N |
| Data-Coding-Scheme | 2001 | - | - | X | - | Integer32 | V,M | P |  |  | N |
| DCD-Information | 2115 | X | - | X | - | refer[223] |  |  |  |  |  |
| Deferred-Location-Event-Type | 1230 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Delivery-Report-Requested | 1216 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Delivery-Status | 2104 | X | - | X | - | refer[223] |  |  |  |  |  |
| Destination-Interface | 2002 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Diagnostics | 2039 | X | - | - | - | Integer32 | V,M | P |  |  | N |
| Domain-Name | 1200 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| DRM-Content | 1221 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Dynamic-Address-Flag | 2051 | X | - | - | - | Enumerated | V,M | P |  |  | N |
| Early-Media-Description | 1272 | X | - | - | - | Grouped | V,M | P |  |  | N |
| Envelope | 1266 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Envelope-End-Time | 1267 | - | - | X | - | Time | V,M | P |  |  | N |
| Envelope-Reporting | 1268 | - | - | - | X | Enumerated | V,M | P |  |  | N |
| Envelope-Start-Time | 1269 | - | - | X | - | Time | V,M | P |  |  | N |
| Event | 825 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Event-Charging-TimeStamp | 1258 | - | - | X | - | Time | V,M | P |  |  | N |
| Event-Type | 823 | X | - | X | - | Grouped | V,M | P |  |  | N |
| Expires | 888 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| File-Repair-Supported | 1224 | X | - | X | - | Enumerated | V,M | P |  |  | Y |
| Flows | 510 | - | - | X | - | refer [214] |  |  |  |  |  |
| GGSN-Address | 847 | X | - | X | - | Address | V,M | P |  |  | N |
| Guaranteed-Bitrate-UL | 1026 | X | - | X | - | refer[215] |  |  |  |  |  |
| IM-Information | 2110 | X | - | X | - | refer[223] |  |  |  |  |  |
| IMS-Application-Reference-Identifier | 2601 | X | - | - | - | UTF8String | V,M | P |  |  | N |
| IMS-Charging-Identifier | 841 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| IMS-Communication-Service-Identifier | 1281 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| IMS-Information | 876 | X | - | X | - | Grouped | V,M | P |  |  | N |
| IMSI-Unauthenticated-Flag | 2308 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| Incoming-Trunk-Group-Id | 852 | X | - | - | - | UTF8String | V,M | P |  |  | N |
| Incremental-Cost | 2062 | - | - | X | X | Grouped | V,M | P |  |  | N |
| Interface-Id | 2003 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Interface-Port | 2004 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Interface-Text | 2005 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Interface-Type | 2006 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Inter-Operator-Identifier | 838 | X | - | X | - | Grouped | V,M | P |  |  | N |
| LCS-Client-Dialed-By-MS | 1233 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| LCS-Client-External-ID | 1234 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| LCS-Client-Id | 1232 | - | - | X | - | Grouped | V,M | P |  |  | N |
| LCS-Client-Name | 1231 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| LCS-Client-Name | 1235 | - | - | X | - | Grouped | V,M | P |  |  | N |
| LCS-Client-Type | 1241 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| LCS-Data-Coding-Scheme | 1236 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| LCS-Format-Indicator | 1237 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| LCS-Information | 878 | - | - | X | - | Grouped | V,M | P |  |  | N |
| LCS-Name-String | 1238 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| LCS-Requestor-Id | 1239 | - | - | X | - | Grouped | V,M | P |  |  | N |
| LCS-Requestor-Id-String | 1240 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Local-Sequence-Number | 2063 | X | - | - | - | Unsigned32 | V,M | P |  |  | N |
| Location-Estimate | 1242 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Location-Estimate-Type | 1243 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Location-Type | 1244 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Low-Balance-Indication | 2020 | - | - | - | X | Enumerated | V,M | P |  |  | N |
| Mandatory-Capability | 604 | X | - | - | - | refer [204] |  |  |  |  |  |
| Max-Requested-Bandwidth-DL | 515 | X | - | X | - | refer [214] |  |  |  |  |  |
| Max-Requested-Bandwidth-UL | 516 | X | - | X | - | refer [214] |  |  |  |  |  |
| MBMS-2G-3G-Indicator | 907 | X | - | X | - | refer [207] |  |  |  |  |  |
| MBMS GW-Address | 2307 | X | - | - | - | Address | V,M | P |  |  | N |
| MBMS-Information | 880 | X | - | X | - | Grouped | V,M | P |  |  | N |
| MBMS-Service-Area | 903 | X | - | X | - | refer [207] |  |  |  |  |  |
| MBMS-Service-Type | 906 | X | - | X | - | refer [207] |  |  |  |  |  |
| MBMS-Session-Identity | 908 | X | - | X | - | refer [207] |  |  |  |  |  |
| MBMS-User-Service-Type | 1225 | X | - | X | - | Enumerated | V,M | P |  |  | Y |
| Media-Initiator-Flag | 882 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| Media-Initiator-Party | 1288 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Message-Body | 889 | X | - | X | - | Grouped | V,M | P |  |  | N |
| Message-Class | 1213 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Message-ID | 1210 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Message-Size | 1212 | - | - | X | - | Unsigned32 | V,M | P |  |  | N |
| Message-Type | 1211 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| MMBox-Storage-Requested | 1248 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| MM-Content-Type | 1203 | - | - | X | - | Grouped | V,M | P |  |  | N |
| MMS-Information | 877 | - | - | X | - | Grouped | V,M | P |  |  | N |
| MMTel-Information | 2030 | X | - | X | - | Grouped | V,M | P |  |  | N |
| MSISDN | 701 | - | - | X | - | refer [221] |  |  |  |  |  |
| Next-Tariff | 2057 |  |  | X | X | Grouped | V/M | P |  |  | N |
| Node-Functionality | 862 | X | -. | X | - | Enumerated | V,M | P |  |  | N |
| Node-Id | 2064 | X | - | - | X | UTF8String | V,M | P |  |  | N |
| Number-Of-Diversions | 2034 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| Number-Of-Messages-Sent | 2019 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| Number-Of-Messages-Successfully-Exploded | 2111 | X | - | X | - | refer[223] |  |  |  |  |  |
| Number-Of-Messages-Successfully-Sent | 2112 | X | - | X | - | refer[223] |  |  |  |  |  |
| Number-Of-Participants | 885 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| Number-Of-Received-Talk-Bursts | 1282 | X | - | - | - | Unsigned32 | V,M | P |  |  | N |
| Number-Of-Talk-Bursts | 1283 | X | - | - | - | Unsigned32 | V,M | P |  |  | N |
| Number-Portability-Routing-Information | 2024 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Offline-Charging | 1278 | - | - | - | X | Grouped | V,M | P |  |  | N |
| Online-Charging-Flag | 2303 | X | - | - | - | Enumerated | V,M | P |  |  | N |
| Optional-Capability | 605 | X | - | - | - | refer [204] |  |  |  |  |  |
| Originating-IOI | 839 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Originator-SCCP-Address | 2008 | - | - | X | - | Address | V,M | P |  |  | N |
| Originator | 864 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| Originator-Address | 886 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Originator-Received-Address | 2027 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Originator-Interface | 2009 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Outgoing-Session-Id | 2320 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Outgoing-Trunk-Group-Id | 853 | X | - | - | - | UTF8String | V,M | P |  |  | N |
| Participant-Access-Priority | 1259 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| Participant-Action-Type | 2049 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| Participant-Group | 1260 | X | - | X | - | Grouped | V,M | P |  |  | N |
| Participants-Involved | 887 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| PDG-Address | 895 | X | - | X | - | Address | V,M | P |  |  | N |
| PDG-Charging-Id | 896 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| PDN-Connection-ID | 2050 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| PDP-Address | 1227 | X | - | X | - | Address | V,M | P |  |  | Y |
| PDP-Context-Type | 1247 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| PoC-Change-Condition | 1261 | X | - | - | - | Enumerated | V,M | P |  |  | N |
| PoC-Change-Time | 1262 | X | - | - | - | Time | V,M | P |  |  | N |
| PoC-Controlling-Address | 858 | X | - | X |  | UTF8String | V,M | P |  |  | N |
| PoC-Event-Type | 2025 | X | - | X |  | Enumerated | V,M | P |  |  | N |
| PoC-Group-Name | 859 | X | - | X |  | UTF8String | V,M | P |  |  | N |
| PoC-Information | 879 | X | - | X | - | Grouped | V,M | P |  |  | N |
| PoC-Server-Role | 883 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| PoC-Session-Id | 1229 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| PoC-Session-Initiation-type | 1277 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| PoC-Session-Type | 884 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| PoC-User-Role | 1252 | X | - | X | - | Grouped | V,M | P |  |  | N |
| PoC-User-Role-IDs | 1253 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| PoC-User-Role-info-Units | 1254 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| Positioning-Data | 1245 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Preferred-AoC-Currency | 2315 | - | - | X | - | Unsigned32 | V,M | P |  |  | N |
| Priority | 1209 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Priority-Level | 1046 | X | - | X | - | refer [215] |  |  |  |  |  |
| PS-Append-Free-Format-Data | 867 | X | - | - | X | Enumerated | V,M | P |  |  | N |
| PS-Free-Format-Data | 866 | X | - | - | X | OctetString | V,M | P |  |  | N |
| PS-Furnish-Charging-Information | 865 | X | - | - | X | Grouped | V,M | P |  |  | N |
| PS-Information | 874 | X | - | X | X | Grouped | V,M | P |  |  | N |
| QoS-Information | 1016 | X | - | - | - | refer [215] |  |  |  |  |  |
| QoS-Class-Identifier | 1028 | X | - | X | - | refer [215] |  |  |  |  |  |
| Quota-Consumption-Time | 881 | - | - | - | X | Unsigned32 | V,M | P |  |  | N |
| Quota-Holding-Time | 871 | - | - | - | X | Unsigned32 | V,M | P |  |  | N |
| RAI | 909 | X | - | X | - | refer [207] |  |  |  |  |  |
| Rate-Element | 2058 | - | - | X | X | Grouped | V,M | P |  |  | N |
| Read-Reply-Report-Requested | 1222 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Reason-Code | 2316 | - | - | X | X | Enumerated | V,M | P |  |  | N |
| Real-Time-Tariff-Information | 2305 | X | - | - | - | Grouped | V,M | P |  |  | N |
| Received-Talk-Burst-Time | 1284 | X | - | - | - | Unsigned32 | V,M | P |  |  | N |
| Received-Talk-Burst-Volume | 1285 | X | - | - | - | Unsigned32 | V,M | P |  |  | N |
| Recipient-Address | 1201 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Recipient-Info | 2026 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Recipient-Received-Address | 2028 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Recipient-SCCP-Address | 2010 | - | - | X | - | Address | V,M | P |  |  | N |
| Refund-Information | 2022 | - | - | X | X | OctetString | V,M | P |  |  | N |
| Remaining-Balance | 2021 | - | - | - | X | Grouped | V,M | P |  |  | N |
| Reply-Applic-ID | 1223 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Reply-Path-Requested | 2011 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Reporting-Reason | 872 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Requested-Party-Address | 1251 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Required-MBMS-Bearer-Capabilities | 901 | X | - | X | - | refer [207] |  |  |  |  |  |
| Role-Of-Node | 829 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| Scale-Factor | 2059 | - | - | X | X | Grouped | V,M | P |  |  | N |
| SDP-Answer-Timestamp | 1275 | X | - | - | - | Time | V,M | P |  |  | N |
| SDP-Media-Component | 843 | X | - | X | - | Grouped | V,M | P |  |  | N |
| SDP-Media-Description | 845 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| SDP-Media-Name | 844 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| SDP-Offer-Timestamp | 1274 | X | - | - | - | Time | V,M | P |  |  | N |
| SDP-Session-Description | 842 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| SDP-TimeStamps | 1273 | X | - | - | - | Grouped | V,M | P |  |  | N |
| SDP-Type | 2036 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| Served-Party-IP-Address | 848 | X | - | - | - | Address | V,M | P |  |  | N |
| Server-Capabilities | 603 | X | - | - | - | refer [204] |  |  |  |  |  |
| Server-Name | 602 | X | - | - | - | refer [204] |  |  |  |  |  |
| Service-Data-Container | 2040 | X | - | - | - | Grouped | V,M | P |  |  | N |
| Service-Generic-Information | 1256 | X | - | X | - | Refer[223] |  |  |  |  |  |
| Service-Id | 855 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Service-Information | 873 | X | - | X | X | Grouped | V,M | P |  |  | N |
| Service-Mode | 2032 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| Service-Specific-Data | 863 | X | - | - | - | UTF8String | V,M | P |  |  | N |
| Service-Specific-Info | 1249 | X | - | - | - | Grouped | V,M | P |  |  | N |
| Service-Specific-Type | 1257 | X | - | - | - | Unsigned32 | V,M | P |  |  | N |
| Serving-Node-Type | 2047 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| Service-Type | 2031 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| Session-Priority | 650 | X | - | X | - | Refer [204] |  |  |  |  |  |
| SGSN-Address | 1228 | X | - | X | - | Address | V,M | P |  |  | N |
| SGW-Change | 2065 | X | - | - | - | Enumerated | V,M | P |  |  | N |
| SIP-Method | 824 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| SIP-Request-Timestamp-Fraction | 2301 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| SIP-Request-Timestamp | 834 | X | - | X | - | Time | V,M | P |  |  | N |
| SIP-Response-Timestamp-Fraction | 2302 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| SIP-Response-Timestamp | 835 | X | - | X | - | Time | V,M | P |  |  | N |
| SM-Discharge-Time | 2012 | - | - | X | - | Time | V,M | P |  |  | N |
| SM-Message-Type | 2007 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| SM-Protocol-ID | 2013 | - | - | X | - | OctetString | V,M | P |  |  | N |
| SMSC-Address | 2017 | - | - | X | - | Address | V,M | P |  |  | N |
| SMS-Information | 2000 | - | - | X | - | Grouped | V,M | P |  |  | N |
| SMS-Node | 2016 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| SM-Service-Type | 2029 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| SM-Status | 2014 | - | - | X | - | OctetString | V,M | P |  |  | N |
| SM-User-Data-Header | 2015 | - | - | X | - | OctetString | V,M | P |  |  | N |
| Start-Time | 2041 | X | - | - | - | Time | V,M | P |  |  | N |
| Stop-Time | 2042 | X | - | - | - | Time | V,M | P |  |  | N |
| Submission-Time | 1202 | - | - | X | - | Time | V,M | P |  |  | N |
| Subscriber-Role | 2033 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| Supplementary-Service | 2048 | X | - | X | - | Grouped | V,M | P |  |  | N |
| Talk-Burst-Exchange | 1255 | X | - | - | - | Grouped | V,M | P |  |  | N |
| Talk-Burst-Time | 1286 | X | - | - | - | Unsigned32 | V,M | P |  |  | N |
| Talk-Burst-Volume | 1287 | X | - | - | - | Unsigned32 | V,M | P |  |  | N |
| Tariff-Information | 2060 | X | - | X | X | Grouped | V,M | P |  |  | N |
| Tariff-XML | 2306 | X | - | - | - | UTF8String | V,M | P |  |  | N |
| Terminal-Information | 1401 | X | - | X | - | refer [219] |  |  |  |  |  |
| Terminating-IOI | 840 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Time-First-Usage | 2043 | X | - | - | - | Time | V,M | P |  |  | N |
| Time-Last-Usage | 2044 | X | - | - | - | Time | V,M | P |  |  | N |
| Time-Quota-Mechanism | 1270 | - | - | - | X | Grouped | V,M | P |  |  | N |
| Time-Quota-Threshold | 868 | - | - | - | X | Unsigned32 | V,M | P |  |  | N |
| Time-Quota-Type | 1271 | - | - | - | X | Enumerated | V,M | P |  |  | N |
| Time-Stamps | 833 | X | - | X | - | Grouped | V,M | P |  |  | N |
| Time-Usage | 2045 | X | - | - | - | Unsigned32 | V,M | P |  |  | N |
| TMGI | 900 | X | - | X | - | refer [207] |  |  |  |  |  |
| Token-Text | 1215 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Total-Number-Of-Messages-Exploded | 2113 | X | - | X | - | refer[223] |  |  |  |  |  |
| Total-Number-Of-Messages-Sent | 2114 | X | - | X | - | refer[223] |  |  |  |  |  |
| Traffic-Data-Volumes | 2046 | X | - | - | - | Grouped | V,M | P |  |  | N |
| Trigger | 1264 | - | - | X | X | Grouped | V,M | P |  |  | N |
| Trigger-Type | 870 | - | - | X | X | Enumerated | V,M | P |  |  | N |
| Trunk-Group-Id | 851 | X | - | - | - | Grouped | V,M | P |  |  | N |
| Type-Number | 1204 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Unit-Cost | 2061 | - | - | X | X | Grouped | V,M | P |  |  | N |
| Unit-Quota-Threshold | 1226 | - | - | - | X | Unsigned32 | V,M | P |  |  | N |
| User-CSG-Information | 2319 | X | - | X | X | Grouped | V,M | P |  |  | N |
| User-Data | 606 | X | - | - | - | refer [204] | V,M | P |  |  | N |
| User-Participating-Type | 1279 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| User–Session-Id | 830 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| VAS-Id | 1102 | - | - | X | - | refer [213] |  |  |  |  |  |
| VASP-Id | 1101 | - | - | X | - | refer [213] |  |  |  |  |  |
| Volume-Quota-Threshold | 869 | - | - | - | X | Unsigned32 | V,M | P |  |  | N |
| WAG-Address | 890 | X | - | X | - | Address | V,M | P |  |  | N |
| WAG-PLMN-Id | 891 | X | - | X | - | OctetString | V,M | P |  |  | N |
| WLAN-Information | 875 | X | - | X | - | Grouped | V,M | P |  |  | N |
| WLAN-Radio-Container | 892 | X | - | X | - | Grouped | V,M | P |  |  | N |
| WLAN-Session-Id | 1246 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| WLAN-Technology | 893 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| WLAN-UE-Local-IPAddress | 894 | X | - | X | - | Address | V,M | P |  |  | N |

Next change

### 7.2.74A	IMS-Application Reference-Identifier AVP

The IMS-Application-Reference-Identifier AVP (AVP code 2601) is of type UTF8String and holds the IMS Application Reference Identifier (IARI) as contained in a SIP request to identify an IMS Application as defined in TS 24.229 [202].

Next change

### 7.2.77	IMS-Information AVP

The IMS-Information AVP (AVP code 876) is of type Grouped. Its purpose is to allow the transmission of additional IMS service specific information elements.

It has the following ABNF grammar:

IMS-Information :: = 	  < AVP Header: 876>

[ Event-Type ]

[ Role-Of-Node ]

{ Node-Functionality }

[ User-Session-Id ]

[ Outgoing-Session-Id ]

[ Session-Priority ]

* [ Calling-Party-Address ]

[ Called-Party-Address ]

* [ Called-Asserted-Identity ]

[ Number-Portability-Routing-Information ]

[ Carrier-Select-Routing-Information ]

[ Alternate-Charged-Party-Address ]

[ Requested-Party-Address ]

* [ Associated-URI ]

[ Time-Stamps ]

* [ Application-Server-Information ]

* [ Inter-Operator-Identifier ]

[ IMS-Charging-Identifier ]

* [ SDP-Session-Description ]

* [ SDP-Media-Component ]

[ Served-Party-IP-Address ]

[ Server-Capabilities ]

[ Trunk-Group-ID ]

[ Bearer-Service ]

[ Service-Id ]

* [ Service-Specific-Info ]

* [ Message-Body ]

[ Cause-Code ]

[ Access-Network-Information ]

* [ Early-Media-Description ]

[ IMS-Communication-Service-Identifier ]

[ IMS-Application-Reference-Identifier ]

[ Online-Charging-Flag ]

[ Real-Time-Tariff-Information ]

[ Account-Expiration ]

End of changes