3GPP TSG-SA5 (Telecom Management)	S5-111426

SA5#76, 28 Feb - 4 Mar 2011; San Diego, USA	revision of S5-111216

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.298 | CR | 0185 |  rev | 1 |  Current version: | 10.3.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | MMTel Charging enhancement for alignment with generic AS Charging description in TS 32.260 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Alcatel-Lucent |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | CH10 |  |  |  |  |  |  | Date:  |  | 04/03/2011 |
| Category:  | C |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change:  |  | Following information provided by IMS AS Charging are also applicable for MMTel Charging: - Outgoing Session ID - Service Request/Delivery Start/Delivery End Time Stamp Fraction - Online Charging Flag - Real Time Tariff Information |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Add missing fields in MMTel CDRs. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | MMTel Charging inconsistent with general IMS Charging. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 5.2.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | x | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
|  |  | X |  | O&M Specifications |  |  | 32.275 |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

First change

#### 5.2.4.5	MMTel CDRs

This subclause contains the abstract syntax definitions that are specific to the CDR types defined in TS 32.275 [35].

MMTelChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) mMTelChargingDataTypes (9) asn1Module (0) version1 (0)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

RecordType, IMSI, IMEI, IPAddress, LocalSequenceNumber, ManagementExtensions, MSISDN, NodeAddress, SubscriptionID, TimeStamp, ServiceContextID

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version1 (0)}

CarrierSelectRouting, CauseForRecordClosing, Early-Media-Components-List, IMS-Charging-Identifier, IMSCommunicationServiceIdentifier, Incomplete-CDR-Indication, InterOperatorIdentifiers, InterOperatorIdentifierlist, InvolvedParty, ListOfInvolvedParties, MessageBody,

Media-Components-List, NumberPortabilityRouting, Service-Id, Session-Id, SessionPriority, SIP-Method

FROM IMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) imsChargingDataTypes (4) asn1Module (0) version1 (0)}

;

------------------------------------------------------------------------------

--

--  MMTel RECORDS

--

------------------------------------------------------------------------------

MMTelServiceRecord	::= CHOICE

--

-- Record values 83 are MMTel specific

--

{

mMTelRecord			[83] MMTelRecord

}

MMTelRecord 	::= SET

{

recordType							[0] RecordType,

retransmission						[1] NULL OPTIONAL,

sIP-Method							[2] SIP-Method OPTIONAL,

role-of-Node						[3] Role-of-Node OPTIONAL,

nodeAddress							[4] NodeAddress OPTIONAL,

session-Id							[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address		[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address				[7] InvolvedParty OPTIONAL,

serviceRequestTimeStamp				[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp		[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp			[11] TimeStamp OPTIONAL,

recordOpeningTime					[12] TimeStamp OPTIONAL,

recordClosureTime					[13] TimeStamp OPTIONAL,

interOperatorIdentifiers			[14] InterOperatorIdentifierlist OPTIONAL,

localRecordSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber				[16] INTEGER OPTIONAL,

causeForRecordClosing				[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication			[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier				[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components		[21] SEQUENCE OF Media-Components-List OPTIONAL,

gGSNaddress							[22] NodeAddress OPTIONAL,

serviceReasonReturnCode				[23] UTF8String OPTIONAL,

list-Of-Message-Bodies				[24] SEQUENCE OF MessageBody OPTIONAL,

recordExtensions					[25] ManagementExtensions OPTIONAL,

expiresInformation					[26] INTEGER OPTIONAL,

event								[28] UTF8String OPTIONAL,

accessNetworkInformation			[29] OCTET STRING OPTIONAL,

serviceContextID					[30] ServiceContextID OPTIONAL,

list-of-subscription-ID				[31] SEQUENCE OF SubscriptionID OPTIONAL,

list-Of-Early-SDP-Media-Components 	[32] SEQUENCE OF Early-Media-Components-List OPTIONAL,

iMSCommunicationServiceIdentifier	[33] IMSCommunicationServiceIdentifier OPTIONAL,

numberPortabilityRouting			[34] NumberPortabilityRouting OPTIONAL,

carrierSelectRouting				[35] CarrierSelectRouting OPTIONAL,

sessionPriority						[36] SessionPriority OPTIONAL,

serviceRequestTimeStampFraction		[37] Milliseconds OPTIONAL,

serviceDeliveryStartTimeStampFraction [38] Milliseconds OPTIONAL,

serviceDeliveryEndTimeStampFraction	[39] Milliseconds OPTIONAL,

online-charging-flag				[43] NULL OPTIONAL,

realTimeTariffInformation			[44] SEQUENCE OF RealTimeTariffInformation OPTIONAL,

requested-Party-Address				[101] InvolvedParty OPTIONAL,

list-Of-Called-Asserted-Identity 	[102] ListOfInvolvedParties OPTIONAL,

outgoingSessionId 					[104] Session-Id OPTIONAL,

mMTelInformation					[110] MMTelInformation	OPTIONAL

}

------------------------------------------------------------------------------

--

--  MMTel DATA TYPES

--

------------------------------------------------------------------------------

MMTelInformation			::= SET

{

ListOfSupplServices	    [0] SEQUENCE OF SupplService OPTIONAL,

subscriberRole			[1] SubscriberRole OPTIONAL

}

ParticipantActionType ::= ENUMERATED

{

CREATE_CONF         (0),

JOIN_CONF           (1),

INVITED_INTO_CONF   (2),

QUIT_CONF           (3)

}

SupplService := SET

{

serviceType				    [0] ServiceType,

serviceMode				    [1] ServiceMode OPTIONAL,

numberOfDiversions		    [2] INTEGER OPTIONAL,

associated-Party-Address	[3] InvolvedParty OPTIONAL,

serviceId		            [4] Service-Id OPTIONAL,

changeTime                  [5] TimeStamp,

numberOfParticipants        [6] INTEGER OPTIONAL,

participantActionType       [7] ParticipantActionType OPTIONAL,

cUGInformation				[8]	OCTET STRING OPTIONAL

}

ServiceType ::= INTEGER

--

-- Values   1024 are reserved for specific Network/Manufacturer variants

--

{

oIPresentation		(0),

oIRestriction		(1),

tIPresentation		(2),

tIRestriction  		(3),

hOLD				(4),

cBarring			(5),

cDIVersion			(6),

cDIVersionNotif		(7),

cWaiting			(8),

mWaitingIndic		(9),

cONF				(10),

fLexibleAlerting	(11),

cCBS				(12),

cCNR				(13),

mCID				(14),

cAT					(15),

cUG					(16) ,

pNM					(17),

cRS					(18)

}

ServiceMode ::= INTEGER

--

-- Values   1024 are reserved for specific Network/Manufacturer variants

--

{	cFunCond			(0),

cFbusy				(1),

cFnoReply 			(2),

cFnotLogged			(3),

deflection			(4),

notReach			(5),

iCBarring			(6),

oCBarring			(7),

aCRejection 		(8),

eCTBlind			(9),

eCTConsultative		(10),

threePTY			(11)

}

SubscriberRole ::= ENUMERATED

{

originating			(0),

terminating			(1)

}

END

End of changes