3GPP TSG-SA5 (Telecom Management)	S5-110507

SA5#75, 24 - 28 Jan 2011; Sorrento, ITALY	revision of S5-110467

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.280 | CR | 0017 |  rev | 2 |  Current version: | 10.2.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Add missing AoC User to Network Interface (UNI) parameters: Type of Charging, Granularity and Pes-transitioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | CH9 |  |  |  |  |  |  | Date:  |  | 28/01/2011 |
| Category:  | A |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change:  |  | The parameters Type of Charging, Granularity and Pes-transitioning behavior are configuration parameters for a tariff on the AoC UNI interface. These three parameters are not available on the Ro interface but may be needed for the tariff structure as static parameters. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Add mapping information in Annex C2. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Population of UNI parameters Type of Charging, Granularity and Pes-transitioning behavior is not possible. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 3.3, C.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | x | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
|  |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [100] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [100].

AACF	Auxiliary AoC Function

ACF	AoC Function

AoC	Advice of Charge

AoC-S	AoC at communication Set-up time

AoC-D	AoC During the communication

AoC-E	AoC at the End of the communication

AoCI	AoC for Information

AoCC		AoC for Charging

CAI	Charge Advice Information

CCF	Charging Collection Function

CCR	Credit Control Request

CDF	Charging Data Function

CDP	Charging Determination Point

CGF	Charging Gateway Function

CPC	Calling Party Category

CSCF	Call Session Control Function (I-Interrogating; P-Proxy; and S-Serving)

ECUR	Event Charging with Unit Reservation

HSS	Home Subscriber Server

IBCF	Interconnection Border Control Function

IEC	Immediate Event Charging

IMS-GWF	IMS Gateway Function

ISC	IMS Service Control

MGCF	Media Gateway Control Function

OCS	Online Charging System

OFCS	Offline Charging System

RTTI 	Realtime Transfer of Tariff Information

SCUR	Session Charging with Unit Reservation

UE	User Equipment

UNI	User to Network Interface

Nexy Modified Section

## C.2	AoC information mapping to Charging Information Elements

Herby is a conceptual mapping advising how to populate the Charging Information Element provided to the UE (as described in TS 24.647 [208] and ETSI TS 183 043 [213] as explicitly mentioned in the table below) out of the AoC Information.

| Charging Information Element | Mapping guidance |
| --- | --- |
| Expressing Charging Rates |  |
| - Price per time unit | When only one Rate Element with Unit Type = TIME exists |
| - Free of charge | When only one Rate Element with Unit Type = MONEY and Unit Value = 0 exists |
| - Flat rate | When only one Rate Element with Unit Type = MONEY and Unit Value > 0 exists |
| - Not available | No Rate Elements provided |
| Charged Items |  |
| - Basic communication | When the Rate Element contains Reason Code = Usage or no Reason Code is available. NOTE: If no other Charged Items are applicable, Basic Communication shall be used as default value |
| - Communication attempt | This parameter may be populated when the Rate Element contains Reason Code = Communication Attempt Charge NOTE: Alternatively, this information may be mapped into “Operation of service” (see below). |
| - Communication setup | This parameter may be populated when the Rate Element contains Reason Code = Set Up Charge NOTE: Alternatively, this information may be mapped into “Operation of service” (see below). |
| - Operation of service | This parameter may be populated depending on Service specific Information or when the Rate Element contains Reason Code <> Usage. NOTE: If this parameter is populated depending on the Reason Code, Communication Attempt and Communication setup shall not be used. |
| - pes_aoc-s_phase_duration | The value of Unit_Quota_Threshold is mapped into pes_aoc-s_phase_duration when the AoC Extended XML schema is needed (see ETSI TS 183 043 [213]) |
| Recorded Charges | Accumulated cost in AoC Cost Information |

NOTE:	Other legacy Charging Information Elements (e.g. Special charging code, Special charging arrangement and Billing Identification) are not supported in IMS.

Additionally, the following parameters are not supported by the Ro interface but locally configurable by operators:

| Type of Charging | This parameter is part of the AoC UNI XML information described in TS 24.647 [208]. |
| --- | --- |
| Granularity | This parameter is part of the AoC UNI XML information described in TS 24.647 [208]. |
| Pes-transitioning behavior | This parameter is part of the AoC Extended XML schema described in ETSI TS 183 043 [213]. |

End of Modified Section