3GPP TSG SA5 Meeting #75	S5-110542

SA5#75, 24 - 28 Jan 2011; Sorrento, Italy	revision of S5-110243

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.299 | CR | 0335 |  rev | 1 |  Current version: | 9.6.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Add internal structure and encoding for the Location-Estimate AVP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | CH9 |  |  |  |  |  |  | Date:  |  | 28/01/2011 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-9 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change:  |  | The internal structure and encoding descriptions are missing for the Location-Estimate AVP. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | This CR proposes to add the internal structure and encoding descriptions by referring to the geographical area descriptions in 3GPP TS 23.032. In addition, this CR fixes the AVP type from UTF8String to OctectString to avoid inconsistency between TS 32.299 and TS 23.032. In addition, this CR fixes the wrong indent in the section 2. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | The location related information cannot be delivered to the G-MSC since the MME cannot set the Location Estimate AVP correctly over the SLg interface without this CR. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 2, 7.2.94 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

* * * First Change start * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging Architecture and Principles".

[2]-[99]	Void.

[100]	3GPP TR 21.905: “Vocabulary for 3GPP Specifications”

[101]	3GPP TS 22.115: "Service aspects; Charging and billing".

[102]-[199]	Void.

[200]	3GPP TS 23.207: "End to end quality of service concept and architecture".

[201]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[202]	3GPP TS 24.229: "IP Multimedia Call Control Protocol based on SIP and SDP; Stage 3."

[203]	3GPP TS 29.207: "Policy control over Go interface".

[204]	3GPP TS 29.229: "Cx and Dx Interfaces based on the Diameter protocol; Protocol Details".

[205]	Void.

[206]	3GPP TS 29.230: "3GPP specific codes and identifiers".

[207]	3GPP TS 29.061: "Interworking between the Public Land Mobile Network (PLMN) supporting packet based services and Packet Data Networks (PDN)".

[208]	3GPP TS 23.140: " Multimedia Messaging Service (MMS); Functional description; Stage 2".

[209]	OMA "Multimedia Messaging Service; Encapsulation Protocol".

[210]	OMNA WSP Content Type Numbers 
http://www.openmobilealliance.org/tech/omna/omna-wsp-content-type.aspx

[211]	OMA-CP-POC: "OMA PoC Control Plane"

[212]	3GPP 29.234: "3GPP system to Wireless Local Area Network (WLAN) interworking; Stage 3".

[213]	3GPP TS 29.140: "MM10 interface based on Diameter protocol; Stage 3".

[214]	3GPP TS 29.214: "Policy and Charging Control over Rx reference point; Stage 3".

[215]	3GPP TS 29.212: "Policy and Charging Control over Gx reference point".

[216]	3GPP TS 23.040: "Technical realization of Short Message Service (SMS)".

[217]	3GPP TS 22.142: "Value Added Services (VAS) for Short Message Service (SMS) requirements".

[218]	3GPP TS 23.203: "Policy and Charging control architecture".

[219]	3GPP TS 29.272: " Mobility Management Entity (MME) and Serving GPRS Support Node (SGSN) related interfaces based on Diameter protocol”.

[220]	3GPP TS 24.605: "Conference (CONF) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[221]	3GPP TS 29.329: "Sh Interface based on the Diameter protocol;Protocol details".

[222]	3GPP TS 29.658: "SIP Transfer of IP Multimedia Service Tariff Information".

[223] 	OMA-DDS-Charging_Data: "Charging Data".

[224] 	3GPP TS 23.003: "Numbering, Addressing and Identification".

[225] 	3GPP TS 29.060: "General Packet Radio Service (GPRS); GPRS Tunnelling Protocol (GTP) across the Gn and Gp interface".

[226] 	3GPP TS 29.274: "Evolved GPRS Tunnelling Protocol for Control Plane (GTPv2-C); Stage 3".

[227]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[228] - [400]	Void.

[401]	IETF RFC 3588: "Diameter Base Protocol".

[402]	IETF RFC 4006: "Diameter Credit Control Application"

[403]	IETF RFC 5580: "Carrying Location Objects in RADIUS"

[404]	IETF RFC 3455 , "Private Extensions to the Session Initiation Protocol (SIP) for the 3rd Generation Partnership Projects (3GPP)".

[405]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[406]	IETF RFC 4566: "SDP: Session Description Protocol".

[407]	IETF RFC 4005: "Diameter Network Access Server Application".

[408]	IETF RFC 3264: An Offer/Answer Model with the Session Description Protocol (SDP).

NOTE:	The above reference will need to be updated to reference the assigned RFC number, once the draft achieves RFC status within the IETF.

* * * Next Change start * * *

## 7.2	3GPP specific AVPs

For the purpose of offline charging additional AVPs are used in ACR / ACA and for online charging additional AVPs are used in CCR / CCA. All 3GPP specific AVPs mentioned are relevant for both offline and online charging unless specifically excluded. The information is summarized in the following table along with the AVP flag rules.

The 3GPP Charging Application uses the value 10415 (3GPP) as Vendor-Id.

Detailed descriptions of AVPs that are used specifically for 3GPP charging are provided in the subclauses below the table. However, for AVPs that are just borrowed from other applications only the reference (e.g. TS 29.229 [204]), is provided in the following table and the detailed description is not repeated.

Where 3GPP RADIUS VSAs are re-used, they shall be translated to Diameter AVPs as described in RFC 4005 [407] with the exception that the 'M' flag shall be set and the ''P' flag may be set.

Table 7.2: 3GPP specific AVPs

| AVP Name | AVP Code | Used in |  |  |  | Value Type | AVP Flag rules |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ACR | ACA | CCR | CCA |  | Must | May | Should not | Must not | May Encr. |
| 3GPP-Charging-Characteristics | 13 | X | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-Charging-Id | 2 | X | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-GGSN-MCC-MNC | 9 | X | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-IMSI | 1 | - | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-IMSI-MCC-MNC | 8 | X | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-MS-TimeZone | 23 | X | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-NSAPI | 10 | X | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-PDP-Type | 3 | X | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-RAT-Type | 21 | X | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-Selection-Mode | 12 | X | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-Session-Stop-Indicator | 11 | - | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-SGSN-MCC-MNC | 18 | X | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-User-Location-Info | 22 | X | - | X | - | refer [207] |  |  |  |  |  |
| Access-Network-Charging-Identifier-Value | 503 | X | - | X | - | refer [214] |  |  |  |  |  |
| Access-Network-Information | 1263 | X | - | X | - | OctetString | V,M | P |  |  | N |
| Account-Expiration | 2309 | - | - | - | X | Time | V,M | P |  |  | N |
| Accumulated-Cost | 2052 | - | - | - | X | Grouped | V,M | P |  |  | N |
| Adaptations | 1217 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Additional-Content-Information | 1207 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Additional-Type-Information | 1205 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Address-Data | 897 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Address-Domain | 898 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Addressee-Type | 1208 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Address-Type | 899 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| AF-Charging-Identifier | 505 | - | - | X | - | refer [214] |  |  |  |  |  |
| AF-Correlation-Information | 1276 | X | - | X | - | Grouped | V,M | P |  |  | N |
| Allocation-Retention-Priority | 1034 | X | - | X | - | refer [215] |  |  |  |  |  |
| Alternate-Charged-Party-Address | 1280 | X | - | - | - | UTF8string | V.M | P |  |  | N |
| AoC-Cost-Information | 2053 | - | - | X | X | Grouped | V,M | P |  |  | N |
| AoC-Format | 2310 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| AoC-Information | 2054 | - | - | - | X | Grouped | V.M | P |  |  | N |
| AoC-Request-Type | 2055 | - | - | X | - | Enumerated | V.M | P |  |  | N |
| AoC-Service | 2311 | - | - | X | - | Grouped | V,M | P |  |  | N |
| AoC-Service-Obligatory-Type | 2312 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| AoC-Service-Type | 2313 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| AoC-Subscription-Information | 2314 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Application-provided-called-party-address | 837 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Application-Server | 836 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Application-Server-ID | 2101 | X | - | X | - | refer[223] |  |  |  |  |  |
| Application-Server-Information | 850 | X | - | X | - | Grouped | V,M | P |  |  | N |
| Application-Service-Type | 2102 | X | - | X | - | refer[223] |  |  |  |  |  |
| Application-Session-ID | 2103 | X | - | X | - | refer[223] |  |  |  |  |  |
| Applic-ID | 1218 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Associated-Party-Address | 2035 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Associated-URI | 856 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Authorized-QoS | 849 | X | - | - | - | UTF8String | V,M | P |  |  | N |
| Aux-Applic-Info | 1219 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Base-Time-Interval | 1265 | - | - | - | X | Unsigned32 | V,M | P |  |  | N |
| Bearer-Service | 854 | X | - | - | - | OctetString | V,M | P |  |  | N |
| Called-Asserted-Identity | 1250 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Called-Party-Address | 832 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Calling-Party-Address | 831 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Carrier-Select-Routing-Information | 2023 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Cause-Code | 861 | X | - | X | - | Integer32 | V,M | P |  |  | N |
| CG-Address | 846 | X | - | X | - | Address | V,M | P |  |  | Y |
| Change-Condition | 2037 | X | - | - | - | Integer32 | V,M | P |  |  | N |
| Change-Time | 2038 | X | - | - | - | Time | V,M | P |  |  | N |
| Charged-Party | 857 | X | - | - | - | UTF8String | V,M | P |  |  | N |
| Charging-Characteristics-Selection-Mode | 2066 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| Charging-Rule-Base-Name | 1004 | X | - | X | - | refer [215] |  |  |  |  |  |
| Class-Identifier | 1214 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Client-Address | 2018 | - | - | X | - | Address | V,M | P |  |  | N |
| CN-IP-Multicast-Distribution | 921 | X | - | - | - | refer [207] |  |  |  |  |  |
| Content-Class | 1220 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Content-Disposition | 828 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Content-ID | 2116 | X | - | X | - | refer[223] |  |  |  |  |  |
| Content-Provider-ID | 2117 | X | - | X | - | refer[223] |  |  |  |  |  |
| Content-Length | 827 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| Content-Size | 1206 | - | - | X | - | Unsigned32 | V,M | P |  |  | N |
| CSG-Access-Mode | 2317 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| CSG-Id | 1437 | X | - | X | - | refer[219] |  |  |  |  |  |
| CSG-Membership-Indication | 2318 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| Content-Type | 826 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Current-Tariff | 2056 | - | - | X | X | Grouped | V,M | P |  |  | N |
| CUG-Information | 2304 | X | - | X | - | OctetString | V,M | P |  |  | N |
| Data-Coding-Scheme | 2001 | - | - | X | - | Integer32 | V,M | P |  |  | N |
| DCD-Information | 2115 | X | - | X | - | refer[223] |  |  |  |  |  |
| Deferred-Location-Event-Type | 1230 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Delivery-Report-Requested | 1216 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Delivery-Status | 2104 | X | - | X | - | refer[223] |  |  |  |  |  |
| Destination-Interface | 2002 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Diagnostics | 2039 | X | - | - | - | Integer32 | V,M | P |  |  | N |
| Domain-Name | 1200 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| DRM-Content | 1221 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Dynamic-Address-Flag | 2051 | X | - | - | - | Enumerated | V,M | P |  |  | N |
| Early-Media-Description | 1272 | X | - | - | - | Grouped | V,M | P |  |  | N |
| Envelope | 1266 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Envelope-End-Time | 1267 | - | - | X | - | Time | V,M | P |  |  | N |
| Envelope-Reporting | 1268 | - | - | - | X | Enumerated | V,M | P |  |  | N |
| Envelope-Start-Time | 1269 | - | - | X | - | Time | V,M | P |  |  | N |
| Event | 825 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Event-Charging-TimeStamp | 1258 | - | - | X | - | Time | V,M | P |  |  | N |
| Event-Type | 823 | X | - | X | - | Grouped | V,M | P |  |  | N |
| Expires | 888 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| File-Repair-Supported | 1224 | X | - | X | - | Enumerated | V,M | P |  |  | Y |
| Flows | 510 | - | - | X | - | refer [214] |  |  |  |  |  |
| GGSN-Address | 847 | X | - | X | - | Address | V,M | P |  |  | N |
| Guaranteed-Bitrate-UL | 1026 | X | - | X | - | refer[215] |  |  |  |  |  |
| IM-Information | 2110 | X | - | X | - | refer[223] |  |  |  |  |  |
| IMS-Charging-Identifier | 841 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| IMS-Communication-Service-Identifier | 1281 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| IMS-Information | 876 | X | - | X | - | Grouped | V,M | P |  |  | N |
| IMSI-Unauthenticated-Flag | 2308 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| Incoming-Trunk-Group-Id | 852 | X | - | - | - | UTF8String | V,M | P |  |  | N |
| Incremental-Cost | 2062 | - | - | X | X | Grouped | V,M | P |  |  | N |
| Interface-Id | 2003 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Interface-Port | 2004 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Interface-Text | 2005 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Interface-Type | 2006 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Inter-Operator-Identifier | 838 | X | - | X | - | Grouped | V,M | P |  |  | N |
| LCS-Client-Dialed-By-MS | 1233 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| LCS-Client-External-ID | 1234 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| LCS-Client-Id | 1232 | - | - | X | - | Grouped | V,M | P |  |  | N |
| LCS-Client-Name | 1231 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| LCS-Client-Name | 1235 | - | - | X | - | Grouped | V,M | P |  |  | N |
| LCS-Client-Type | 1241 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| LCS-Data-Coding-Scheme | 1236 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| LCS-Format-Indicator | 1237 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| LCS-Information | 878 | - | - | X | - | Grouped | V,M | P |  |  | N |
| LCS-Name-String | 1238 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| LCS-Requestor-Id | 1239 | - | - | X | - | Grouped | V,M | P |  |  | N |
| LCS-Requestor-Id-String | 1240 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Local-Sequence-Number | 2063 | X | - | - | - | Unsigned32 | V,M | P |  |  | N |
| Location-Estimate | 1242 | - | - | X | - | OctetString | V,M | P |  |  | N |
| Location-Estimate-Type | 1243 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Location-Type | 1244 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Low-Balance-Indication | 2020 | - | - | - | X | Enumerated | V,M | P |  |  | N |
| Mandatory-Capability | 604 | X | - | - | - | refer [204] |  |  |  |  |  |
| Max-Requested-Bandwidth-DL | 515 | X | - | X | - | refer [214] |  |  |  |  |  |
| Max-Requested-Bandwidth-UL | 516 | X | - | X | - | refer [214] |  |  |  |  |  |
| MBMS-2G-3G-Indicator | 907 | X | - | X | - | refer [207] |  |  |  |  |  |
| MBMS GW-Address | 2307 | X | - | - | - | Address | V,M | P |  |  | N |
| MBMS-Information | 880 | X | - | X | - | Grouped | V,M | P |  |  | N |
| MBMS-Service-Area | 903 | X | - | X | - | refer [207] |  |  |  |  |  |
| MBMS-Service-Type | 906 | X | - | X | - | refer [207] |  |  |  |  |  |
| MBMS-Session-Identity | 908 | X | - | X | - | refer [207] |  |  |  |  |  |
| MBMS-User-Service-Type | 1225 | X | - | X | - | Enumerated | V,M | P |  |  | Y |
| Media-Initiator-Flag | 882 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| Media-Initiator-Party | 1288 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Message-Body | 889 | X | - | X | - | Grouped | V,M | P |  |  | N |
| Message-Class | 1213 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Message-ID | 1210 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Message-Size | 1212 | - | - | X | - | Unsigned32 | V,M | P |  |  | N |
| Message-Type | 1211 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| MMBox-Storage-Requested | 1248 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| MM-Content-Type | 1203 | - | - | X | - | Grouped | V,M | P |  |  | N |
| MMS-Information | 877 | - | - | X | - | Grouped | V,M | P |  |  | N |
| MMTel-Information | 2030 | X | - | X | - | Grouped | V,M | P |  |  | N |
| MSISDN | 701 | - | - | X | - | refer [221] |  |  |  |  |  |
| Next-Tariff | 2057 |  |  | X | X | Grouped | V/M | P |  |  | N |
| Node-Functionality | 862 | X | -. | X | - | Enumerated | V,M | P |  |  | N |
| Node-Id | 2064 | X | - | - | X | UTF8String | V,M | P |  |  | N |
| Number-Of-Diversions | 2034 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| Number-Of-Messages-Sent | 2019 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| Number-Of-Messages-Successfully-Exploded | 2111 | X | - | X | - | refer[223] |  |  |  |  |  |
| Number-Of-Messages-Successfully-Sent | 2112 | X | - | X | - | refer[223] |  |  |  |  |  |
| Number-Of-Participants | 885 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| Number-Of-Received-Talk-Bursts | 1282 | X | - | - | - | Unsigned32 | V,M | P |  |  | N |
| Number-Of-Talk-Bursts | 1283 | X | - | - | - | Unsigned32 | V,M | P |  |  | N |
| Number-Portability-Routing-Information | 2024 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Offline-Charging | 1278 | - | - | - | X | Grouped | V,M | P |  |  | N |
| Online-Charging-Flag | 2303 | X | - | - | - | Enumerated | V,M | P |  |  | N |
| Optional-Capability | 605 | X | - | - | - | refer [204] |  |  |  |  |  |
| Originating-IOI | 839 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Originator-SCCP-Address | 2008 | - | - | X | - | Address | V,M | P |  |  | N |
| Originator | 864 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| Originator-Address | 886 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Originator-Received-Address | 2027 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Originator-Interface | 2009 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Outgoing-Session-Id | 2320 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Outgoing-Trunk-Group-Id | 853 | X | - | - | - | UTF8String | V,M | P |  |  | N |
| Participant-Access-Priority | 1259 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| Participant-Action-Type | 2049 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| Participant-Group | 1260 | X | - | X | - | Grouped | V,M | P |  |  | N |
| Participants-Involved | 887 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| PDG-Address | 895 | X | - | X | - | Address | V,M | P |  |  | N |
| PDG-Charging-Id | 896 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| PDN-Connection-ID | 2050 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| PDP-Address | 1227 | X | - | X | - | Address | V,M | P |  |  | Y |
| PDP-Context-Type | 1247 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| PoC-Change-Condition | 1261 | X | - | - | - | Enumerated | V,M | P |  |  | N |
| PoC-Change-Time | 1262 | X | - | - | - | Time | V,M | P |  |  | N |
| PoC-Controlling-Address | 858 | X | - | X |  | UTF8String | V,M | P |  |  | N |
| PoC-Event-Type | 2025 | X | - | X |  | Enumerated | V,M | P |  |  | N |
| PoC-Group-Name | 859 | X | - | X |  | UTF8String | V,M | P |  |  | N |
| PoC-Information | 879 | X | - | X | - | Grouped | V,M | P |  |  | N |
| PoC-Server-Role | 883 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| PoC-Session-Id | 1229 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| PoC-Session-Initiation-type | 1277 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| PoC-Session-Type | 884 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| PoC-User-Role | 1252 | X | - | X | - | Grouped | V,M | P |  |  | N |
| PoC-User-Role-IDs | 1253 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| PoC-User-Role-info-Units | 1254 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| Positioning-Data | 1245 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Preferred-AoC-Currency | 2315 | - | - | X | - | Unsigned32 | V,M | P |  |  | N |
| Priority | 1209 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Priority-Level | 1046 | X | - | X | - | refer [215] |  |  |  |  |  |
| PS-Append-Free-Format-Data | 867 | X | - | - | X | Enumerated | V,M | P |  |  | N |
| PS-Free-Format-Data | 866 | X | - | - | X | OctetString | V,M | P |  |  | N |
| PS-Furnish-Charging-Information | 865 | X | - | - | X | Grouped | V,M | P |  |  | N |
| PS-Information | 874 | X | - | X | X | Grouped | V,M | P |  |  | N |
| QoS-Information | 1016 | X | - | - | - | refer [215] |  |  |  |  |  |
| QoS-Class-Identifier | 1028 | X | - | X | - | refer [215] |  |  |  |  |  |
| Quota-Consumption-Time | 881 | - | - | - | X | Unsigned32 | V,M | P |  |  | N |
| Quota-Holding-Time | 871 | - | - | - | X | Unsigned32 | V,M | P |  |  | N |
| RAI | 909 | X | - | X | - | refer [207] |  |  |  |  |  |
| Rate-Element | 2058 | - | - | X | X | Grouped | V,M | P |  |  | N |
| Read-Reply-Report-Requested | 1222 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Reason-Code | 2316 | - | - | X | X | Enumerated | V,M | P |  |  | N |
| Real-Time-Tariff-Information | 2305 | X | - | - | - | Grouped | V,M | P |  |  | N |
| Received-Talk-Burst-Time | 1284 | X | - | - | - | Unsigned32 | V,M | P |  |  | N |
| Received-Talk-Burst-Volume | 1285 | X | - | - | - | Unsigned32 | V,M | P |  |  | N |
| Recipient-Address | 1201 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Recipient-Info | 2026 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Recipient-Received-Address | 2028 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Recipient-SCCP-Address | 2010 | - | - | X | - | Address | V,M | P |  |  | N |
| Refund-Information | 2022 | - | - | X | X | OctetString | V,M | P |  |  | N |
| Remaining-Balance | 2021 | - | - | - | X | Grouped | V,M | P |  |  | N |
| Reply-Applic-ID | 1223 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Reply-Path-Requested | 2011 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Reporting-Reason | 872 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Requested-Party-Address | 1251 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Required-MBMS-Bearer-Capabilities | 901 | X | - | X | - | refer [207] |  |  |  |  |  |
| Role-Of-Node | 829 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| Scale-Factor | 2059 | - | - | X | X | Grouped | V,M | P |  |  | N |
| SDP-Answer-Timestamp | 1275 | X | - | - | - | Time | V,M | P |  |  | N |
| SDP-Media-Component | 843 | X | - | X | - | Grouped | V,M | P |  |  | N |
| SDP-Media-Description | 845 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| SDP-Media-Name | 844 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| SDP-Offer-Timestamp | 1274 | X | - | - | - | Time | V,M | P |  |  | N |
| SDP-Session-Description | 842 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| SDP-TimeStamps | 1273 | X | - | - | - | Grouped | V,M | P |  |  | N |
| SDP-Type | 2036 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| Served-Party-IP-Address | 848 | X | - | - | - | Address | V,M | P |  |  | N |
| Server-Capabilities | 603 | X | - | - | - | refer [204] |  |  |  |  |  |
| Server-Name | 602 | X | - | - | - | refer [204] |  |  |  |  |  |
| Service-Data-Container | 2040 | X | - | - | - | Grouped | V,M | P |  |  | N |
| Service-Generic-Information | 1256 | X | - | X | - | Refer[223] |  |  |  |  |  |
| Service-Id | 855 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Service-Information | 873 | X | - | X | X | Grouped | V,M | P |  |  | N |
| Service-Mode | 2032 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| Service-Specific-Data | 863 | X | - | - | - | UTF8String | V,M | P |  |  | N |
| Service-Specific-Info | 1249 | X | - | - | - | Grouped | V,M | P |  |  | N |
| Service-Specific-Type | 1257 | X | - | - | - | Unsigned32 | V,M | P |  |  | N |
| Serving-Node-Type | 2047 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| Service-Type | 2031 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| Session-Priority | 650 | X | - | X | - | Refer [204] |  |  |  |  |  |
| SGSN-Address | 1228 | X | - | X | - | Address | V,M | P |  |  | N |
| SGW-Change | 2065 | X | - | - | - | Enumerated | V,M | P |  |  | N |
| SIP-Method | 824 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| SIP-Request-Timestamp-Fraction | 2301 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| SIP-Request-Timestamp | 834 | X | - | X | - | Time | V,M | P |  |  | N |
| SIP-Response-Timestamp-Fraction | 2302 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| SIP-Response-Timestamp | 835 | X | - | X | - | Time | V,M | P |  |  | N |
| SM-Discharge-Time | 2012 | - | - | X | - | Time | V,M | P |  |  | N |
| SM-Message-Type | 2007 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| SM-Protocol-ID | 2013 | - | - | X | - | OctetString | V,M | P |  |  | N |
| SMSC-Address | 2017 | - | - | X | - | Address | V,M | P |  |  | N |
| SMS-Information | 2000 | - | - | X | - | Grouped | V,M | P |  |  | N |
| SMS-Node | 2016 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| SM-Service-Type | 2029 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| SM-Status | 2014 | - | - | X | - | OctetString | V,M | P |  |  | N |
| SM-User-Data-Header | 2015 | - | - | X | - | OctetString | V,M | P |  |  | N |
| Start-Time | 2041 | X | - | - | - | Time | V,M | P |  |  | N |
| Stop-Time | 2042 | X | - | - | - | Time | V,M | P |  |  | N |
| Submission-Time | 1202 | - | - | X | - | Time | V,M | P |  |  | N |
| Subscriber-Role | 2033 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| Supplementary-Service | 2048 | X | - | X | - | Grouped | V,M | P |  |  | N |
| Talk-Burst-Exchange | 1255 | X | - | - | - | Grouped | V,M | P |  |  | N |
| Talk-Burst-Time | 1286 | X | - | - | - | Unsigned32 | V,M | P |  |  | N |
| Talk-Burst-Volume | 1287 | X | - | - | - | Unsigned32 | V,M | P |  |  | N |
| Tariff-Information | 2060 | X | - | X | X | Grouped | V,M | P |  |  | N |
| Tariff-XML | 2306 | X | - | - | - | UTF8String | V,M | P |  |  | N |
| Terminal-Information | 1401 | X | - | X | - | refer [219] |  |  |  |  |  |
| Terminating-IOI | 840 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Time-First-Usage | 2043 | X | - | - | - | Time | V,M | P |  |  | N |
| Time-Last-Usage | 2044 | X | - | - | - | Time | V,M | P |  |  | N |
| Time-Quota-Mechanism | 1270 | - | - | - | X | Grouped | V,M | P |  |  | N |
| Time-Quota-Threshold | 868 | - | - | - | X | Unsigned32 | V,M | P |  |  | N |
| Time-Quota-Type | 1271 | - | - | - | X | Enumerated | V,M | P |  |  | N |
| Time-Stamps | 833 | X | - | X | - | Grouped | V,M | P |  |  | N |
| Time-Usage | 2045 | X | - | - | - | Unsigned32 | V,M | P |  |  | N |
| TMGI | 900 | X | - | X | - | refer [207] |  |  |  |  |  |
| Token-Text | 1215 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Total-Number-Of-Messages-Exploded | 2113 | X | - | X | - | refer[223] |  |  |  |  |  |
| Total-Number-Of-Messages-Sent | 2114 | X | - | X | - | refer[223] |  |  |  |  |  |
| Traffic-Data-Volumes | 2046 | X | - | - | - | Grouped | V,M | P |  |  | N |
| Trigger | 1264 | - | - | X | X | Grouped | V,M | P |  |  | N |
| Trigger-Type | 870 | - | - | X | X | Enumerated | V,M | P |  |  | N |
| Trunk-Group-Id | 851 | X | - | - | - | Grouped | V,M | P |  |  | N |
| Type-Number | 1204 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Unit-Cost | 2061 | - | - | X | X | Grouped | V,M | P |  |  | N |
| Unit-Quota-Threshold | 1226 | - | - | - | X | Unsigned32 | V,M | P |  |  | N |
| User-CSG-Information | 2319 | X | - | X | X | Grouped | V,M | P |  |  | N |
| User-Data | 606 | X | - | - | - | refer [204] | V,M | P |  |  | N |
| User-Participating-Type | 1279 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| User–Session-Id | 830 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| VAS-Id | 1102 | - | - | X | - | refer [213] |  |  |  |  |  |
| VASP-Id | 1101 | - | - | X | - | refer [213] |  |  |  |  |  |
| Volume-Quota-Threshold | 869 | - | - | - | X | Unsigned32 | V,M | P |  |  | N |
| WAG-Address | 890 | X | - | X | - | Address | V,M | P |  |  | N |
| WAG-PLMN-Id | 891 | X | - | X | - | OctetString | V,M | P |  |  | N |
| WLAN-Information | 875 | X | - | X | - | Grouped | V,M | P |  |  | N |
| WLAN-Radio-Container | 892 | X | - | X | - | Grouped | V,M | P |  |  | N |
| WLAN-Session-Id | 1246 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| WLAN-Technology | 893 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| WLAN-UE-Local-IPAddress | 894 | X | - | X | - | Address | V,M | P |  |  | N |

* * * Next Change start * * *

### 7.2.94	Location-Estimate AVP

The Location-Estimate AVP (AVP code 1242) is of type OctetString and contains an estimate of the location of an MS in universal coordinates and the accuracy of the estimate. Refers to geographical area descriptions in 3GPP TS 23.032　[227] for the internal structure and encoding of this AVP.

* * * End of Changes * * *