3GPP TSG-SA5 (Telecom Management)	S5-111405

SA5#76, 28 Feb - 4 Mar 2011; San Diego, USA	revision of S5-111275

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.298 | CR | 0187 |  rev | 1 |  Current version: | 10.3.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Correction on availability of Called Asserted Identity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Nokia Siemens Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | CH9 |  |  |  |  |  |  | Date:  |  | 04/03/2011 |
| Category:  | A |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change:  |  | Called Asserted Identity is available through P-Asserted-Identity in 2xx responses at the P-CSCF according to TS 24.229, but not outputted on the Rf interface. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Called Asserted Identity is added on Rf from P-CSCF and into P-CSCF CDRs |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Charging error due to the lack of this essential parameter. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 5.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  | X |  | O&M Specifications |  |  | 32.260 |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

First change

#### 5.2.3.1	IMS CDRs

This subclause contains the abstract syntax definitions that are specific to the CDR types defined in TS 32.260 [20].

IMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) imsChargingDataTypes (4) asn1Module (0) version1 (0)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

-- EXPORTS everything

IMPORTS

RecordType, IPAddress, ManagementExtensions, NodeAddress, LocalSequenceNumber, SubscriptionID, TimeStamp, ServiceContextID, ServiceSpecificInfo

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version1 (0)}

;

------------------------------------------------------------------------------

--

--  IMS RECORDS

--

------------------------------------------------------------------------------

IMSRecord ::= CHOICE

--

-- Record values 63-69, 82 are IMS specific

--

{

sCSCFRecord			[63] SCSCFRecord,

pCSCFRecord			[64] PCSCFRecord,

iCSCFRecord			[65] ICSCFRecord,

mRFCRecord			[66] MRFCRecord,

mGCFRecord			[67] MGCFRecord,

bGCFRecord			[68] BGCFRecord,

aSRecord			[69] ASRecord,

eCSCFRecord			[70] ECSCFRecord,

iBCFRecord			[82] IBCFRecord

}

SCSCFRecord 	::= SET

{

recordType							[0] RecordType,

retransmission						[1] NULL OPTIONAL,

sIP-Method							[2] SIP-Method OPTIONAL,

role-of-Node						[3] Role-of-Node OPTIONAL,

nodeAddress							[4] NodeAddress OPTIONAL,

session-Id							[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address		[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address				[7] InvolvedParty OPTIONAL,

privateUserID						[8] GraphicString OPTIONAL,

serviceRequestTimeStamp				[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp		[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp			[11] TimeStamp OPTIONAL,

recordOpeningTime					[12] TimeStamp OPTIONAL,

recordClosureTime					[13] TimeStamp OPTIONAL,

interOperatorIdentifiers			[14] InterOperatorIdentifierlist OPTIONAL,

localRecordSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber				[16] INTEGER OPTIONAL,

causeForRecordClosing				[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication			[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier				[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components		[21] SEQUENCE OF Media-Components-List OPTIONAL,

gGSNaddress							[22] NodeAddress OPTIONAL,

serviceReasonReturnCode				[23] UTF8String OPTIONAL,

list-Of-Message-Bodies				[24] SEQUENCE OF MessageBody OPTIONAL,

recordExtensions					[25] ManagementExtensions OPTIONAL,

expiresInformation					[26] INTEGER OPTIONAL,

list-Of-Associated-URI				[27] ListOfInvolvedParties OPTIONAL,

event								[28] UTF8String OPTIONAL,

accessNetworkInformation			[29] OCTET STRING OPTIONAL,

serviceContextID					[30] ServiceContextID OPTIONAL,

list-of-subscription-ID				[31] SEQUENCE OF SubscriptionID OPTIONAL,

list-Of-Early-SDP-Media-Components 	[32] SEQUENCE OF Early-Media-Components-List OPTIONAL,

iMSCommunicationServiceIdentifier	[33] IMSCommunicationServiceIdentifier OPTIONAL,

numberPortabilityRouting			[34] NumberPortabilityRouting OPTIONAL,

carrierSelectRouting				[35] CarrierSelectRouting OPTIONAL,

sessionPriority						[36] SessionPriority OPTIONAL,

serviceRequestTimeStampFraction		[37] Milliseconds OPTIONAL,

serviceDeliveryStartTimeStampFraction [38] Milliseconds OPTIONAL,

serviceDeliveryEndTimeStampFraction	[39] Milliseconds OPTIONAL,

applicationServersInformation		[40] SEQUENCE OF ApplicationServersInformation OPTIONAL,

requested-Party-Address				[41] InvolvedParty OPTIONAL,

list-Of-Called-Asserted-Identity 	[42] ListOfInvolvedParties OPTIONAL,

online-charging-flag                [43] NULL OPTIONAL,

realTimeTariffInformation			[44] SEQUENCE OF RealTimeTariffInformation OPTIONAL

}

PCSCFRecord 	::= SET

{

recordType							[0] RecordType,

retransmission						[1] NULL OPTIONAL,

sIP-Method							[2] SIP-Method OPTIONAL,

role-of-Node						[3] Role-of-Node OPTIONAL,

nodeAddress							[4] NodeAddress OPTIONAL,

session-Id							[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address		[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address				[7] InvolvedParty OPTIONAL,

serviceRequestTimeStamp				[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp		[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp			[11] TimeStamp OPTIONAL,

recordOpeningTime					[12] TimeStamp OPTIONAL,

recordClosureTime					[13] TimeStamp OPTIONAL,

interOperatorIdentifiers			[14] InterOperatorIdentifiers OPTIONAL,

localRecordSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber				[16] INTEGER OPTIONAL,

causeForRecordClosing				[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication			[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier				[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components		[21] SEQUENCE OF Media-Components-List OPTIONAL,

gGSNaddress							[22] NodeAddress OPTIONAL,

serviceReasonReturnCode				[23] UTF8String OPTIONAL,

list-Of-Message-Bodies				[24] SEQUENCE OF MessageBody OPTIONAL,

recordExtensions					[25] ManagementExtensions OPTIONAL,

expiresInformation					[26] INTEGER OPTIONAL,

list-Of-Associated-URI				[27] ListOfInvolvedParties OPTIONAL,

event								[28] UTF8String OPTIONAL,

accessNetworkInformation			[29] OCTET STRING OPTIONAL,

serviceContextID					[30] ServiceContextID OPTIONAL,

list-of-subscription-ID				[31] SEQUENCE OF SubscriptionID OPTIONAL,

list-Of-Early-SDP-Media-Components 	[32] SEQUENCE OF Early-Media-Components-List OPTIONAL,

sessionPriority						[36] SessionPriority OPTIONAL,

serviceRequestTimeStampFraction		[37] Milliseconds OPTIONAL,

serviceDeliveryStartTimeStampFraction [38] Milliseconds OPTIONAL,

serviceDeliveryEndTimeStampFraction	[39] Milliseconds OPTIONAL,

list-Of-Called-Asserted-Identity 	[42] ListOfInvolvedParties OPTIONAL,

servedPartyIPAddress				[50] ServedPartyIPAddress OPTIONAL

}

ICSCFRecord 	::= SET

{

recordType						[0] RecordType,

retransmission					[1] NULL OPTIONAL,

sIP-Method						[2] SIP-Method OPTIONAL,

role-of-Node					[3] Role-of-Node OPTIONAL,

nodeAddress						[4] NodeAddress OPTIONAL,

session-Id						[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address	[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address			[7] InvolvedParty OPTIONAL,

serviceRequestTimeStamp			[9] TimeStamp OPTIONAL,

interOperatorIdentifiers		[14] InterOperatorIdentifiers OPTIONAL,

localRecordSequenceNumber		[15] LocalSequenceNumber OPTIONAL,

causeForRecordClosing			[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication		[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier			[19] IMS-Charging-Identifier OPTIONAL,

serviceReasonReturnCode			[23] UTF8String OPTIONAL,

recordExtensions				[25] ManagementExtensions OPTIONAL,

expiresInformation				[26] INTEGER OPTIONAL,

list-Of-Associated-URI			[27] ListOfInvolvedParties OPTIONAL,

event							[28] UTF8String OPTIONAL,

accessNetworkInformation		[29] OCTET STRING OPTIONAL,

serviceContextID				[30] ServiceContextID OPTIONAL,

numberPortabilityRouting		[34] NumberPortabilityRouting OPTIONAL,

carrierSelectRouting			[35] CarrierSelectRouting OPTIONAL,

sessionPriority					[36] SessionPriority OPTIONAL,

serviceRequestTimeStampFraction	[37] Milliseconds OPTIONAL,

s-CSCF-Information				[61] S-CSCF-Information OPTIONAL

}

MRFCRecord 	::= SET

{

recordType							[0] RecordType,

retransmission						[1] NULL OPTIONAL,

sIP-Method							[2] SIP-Method OPTIONAL,

nodeAddress							[4] NodeAddress OPTIONAL,

session-Id							[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address		[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address				[7] InvolvedParty OPTIONAL,

serviceRequestTimeStamp				[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp		[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp			[11] TimeStamp OPTIONAL,

recordOpeningTime					[12] TimeStamp OPTIONAL,

recordClosureTime					[13] TimeStamp OPTIONAL,

interOperatorIdentifiers			[14] InterOperatorIdentifiers OPTIONAL,

localRecordSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber				[16] INTEGER OPTIONAL,

causeForRecordClosing				[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication			[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier				[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components		[21] SEQUENCE OF Media-Components-List OPTIONAL,

gGSNaddress							[22] NodeAddress OPTIONAL,

serviceReasonReturnCode				[23] UTF8String OPTIONAL,

recordExtensions					[25] ManagementExtensions OPTIONAL,

expiresInformation					[26] INTEGER OPTIONAL,

event								[28] UTF8String OPTIONAL,

accessNetworkInformation			[29] OCTET STRING OPTIONAL,

serviceContextID					[30] ServiceContextID OPTIONAL,

list-of-subscription-ID				[31] SEQUENCE OF SubscriptionID OPTIONAL,

list-Of-Early-SDP-Media-Components 	[32] SEQUENCE OF Early-Media-Components-List OPTIONAL,

sessionPriority						[36] SessionPriority OPTIONAL,

serviceRequestTimeStampFraction		[37] Milliseconds OPTIONAL,

serviceDeliveryStartTimeStampFraction [38] Milliseconds OPTIONAL,

serviceDeliveryEndTimeStampFraction	[39] Milliseconds OPTIONAL,

applicationServersInformation		[40] SEQUENCE OF ApplicationServersInformation OPTIONAL,

online-charging-flag				[43] NULL OPTIONAL,

service-Id							[70] Service-Id OPTIONAL,

requested-Party-Address				[71] InvolvedParty OPTIONAL,

list-Of-Called-Asserted-Identity 	[72] ListOfInvolvedParties OPTIONAL

}

MGCFRecord 	::= SET

{

recordType							[0] RecordType,

retransmission						[1] NULL OPTIONAL,

sIP-Method							[2] SIP-Method OPTIONAL,

role-of-Node						[3] Role-of-Node OPTIONAL,

nodeAddress							[4] NodeAddress OPTIONAL,

session-Id							[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address		[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address				[7] InvolvedParty OPTIONAL,

serviceRequestTimeStamp				[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp		[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp			[11] TimeStamp OPTIONAL,

recordOpeningTime					[12] TimeStamp OPTIONAL,

recordClosureTime					[13] TimeStamp OPTIONAL,

interOperatorIdentifiers			[14] InterOperatorIdentifiers OPTIONAL,

localRecordSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber				[16] INTEGER OPTIONAL,

causeForRecordClosing				[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication			[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier				[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components		[21] SEQUENCE OF Media-Components-List OPTIONAL,

serviceReasonReturnCode				[23] UTF8String OPTIONAL,

recordExtensions					[25] ManagementExtensions OPTIONAL,

expiresInformation					[26] INTEGER OPTIONAL,

event								[28] UTF8String OPTIONAL,

accessNetworkInformation			[29] OCTET STRING OPTIONAL,

serviceContextID					[30] ServiceContextID OPTIONAL,

list-Of-Early-SDP-Media-Components 	[32] SEQUENCE OF Early-Media-Components-List OPTIONAL,

numberPortabilityRouting			[34] NumberPortabilityRouting OPTIONAL,

carrierSelectRouting				[35] CarrierSelectRouting OPTIONAL,

sessionPriority						[36] SessionPriority OPTIONAL,

serviceRequestTimeStampFraction		[37] Milliseconds OPTIONAL,

serviceDeliveryStartTimeStampFraction [38] Milliseconds OPTIONAL,

serviceDeliveryEndTimeStampFraction	[39] Milliseconds OPTIONAL,

realTimeTariffInformation			[44] SEQUENCE OF RealTimeTariffInformation OPTIONAL,

trunkGroupID						[80] TrunkGroupID OPTIONAL,

bearerService						[81] TransmissionMedium OPTIONAL

}

BGCFRecord 	::= SET

{

recordType							[0] RecordType,

retransmission						[1] NULL OPTIONAL,

sIP-Method							[2] SIP-Method OPTIONAL,

role-of-Node						[3] Role-of-Node OPTIONAL,

nodeAddress							[4] NodeAddress OPTIONAL,

session-Id							[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address		[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address				[7] InvolvedParty OPTIONAL,

serviceRequestTimeStamp				[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp		[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp			[11] TimeStamp OPTIONAL,

recordOpeningTime					[12] TimeStamp OPTIONAL,

recordClosureTime					[13] TimeStamp OPTIONAL,

interOperatorIdentifiers			[14] InterOperatorIdentifiers OPTIONAL,

localRecordSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber				[16] INTEGER OPTIONAL,

causeForRecordClosing				[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication			[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier				[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components		[21] SEQUENCE OF Media-Components-List OPTIONAL,

serviceReasonReturnCode				[23] UTF8String OPTIONAL,

recordExtensions					[25] ManagementExtensions OPTIONAL,

expiresInformation					[26] INTEGER OPTIONAL,

event								[28] UTF8String OPTIONAL,

serviceContextID					[30] ServiceContextID OPTIONAL,

list-Of-Early-SDP-Media-Components 	[32] SEQUENCE OF Early-Media-Components-List OPTIONAL,

numberPortabilityRouting			[34] NumberPortabilityRouting OPTIONAL,

carrierSelectRouting				[35] CarrierSelectRouting OPTIONAL,

sessionPriority						[36] SessionPriority OPTIONAL,

serviceRequestTimeStampFraction		[37] Milliseconds OPTIONAL,

serviceDeliveryStartTimeStampFraction [38] Milliseconds OPTIONAL,

serviceDeliveryEndTimeStampFraction	[39] Milliseconds OPTIONAL

}

ASRecord 	::= SET

{

recordType							[0] RecordType,

retransmission						[1] NULL OPTIONAL,

sIP-Method							[2] SIP-Method OPTIONAL,

role-of-Node						[3] Role-of-Node OPTIONAL,

nodeAddress							[4] NodeAddress OPTIONAL,

session-Id							[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address		[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address				[7] InvolvedParty OPTIONAL,

serviceRequestTimeStamp				[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp		[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp			[11] TimeStamp OPTIONAL,

recordOpeningTime					[12] TimeStamp OPTIONAL,

recordClosureTime					[13] TimeStamp OPTIONAL,

interOperatorIdentifiers			[14] InterOperatorIdentifiers OPTIONAL,

localRecordSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber				[16] INTEGER OPTIONAL,

causeForRecordClosing				[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication			[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier				[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components		[21] SEQUENCE OF Media-Components-List OPTIONAL,

gGSNaddress							[22] NodeAddress OPTIONAL,

serviceReasonReturnCode				[23] UTF8String OPTIONAL,

list-Of-Message-Bodies				[24] SEQUENCE OF MessageBody OPTIONAL,

recordExtensions					[25] ManagementExtensions OPTIONAL,

expiresInformation					[26] INTEGER OPTIONAL,

event								[28] UTF8String OPTIONAL,

accessNetworkInformation			[29] OCTET STRING OPTIONAL,

serviceContextID					[30] ServiceContextID OPTIONAL,

list-of-subscription-ID				[31] SEQUENCE OF SubscriptionID OPTIONAL,

list-Of-Early-SDP-Media-Components 	[32] SEQUENCE OF Early-Media-Components-List OPTIONAL,

iMSCommunicationServiceIdentifier	[33] IMSCommunicationServiceIdentifier OPTIONAL,

numberPortabilityRouting			[34] NumberPortabilityRouting OPTIONAL,

carrierSelectRouting				[35] CarrierSelectRouting OPTIONAL,

sessionPriority						[36] SessionPriority OPTIONAL,

serviceRequestTimeStampFraction		[37] Milliseconds OPTIONAL,

serviceDeliveryStartTimeStampFraction [38] Milliseconds OPTIONAL,

serviceDeliveryEndTimeStampFraction	[39] Milliseconds OPTIONAL,

online-charging-flag				[43] NULL OPTIONAL,

realTimeTariffInformation			[44] SEQUENCE OF RealTimeTariffInformation OPTIONAL,

serviceSpecificInfo					[100] SEQUENCE OF ServiceSpecificInfo OPTIONAL,

requested-Party-Address				[101] InvolvedParty OPTIONAL,

list-Of-Called-Asserted-Identity 	[102] ListOfInvolvedParties OPTIONAL,

alternateChargedPartyAddress		[103] UTF8string OPTIONAL,

outgoingSessionId 					[104] Session-Id OPTIONAL

}

ECSCFRecord 	::= SET

{

recordType							[0] RecordType,

retransmission						[1] NULL OPTIONAL,

sIP-Method							[2] SIP-Method OPTIONAL,

role-of-Node						[3] Role-of-Node OPTIONAL,

nodeAddress							[4] NodeAddress OPTIONAL,

session-Id							[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address		[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address				[7] InvolvedParty OPTIONAL,

privateUserID						[8] GraphicString OPTIONAL,

serviceRequestTimeStamp				[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp		[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp			[11] TimeStamp OPTIONAL,

recordOpeningTime					[12] TimeStamp OPTIONAL,

recordClosureTime					[13] TimeStamp OPTIONAL,

interOperatorIdentifiers			[14] InterOperatorIdentifierlist OPTIONAL,

localRecordSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber				[16] INTEGER OPTIONAL,

causeForRecordClosing				[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication			[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier				[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components		[21] SEQUENCE OF Media-Components-List OPTIONAL,

gGSNaddress							[22] NodeAddress OPTIONAL,

serviceReasonReturnCode				[23] UTF8String OPTIONAL,

list-Of-Message-Bodies				[24] SEQUENCE OF MessageBody OPTIONAL,

recordExtensions					[25] ManagementExtensions OPTIONAL,

expiresInformation					[26] INTEGER OPTIONAL,

list-Of-Associated-URI				[27] ListOfInvolvedParties OPTIONAL,

event								[28] UTF8String OPTIONAL,

accessNetworkInformation			[29] OCTET STRING OPTIONAL,

serviceContextID					[30] ServiceContextID OPTIONAL,

list-of-subscription-ID				[31] SEQUENCE OF SubscriptionID OPTIONAL,

list-Of-Early-SDP-Media-Components 	[32] SEQUENCE OF Early-Media-Components-List OPTIONAL,

iMSCommunicationServiceIdentifier	[33] IMSCommunicationServiceIdentifier OPTIONAL,

numberPortabilityRouting			[34] NumberPortabilityRouting OPTIONAL,

carrierSelectRouting				[35] CarrierSelectRouting OPTIONAL,

sessionPriority						[36] SessionPriority OPTIONAL,

serviceRequestTimeStampFraction		[37] Milliseconds OPTIONAL,

serviceDeliveryStartTimeStampFraction [38] Milliseconds OPTIONAL,

serviceDeliveryEndTimeStampFraction	[39] Milliseconds OPTIONAL,

applicationServersInformation		[40] SEQUENCE OF ApplicationServersInformation OPTIONAL,

requested-Party-Address				[41] InvolvedParty OPTIONAL,

list-Of-Called-Asserted-Identity 	[42] ListOfInvolvedParties OPTIONAL

}

--

-- Editor’s note: The completeness of all parameters for the E-CSCF CDR is ffs.

--

IBCFRecord 	::= SET

{

recordType							[0] RecordType,

retransmission						[1] NULL OPTIONAL,

sIP-Method							[2] SIP-Method OPTIONAL,

role-of-Node						[3] Role-of-Node OPTIONAL,

nodeAddress							[4] NodeAddress OPTIONAL,

session-Id							[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address		[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address				[7] InvolvedParty OPTIONAL,

serviceRequestTimeStamp				[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp		[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp			[11] TimeStamp OPTIONAL,

recordOpeningTime					[12] TimeStamp OPTIONAL,

recordClosureTime					[13] TimeStamp OPTIONAL,

interOperatorIdentifiers			[14] InterOperatorIdentifiers OPTIONAL,

localRecordSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber				[16] INTEGER OPTIONAL,

causeForRecordClosing				[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication			[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier				[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components		[21] SEQUENCE OF Media-Components-List OPTIONAL,

serviceReasonReturnCode				[23] UTF8String OPTIONAL,

recordExtensions					[25] ManagementExtensions OPTIONAL,

expiresInformation					[26] INTEGER OPTIONAL,

event								[28] UTF8String OPTIONAL,

serviceContextID					[30] ServiceContextID OPTIONAL,

list-Of-Early-SDP-Media-Components 	[32] SEQUENCE OF Early-Media-Components-List OPTIONAL,

iMSCommunicationServiceIdentifier	[33] IMSCommunicationServiceIdentifier OPTIONAL,

sessionPriority						[36] SessionPriority OPTIONAL,

serviceRequestTimeStampFraction		[37] Milliseconds OPTIONAL,

serviceDeliveryStartTimeStampFraction [38] Milliseconds OPTIONAL,

serviceDeliveryEndTimeStampFraction	[39] Milliseconds OPTIONAL,

realTimeTariffInformation			[44] SEQUENCE OF RealTimeTariffInformation OPTIONAL

}

------------------------------------------------------------------------------

--

--  IMS DATA TYPES

--

------------------------------------------------------------------------------

AccessCorrelationID ::= CHOICE

{

gPRS-Charging-Id				[2] INTEGER,

accessNetworkChargingIdentifier	[4] GraphicString

}

ACRInterimLost ::= ENUMERATED

{

no 		(0),

yes 	(1),

unknown (2)

}

ApplicationServersInformation ::= SEQUENCE 
{

applicationServersInvolved 			[0] NodeAddress OPTIONAL,

applicationProvidedCalledParties	[1] SEQUENCE OF InvolvedParty OPTIONAL

}

CarrierSelectRouting ::= GraphicString

CauseForRecordClosing ::= ENUMERATED

{

serviceDeliveryEndSuccessfully	(0),

unSuccessfulServiceDelivery		(1),

timeLimit						(3),

serviceChange					(4), -- e.g. change in media due to Re-Invite

managementIntervention			(5)  -- partial record generation reasons to be added

--  Additional codes are for further study

}

- Early-Media-Components-List ::= SEQUENCE

{

sDP-Offer-Timestamp 	[0] TimeStamp OPTIONAL,

sDP-Answer-Timestamp	[1] TimeStamp OPTIONAL,

sDP-Media-Components	[2] SEQUENCE OF SDP-Media-Component OPTIONAL,

mediaInitiatorFlag		[3] NULL OPTIONAL,

sDP-Session-Description	[4] SEQUENCE OF GraphicString OPTIONAL,

sDP-Type				[5] SDP-Type OPTIONAL

}

IMS-Charging-Identifier ::= OCTET STRING

IMSCommunicationServiceIdentifier	::= OCTET STRING

Incomplete-CDR-Indication ::= SET

{

aCRStartLost	[0] BOOLEAN, –- TRUE if ACR[Start] was lost, FALSE otherwise

aCRInterimLost	[1] ACRInterimLost,

aCRStopLost		[2] BOOLEAN –- TRUE if ACR[Stop] was lost, FALSE otherwise

}

InterOperatorIdentifierlist ::= SEQUENCE OF InterOperatorIdentifiers

InterOperatorIdentifiers ::= SEQUENCE

{

originatingIOI[0] GraphicString OPTIONAL,

terminatingIOI[1] GraphicString OPTIONAL

}

InvolvedParty ::= CHOICE

{

sIP-URI[0] GraphicString, -- refer to rfc3261 [401]

tEL-URI[1] GraphicString  -- refer to rfc3966 [402]

}

Editor’s note:  the constructs below are imported from the generic module

ListOfInvolvedParties ::= SEQUENCE OF InvolvedParty

Media-Components-List ::= SEQUENCE

--

-- MediaInitiatorParty is used to identify the initiator of the media

-- multi-participants session e.g. in AS PoC Server

--

{

sIP-Request-Timestamp 			 [0] TimeStamp OPTIONAL,

sIP-Response-Timestamp			 [1] TimeStamp OPTIONAL,

sDP-Media-Components			 [2] SEQUENCE OF SDP-Media-Component OPTIONAL,

mediaInitiatorFlag				 [3] NULL OPTIONAL,

sDP-Session-Description			 [4] SEQUENCE OF GraphicString OPTIONAL,

mediaInitiatorParty				 [5] InvolvedParty OPTIONAL,

sIP-Request-Timestamp-Fraction	 [6] Milliseconds OPTIONAL,

sIP-Response-Timestamp-Fraction	 [7] Milliseconds OPTIONAL,

sDP-Type						 [8] SDP-Type OPTIONAL

}

MessageBody ::= SEQUENCE

{

content-Type			[0] GraphicString,

content-Disposition		[1] GraphicString OPTIONAL,

content-Length			[2] INTEGER,

originator				[3] InvolvedParty OPTIONAL

}

Milliseconds ::= INTEGER (0..999)

NumberPortabilityRouting ::= GraphicString

RateElement ::= SEQUENCE

{

unitType				[0] INTEGER,

unitValue				[1] REAL,

unitCost				[2] REAL,

unitQuotaThreshold		[3] REAL

}

RealTimeTariffInformation ::= CHOICE

{

tariffInformation		[0] TariffInformation OPTIONAL,

tariffXml				[1] UTF8string OPTIONAL

}

Role-of-Node ::= ENUMERATED

{

originating (0),

terminating (1)

}

S-CSCF-Information ::=  SEQUENCE

{

mandatoryCapabilities [0] SEQUENCE OF GraphicString OPTIONAL,

optionalCapabilities	 [1] SEQUENCE OF GraphicString OPTIONAL,

serverName 				 [2] GraphicString OPTIONAL

}

SDP-Media-Component ::=  SEQUENCE

{

sDP-Media-Name        	[0] GraphicString OPTIONAL,

sDP-Media-Descriptions	[1] SDP-Media-Description OPTIONAL,

accessCorrelationID			AccessCorrelationID OPTIONAL, 		-- not used in MGCF and BGCF

-- [2] is used by gPRS-Charging-Id

-- [4] is used by accessNetworkChargingIdentifier

authorized-QoS        	[3] GraphicString OPTIONAL,

}

SDP-Media-Description ::= SEQUENCE OF GraphicString

ServedPartyIPAddress  ::=  IPAddress

Service-Id ::= GraphicString

Session-Id ::= GraphicString

--

-- rfc3261 [401]: example for SIP Call-ID: f81d4fae-7dec-11d0-a765-00a0c91e6bf6@foo.bar.com

--

SessionPriority :: = ENUMERATED

{

PRIORITY-0 (0),

PRIORITY-1 (1),

PRIORITY-2 (2),

PRIORITY-3 (3),

PRIORITY-4 (4)

}

--

-- PRIORITY-4 is the highest priority and Priority-0 is the lowest priority.

--

SIP-Method ::= GraphicString

SDP-Type ::= ENUMERATED

{

sDP-offer (0),

sDP-answer (1)

}

TariffInformation ::= SEQUENCE

{

currencyCode 			[0] INTEGER,

scaleFactor				[1] REAL,

rateElements			[2] SEQUENCE OF RateElement OPTIONAL

}

TransmissionMedium ::= SEQUENCE

{

tMR	[0] OCTET STRING (SIZE (1)) OPTIONAL, -- required TM, refer to Q.763

tMU	[1] OCTET STRING (SIZE (1)) OPTIONAL  -- used TM, refer to Q.763

}

TrunkGroupID ::= CHOICE

{

incoming	[0] GraphicString,

outgoing	[1] GraphicString

}

END

End of changes