u3GPP TSG-SA5 (Telecom Management)	S5-111474

SA5#76, 28 Feb - 4 Mar 2011; San Diego, USA	revision of S5-111100

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.251 | CR | 0156 |  rev | 3 |  Current version: | 10.3.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Adding CDR fields needed for Machine Type Communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | NIMTC-CH |  |  |  |  |  |  | Date:  |  | 04/03/2011 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change:  |  | Machine Type Communication introduces some features on EPC and information about these is valuable for statistics in the Billing Domain. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Adding a new field “Low priority indicator” in the CDRs from the EPC nodes. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | No indication available about Machine Type Communication. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 6.1.2, 6.1.3, 6.3.1.2, 6.3.2, 6.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  | X |  | O&M Specifications |  |  | 32.298, 32.299 |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

First change

### 6.1.2	IP CAN bearer charging data in S-GW (SGW-CDR)

If FBC is disabled and the collection of CDR data is enabled then the S-GW data specified in the following table shall be available for each IP CAN bearer.

Table 6.1.2: S-GW IP CAN bearer data (SGW-CDR)

| Field | Category | Description |
| --- | --- | --- |
| Record Type | M | S-GW IP CAN bearer record. |
| Served IMSI | C | IMSI of the served party, if available. |
| IMSI Unauthenticated Flag | OC | This field indicates the provided served IMSI is not authenticated (emergency bearer service situation). |
| Served IMEISV | OC | IMEISV of the ME, if available. It is used for identifying the user in case Served IMSI is not present during emergency bearer service. |
| S-GW Address used | M | The control plane IP address of the S-GW used. |
| Charging ID | M | IP CAN bearer Charging identifier used to identify this IP CAN bearer in different records created by PCNs |
| PDN Connection Id | OM | This field holds the PDN connection (IP-CAN session) identifier to identify different records belonging to same PDN connection. |
| Serving Node Address | M | List of serving node control plane IP addresses (e.g. SGSN, MME, …) used during this record. |
| Serving node Type | M | List of serving node types in control plane. The serving node types listed here map to the serving node addresses listed in the field "Serving node Address" in sequence. |
| S-GW Change | OC | Present if this is first record after S-GW change. |
| PGW PLMN Identifier | Oc | PLMN identifier (MCC MNC) of the PGW used. |
| Access Point Name Network Identifier | OM | The logical name of the connected access point to the external packet data network (network identifier part of APN). |
| PDP/PDN Type | OM | This field indicates PDN type (i.e IPv4, IPv6 or IPv4v6). |
| Served PDP/PDN Address | OC | IP address allocated for the PDP context / PDN connection, if available, i.e. IPv4 when PDN Type is IPv4 or IPv6 when PDN Type is IPv6 or IPv4v6. |
| Served PDP/PDN Address extension | OC | This field holds IPv4 address of the served IMSI, if available, when PDN type is IPv4v6. |
| Dynamic Address Flag | OC | Indicates whether served PDP/PDN address is dynamic, which is allocated during IP CAN bearer activation, initial attach (E-UTRAN or over S2x) and UE requested PDN connectivity. This field is missing if address is static. |
| List of Traffic Data Volumes | OM | A list of changes in charging conditions for this QCI/ARP pair, each change is time stamped. Charging conditions are used to categorize traffic volumes, such as per tariff period. Initial and subsequently changed QoS and corresponding data values are also listed. |
| Record Opening Time | M | Time stamp when IP CAN bearer is activated in this S-GW or record opening time on subsequent partial records. |
| MS Time Zone | OC | This field contains the MS Time Zone the MS is currently located as defined in TS 29.060 [203], if available. |
| Duration | M | Duration of this record in the S-GW. |
| Cause for Record Closing | M | The reason for the release of record from this S-GW. |
| Diagnostics | OM | A more detailed reason for the release of the connection. |
| Record Sequence Number | C | Partial record sequence number, only present in case of partial records. |
| Node ID | OM | Name of the recording entity. |
| Record Extensions | OC | A set of network operator/manufacturer specific extensions to the record. Conditioned upon the existence of an extension. |
| Local Record Sequence Number | OM | Consecutive record number created by this node. The number is allocated sequentially including all CDR types. |
| APN Selection Mode | OM | An index indicating how the APN was selected. |
| Served MSISDN | OM | The primary MSISDN of the subscriber. |
| User Location Information | OC | This field contains the User Location Information of the MS as defined in TS 29.060 [203] for GPRS case, and in TS 29.274 [210] for EPC case, if available. |
| User CSG information | OC | This field contains the User CSG information of the UE, if available, including CSG ID, access mode and CSG membership indication. |
| Charging Characteristics | M | The Charging Characteristics applied to the IP CAN bearer. |
| Charging Characteristics Selection Mode | OM | Holds information about how Charging Characteristics were selected. |
| IMS Signalling Context | OC | Included if the IM-CN Subsystem Signalling Flag is set, see [201] IP CAN bearer is used for IMS signalling. |
| P-GW Address used. | OC | This field is the P-GW IP Address for the Control Plane |
| Serving Node PLMN Identifier | OC | Serving node PLMN Identifier (MCC and MNC) used during this record, if available. |
| RAT Type | OC | This field indicates the Radio Access Technology (RAT) type currently used by the Mobile Station, when available. This RAT type is defined in TS 29.060 [204] for GTP case, in TS 29.274 [210] for eGTP case and in TS 29.275 [211] for PMIP case. |
| Start Time |  | This field holds the time when User IP-CAN session starts, available in the CDR for the first bearer in an IP-CAN session. |
| Stop Time |  | This field holds the time when User IP-CAN session is terminated, available in the CDR for the last bearer in an IP-CAN session. |
| Low Priority Indicator |  | This field indicates if this IP-CAN session has a low priority, i.e. for Machine Type Communication. |

Next change

### 6.1.3	FBC IP CAN bearer charging data in P-GW (PGW-CDR)

If FBC is enabled and the collection of CDR data is enabled then the P-GW data specified in the following table shall be available for each IP CAN bearer.

Table 6.1.3: P-GW IP CAN bearer data (PGW-CDR)

| Field | Category | Description |
| --- | --- | --- |
| Record Type | M | P-GW IP CAN bearer record. |
| Served IMSI | C | IMSI of the served party, if available. |
| IMSI Unauthenticated Flag | OC | This field indicates the provided served IMSI is not authenticated (emergency bearer service situation). |
| Served IMEISV | OC | IMEISV of the ME, if available. It is used for identifying the user in case Served IMSI is not present during emergency bearer service. |
| Served 3GPP2 MEID | OC | MEID of the served party’s terminal equipment for 3GPP2 access. |
| Served MN NAI | OC | Mobile Node Identifier in NAI format (based on IMSI), if available. |
| P-GW Address used | M | The control plane IP address of the P-GW used. |
| Charging ID | M | IP CAN bearer Charging identifier used to identify this IP CAN bearer in different records created by PCNs |
| PDN Connection Id | OM | This field holds the PDN connection (IP-CAN session) identifier to identify different records belonging to same PDN connection. |
| Serving node Address | M | List of SGSN/S-GW control plane IP addresses, or the ePDG address used during this record. |
| Serving node Type | M | List of serving node types in control plane. The serving node types listed here map to the serving node addresses listed in the field "Serving node Address" in sequence. |
| PGW PLMN Identifier | Oc | PLMN identifier (MCC MNC) of the PGW. |
| Access Point Name Network Identifier | OM | The logical name of the connected access point to the external packet data network (network identifier part of APN). |
| PDP/PDN Type | OM | PDP/PDN type, i.e. IPv4, IPv6, IPv4v6 , or PDP type PPP, or IHOSS:OSP. |
| Served PDP/PDN Address | OC | IP address allocated for the PDP context / PDN connection, i.e. IPv4 when PDP/PDN Type is IPv4 or IPv6 when PDP/PDN Type is IPv6 or IPv4v6. This parameter shall be present except when both the PDP type is PPP and dynamic IP CAN bearer address assignment is used. |
| Served PDP/PDN Address extension | OC | This field holds IPv4 address of the served IMSI, if available, when PDP/PDN type is IPv4v6. |
| Dynamic Address Flag | OC | Indicates whether served PDP/PDN address is dynamic, which is allocated during IP CAN bearer activation, initial attach (E-UTRAN or over S2x) and UE requested PDN connectivity. This field is missing if address is static. |
| List of Service Data | OM | A list of changes in charging conditions for all service data flows within this IP CAN bearer categorized per rating group or per combination of the rating group and service id. Each change is time stamped. Charging conditions are used to categorize traffic volumes, elapsed time and number of events, such as per tariff period. Initial and subsequently changed QoS and corresponding data values are also listed. Online charging information (PS Furnish Charging Information) may be added per each service data flow container in case it is sent by the OCS. Failure-Handling: This field shall be present in case P-GW triggers the Failure-Handling procedure. It shall indicate the Failure Handling scenario and the instant the Failure Action is triggered (see annex B). Scenarios: Continue/New Session; Continue/Ongoing Session; Retry&Terminate/Ongoing Session; Terminate/Ongoing Session. |
| Record Opening Time | M | Time stamp when IP CAN bearer is activated in this P-GW or record opening time on subsequent partial records. |
| MS Time Zone | OC | This field contains the MS Time Zone the MS is currently located as defined in TS 29.060 [203], if available. |
| Duration | M | Duration of this record in the P-GW. |
| Cause for Record Closing | M | The reason for the release of record from this P-GW. |
| Diagnostics | OM | A more detailed reason for the release of the connection. |
| Record Sequence Number | C | Partial record sequence number, only present in case of partial records. |
| Node ID | OM | Name of the recording entity. |
| Record Extensions | OC | A set of network operator/manufacturer specific extensions to the record. Conditioned upon the existence of an extension. |
| Local Record Sequence Number | OM | Consecutive record number created by this node. The number is allocated sequentially including all CDR types. |
| APN Selection Mode | OM | An index indicating how the APN was selected. |
| Served MSISDN | OM | The primary MSISDN of the subscriber. |
| User Location Information | OC | This field contains the User Location Information of the MS as defined in TS 29.060 [203] for GPRS case, and in TS 29.274 [210] for EPC case, if available. |
| User CSG information | OC | This field contains the User CSG Information of the UE, if available, including CSG ID, access mode and CSG membership indication. |
| 3GPP2 User Location information | OC | This field contains the User Location Information of the MS as defined in TS 29.212 [71] for 3GPP2 access, if available. |
| Charging Characteristics | M | The Charging Characteristics applied to the IP CAN bearer. |
| Charging Characteristics Selection Mode | OM | Holds information about how Charging Characteristics were selected. |
| IMS Signalling Context | OC | Included if the IP CAN bearer IM-CN Subsystem Signalling Flag is set, see [201]is used for IMS signalling. |
| External Charging Identifier | OC | A Charging Identifier received from a non-EPC, external network entity e.g ICID. |
| Serving node PLMN Identifier | OM | Serving node PLMN Identifier (MCC and MNC) used during this record. |
| PS Furnish Charging Information | OC | Online charging session specific information |
| CAMEL Information | OC | Set of CAMEL information related to IP CAN bearer, if available. This field applies only for GPRS. |
| RAT Type | OC | This field indicates the Radio Access Technology (RAT) type currently used by the Mobile Station, when available. This RAT type is defined in TS 29.060 [204] for GTP case, in TS 29.274 [210] for eGTP case and in TS 29.275 [211] for PMIP case. |
| Start Time | OC | This field holds the time when User IP-CAN session starts, available in the CDR for the first bearer in an IP-CAN session. |
| Stop Time | OC | This field holds the time when User IP-CAN session is terminated, available in the CDR for the last bearer in an IP-CAN session. |
| Low Priority Indicator | OC | This field indicates if this IP-CAN session has a low priority, i.e. for Machine Type Communication. |

Next change

#### 6.3.1.2	Definition of the PS Information

PS specific charging information is provided within the PS Information. The fields of the PS information are indicated with the node (S-GW, P-GW) from which the information is sent.

The detailed structure of the PS Information can be found in the table 6.3.1.2.

Table 6.3.1.2 : Structure of the PS Information

| Field | Category | Description |
| --- | --- | --- |
| Charging Id | OC | This field holds the Charging Id for this IP CAN bearer (this together with the P-GW Address constitutes a unique identifier for the IP CAN bearer). |
| Node Id | OC | This fields holds the name of the Node |
| PDN Connection Id | OC | This field holds the PDN connection (IP-CAN session) identifier to identify different records belonging to same PDN connection. |
| PDP/PDN Type | OC | This field holds the type of IP CAN bearer, e.g. IP or PPP, or PDN type (i.e IPv4, IPv6 or IPv4v6). |
| PDP/PDN Address | OC | This field holds the IP address of the served IMSI allocated for the PDP context / PDN connection, i.e. IPv4 or IPv6. This parameter shall be present except when both the PDP type is PPP and dynamic PDP address assignment is used. It may occur twice within the PS Information field when PDN type is IPv4v6: first occurrence with IPv6 prefix, second occurrence with IPv4 address. |
| Dynamic Address Flag | OC | This field indicates whether served PDP/PDN address is dynamically allocated. This field is missing if address is static. |
| Negotiated QoS Profile | OC | This field holds the authorized QoS applied to IP CAN bearer. See NOTE. |
| Serving Node Address | OC | This field holds the SGSN/S-GW IP address that is used by the control plane for the handling of control messages, or the AGW IP address, or the ePDG address. It may be used to identify the PLMN to which the user is attached. |
| Serving Node Type | OC | This field holds the type of the serving node. |
| SGW Change | OC | This field is present if this is first ACR after S-GW change. |
| PGW Address | OC | This field holds the IP-address of the P-GW that generated the Charging Id. |
| CG Address | OC | This field holds the Charging Gateway IP address if available. Only used in EPC Online Charging. |
| IMSI MCC MNC | OC | This field holds the MCC and MNC extracted from the user's IMSI (first 5 or 6 digits, as applicable from the presented IMSI). |
| IMSI Unauthenticated Flag | OC | This field is present when IMSI is provided in Subscription-Id and this IMSI is unauthenticated (due to emergency bearer service situations). |
| PGW MCC MNC | OC | This field holds the MCC-MNC of the network the P-GW belongs to. |
| NSAPI | OC | This field Identifies a particular IP CAN bearer for the associated PDN and MSISDN/IMSI from creation to deletion. See NOTE. |
| Called Station Id | OC | This field contains the identifier of the access point (APN) the user is connected to. |
| Session Stop Indicator | OC | This field indicates to the OCS that the last IP CAN bearer of a session is released and that the IP CAN session has been terminated. |
| Selection Mode | OC | This field contains the Selection mode for this APN received in the Create PDP context request, on E-UTRAN initial attach and UE requested PDN connectivity. |
| Charging Characteristics | OC | This field contains the Charging Characteristics for this IP CAN bearer received in the Create IP CAN bearer Request Message (only available in R99 and later releases). |
| Charging Characteristics Selection Mode | OC | This field holds information about how the "Charging Characteristics" was selected. |
| Serving Node MCC MNC | OC | This field holds the MCC and MNC extracted from the RAI within the IP CAN beareractivation or Update messages, or provided by SGW/AGW. |
| MS Time Zone | OC | This field indicates the offset between universal time and local time in steps of 15 minutes of where the MS currently resides. |
| Charging Rule Base Name | OC | This field indicates the name of a pre-defined group of PCC rules residing at the PCEF. It may occur several times within the PS Information field. |
| User Location Info | OC | This field indicates details of where the UE is currently located (e.g. SAI, TAI, RAI, CGI, ECGI or access-specific user location information). |
| User CSG Information | OC | This field indicates details of the User CSG Information of the UE, if available, including CSG ID, access mode and CSG membership indication. |
| 3GPP2 User Location Info | OC | This field holds the 3GPP2 User Location Info of where the UE is currently located (i.e. 3GPP2 BSID: Cell-Id, SID, NID), as defined in TS 29.212 [71] |
| RAT Type | OC | This field indicates which Radio Access Technology is currently serving the UE. |
| Furnish Charging Information | OC | This field contains the PS Furnish Information Elements. |
| Offline Charging | OC | This field contains the Offline Charging parameters to control offline charging. |
| PDP Context Type | OC | This field indicates the type of a PDP context (i.e. Primary or Secondary). This field is present when using the Gn/Gp reference point only. |
| Traffic data volumes | OC | This field holds the containers associated to a charging condition change on an IP-CAN bearer . This is included when triggers conditions are met (Qos change, tariff time change ..). It may occur several times within the PS Information field. |
| Service data container | OC | This field holds the container associated to a service condition change on a service data flow (categorized per rating group or per combination of the rating group and service id) within this IP CAN bearer. It may occur several times within the PS Information field. |
| User-Equipment-Info | OC | This field holds the identification of the terminal (IMEISV…) It is used for identifying the user in case IMSI is not present during emergency bearer service. Editor’s Note: Alignement with online charging is needed. |
| Terminal Information | OC | This field holds the identification of the terminal (IMEISV, 3GPP2-MEID..).It is used for identifying the user in case IMSI is not present during emergency bearer service. Editor’s Note : this parameter is only for offline charging and use for online charging instead of User-Equipment-info is ffs |
| start time | OC | Timestamp when User IP-CAN session starts |
| stop time | OC | Timestamp when User IP-CAN session terminates. |
| Change Condition | OC | This field holds the reason for sending ACR from the PCN Nodes. |
| Diagnostics | OC | This field holds a more detailed reason for the release of the connection, and complements the "Change Condition" information |
| Low Priority Indicator | OC | This field indicates if this IP-CAN session has a low priority, i.e. for Machine Type Communication. |

NOTE:	For Network Requested Secondary IP CAN bearer Activation, the CCR Initial does not contain a value of NSAPI, whilst the Negotiated QoS profile reflects the requested QoS profile used in the activation request.

Next change

### 6.3.2	Detailed Message Format for offline charging

The following chapter specifies per Operation Type the charging data that are sent by S-GW and P-GW.

The Operation Types are listed in the following order: S (start)/I (interim)/S (stop)/E (event). Therefore, when all Operation Types are possible it is marked as SISE. If only some Operation Types are allowed for a node, only the appropriate letters are used (i.e. SIS or E) as indicated in the table heading. The omission of an Operation Type for a particular field is marked with "-" (i.e. SI-E). Also, when an entire field is not allowed in a node the entire cell is marked as "-".

Table 6.3.2.1 illustrates the basic structure of the supported fields in the Accounting Request message for PS offline charging.

Table 6.3.2.1 : Supported fields in Accounting Request Message

| Field | Node Type | S-GW | P-GW |
| --- | --- | --- | --- |
| Supported Operation Types |  | S/I/S/E | S/I/S/E |
| Session-Id |  | SIS- | SIS- |
| Origin-Host |  | SIS- | SIS- |
| Origin-Realm |  | SIS- | SIS- |
| Destination-Realm |  | SIS- | SIS- |
| Accounting-Record-Type |  | SIS- | SIS- |
| Accounting-Record-Number |  | SIS- | SIS- |
| Acct-Application-Id |  | SIS- | SIS- |
| User -Name |  | SIS- | SIS- |
| Acct-Interim-Interval |  | SIS- | SIS- |
| Origin-State-Id |  | SIS- | SIS- |
| Event-Timestamp |  | SIS- | SIS- |
| Proxy-Info |  | SIS- | SIS- |
| Route-Record |  | SIS- | SIS- |
| Service-Context-Id |  | SIS- | SIS- |
| Service Information with PS and IMS Information |  |  |  |
| Subscription-Id |  | SIS- | SIS- |
| IMSI Unauthenticated Flag |  | SIS- | SIS- |
| Node Functionality |  | SIS- | SIS- |
| Charging Id |  | SIS- | SIS- |
| Node Id |  | SIS- | SIS- |
| PDN Connection Id |  | SIS- | SIS- |
| PDP/PDN Type |  | SIS- | SIS- |
| PDP/PDN Address |  | SIS- | SIS- |
| Dynamic Address Flag |  | SIS- | SIS- |
| Negotiated QoS Profile |  | - | - |
| S-GW Address |  | - | SIS- |
| Serving Node Type |  | SIS- | SIS- |
| SGW Change |  | S--- | - |
| P-GW Address |  | SIS- | - |
| CG Address |  | - | - |
| IMSI MCC MNC |  | SIS- | SIS- |
| PGW MCC MNC |  | SIS- | SIS- |
| NSAPI |  | - | SIS- |
| Called Station Id |  | SIS- | SIS- |
| Session Stop Indicator |  | - | --S- |
| Selection Mode |  | SIS- | SIS- |
| Charging Characteristics |  | SIS- | SIS- |
| Charging Characteristics Selection Mode |  | SIS- | SIS- |
| Serving Node MCC MNC |  | SIS- | SIS- |
| MS Time Zone |  | SIS- | SIS- |
| Charging Rule Base Name |  | - | SIS- |
| User Location Info |  | SIS- | SIS- |
| User CSG Information |  | SIS- | SIS- |
| 3GPP2 User Location Info |  | SIS- | SIS- |
| RAT Type |  | SIS- | SIS- |
| Furnish Charging Information |  | - | SIS- |
| Offline Charging |  | - | - |
| QoS Information |  | SIS- | SIS- |
| Traffic data volumes |  | -IS- | - |
| Service data container |  | - | -IS- |
| User-Equipment-Info |  | - | - |
| Terminal-Information |  | S--- | S--- |
| Start time |  | S--- | S--- |
| Stop time |  | --S- | --S- |
| Change Condition |  | -IS- | -IS- |
| Diagnostics |  | --S- | --S- |
| IMS Charging Identifier |  | SIS- | SIS- |
| Low Priority Indicator |  | S--- | S--- |

Table 6.3.2.2 illustrates the basic structure of the supported fields in the Accounting Response message for PS offline charging.

Table 6.3.2.2 : Supported fields in Accounting Response Message

| Field | Node Type | S-GW | P-GW |
| --- | --- | --- | --- |
|  | Supported Operation Types | S/I/S/E | S/I/S/E |
| Session-Id |  | SIS- | SIS- |
| Origin-Host |  | SIS- | SIS- |
| Origin-Realm |  | SIS- | SIS- |
| Destination-Realm |  | SIS- | SIS- |
| Accounting-Record-Type |  | SIS- | SIS- |
| Accounting-Record-Number |  | SIS- | SIS- |
| Acct-Application-Id |  | SIS- | SIS- |
| User -Name |  | SIS- | SIS- |
| Acct-Interim-Interval |  | SIS- | SIS- |
| Origin-State-Id |  | SIS- | SIS- |
| Event-Timestamp |  | SIS- | SIS- |
| Proxy-Info |  | SIS- | SIS- |
| Route-Record |  | SIS- | SIS- |

Next change

## 6.5	Bindings for EPC Offline Charging

This clause aims to describe the mapping between the Diameter messages AVP and CDR parameter for EPC Offline Charging.

Table 6.5.1 describes the mapping of the Diameter Accounting AVP to the CDR paramter of S-GW-CDR and P-GW-CDR in EPC Offline Charging.

Table 6.5.1: Bindings of Accounting AVP to S-/P-GW-CDR parameter

| Diameter ACR AVP | S-/P-GW-CDR field |
| --- | --- |
| Called-Station-Id | Access Point Name Network Identifier |
| AF-Correlation-Information | AF-record-information |
| 3GPP-Selection-Mode | APN Selection Mode |
| Change-Condition | Change Condition |
| Change-Time | Change Time |
| 3GPP-Charging-Characteristics | Charging Characteristics |
| Charging-Characteristics-Selection-Mode | Charging Characteristics Selection Mode |
| 3GPP-Charging-Id | Charging Id |
| Charging-Rule-Base-Name | Charging Rule Base Name |
| Change-Condition | Cause for Record Closing |
| Accounting-Input-Octets | Data Volume Uplink |
| Accounting-Output-Octets | Data Volume Downlink |
| Diagnostics | Diagnostics |
| Dynamic-Address-Flag | Dynamic Address Flag |
| IMSI-Unauthenticated-Flag | IMSI Unauthenticated Flag |
| Traffic-Data-Volumes | List of Traffic Data Volumes |
| Service-Data-Container | List of Service Data |
| 3GPP-MS-Time-Zone | MS Time Zone |
| Node ID | Node ID |
| PDN-Connection-ID | PDN Connection Id |
| 3GPP-PDP-Type | PDP/PDN Type |
| 3GPP-GGSN-MCC-MNC | PGW PLMN Identifier |
| GGSN-Address | PGW Address Used |
| Qos-Information | EPC QoS Information/Qos Information |
| 3GPP-RAT-Type | RAT Type |
| Rating-Group | Rating Group |
| Node-Functionality | Record Type |
| Change-Time | Report Time |
| Result-Code | Result Code |
| Change-Condition | Service Condition Change |
| Service-Identifier | Service Identifier |
| Subscription-Id | Served IMSI |
| Terminal-Information | Served IMEISV |
| Subscription-Id | Served MN NAI |
| Subscription-Id | Served MSISDN |
| Terminal-Information | Served 3GPP2 MEID |
| 3GPP-SGSN-MCC-MNC | Serving node PLMN Identifier |
| PDP-Address | Served PDP/PDN Address |
| PDP-Address | Served PDP/PDN Address extension |
| Service-Specific-Info | Service Specific Info |
| SGSN-Address | Serving Node Address |
| SGSN-Address | S-GW Address used |
| SGW Change | S-GW Change |
| Start time | Start Time |
| Stop time | Stop Time |
| Time-First-Usage | Time of First Usage |
| Time–Last-Usage | Time of Last Usage |
| Time-Usage | Time Usage |
| 3GPP-User-Location-Info | User Location Information |
| User-CSG-Information | User CSG Information |
| 3GPP2-BSID | 3GPP2 User Location information |
| Low-Priority-Indicator | Low Priority Indicator |

NOTE:	The whole set of S/PGW-CDR parameters is described in the TS 32.298 [51].

End of changes