3GPP TSG-SA5 (Telecom Management)	S5-111475

SA5#76, 28 Feb - 4 Mar 2011; San Diego, USA	revision of S5-111101

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.298 | CR | 0172 |  rev | 3 |  Current version: | 10.3.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Adding CDR fields needed for Machine Type Communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | NIMTC-CH |  |  |  |  |  |  | Date:  |  | 04/03/2011 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change:  |  | Machine Type Communication introduces some features on EPC and information about these is valuable for statistics in the Billing Domain. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Adding a new field “Low priority indicator” in the CDRs from the EPC nodes. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | No indication available about Machine Type Communication. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 5.1.2.2.29A (new), 5.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  | X |  | O&M Specifications |  |  | 32.251, 32.299 |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

First change

##### 5.1.2.2.29A	Low Priority Indicator

This field indicates if the PDN connection has a low priority, i.e. for Machine Type Communication.

Next change

#### 5.2.2.2	PS domain CDRs

This subclause contains the abstract syntax definitions that are specific to the GPRS and EPC CDR types defined in TS 32.251 [11].

GPRSChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) gprsChargingDataTypes (2) asn1Module (0) version1 (0)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

CallDuration, CalledNumber, RecordType, CallingNumber, CallReferenceNumber, CellId, DefaultSMS-Handling, Diagnostics, Ext-GeographicalInformation, IMSI, IMEI, IPAddress, ISDN-AddressString, LCSCause, LCSClientExternalID, LCSClientIdentity, LCSClientInternalID, LCSClientType, LCS-Priority, LCSQoSInfo, LevelOfCAMELService, LocalSequenceNumber, LocationAreaAndCell, LocationAreaCode, LocationType, ManagementExtensions, MessageReference, MSISDN, NotificationToMSUser, PositioningData, RecordingEntity, ServiceKey, ServiceSpecificInfo, SMSResult, SmsTpDestinationNumber, SubscriptionID, TimeStamp, AddressString, MSTimeZone

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version1 (0)}

DefaultGPRS-Handling, RAIdentity

FROM MAP-MS-DataTypes { itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-MS-DataTypes (11) version6 (6)}

--

-- from TS 29.002 [214]

--

LocationMethod

FROM SS-DataTypes { itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Access (2) modules (3) ss-DataTypes (2) version7 (7)}

--

-- from TS 24.080 [209]

--

MBMS2G3GIndicator, FileRepairSupported, MBMSServiceType, MBMSUserServiceType, RequiredMBMSBearerCapabilities, MBMSSessionIdentity, TMGI, MBMSInformation

FROM MBMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) mbmsChargingDataTypes (8) asn1Module (0) version1 (0)}

Editor’s note:  consider moving the above 2 items also into the generic module in order to avoid again copying from external sources.

;

------------------------------------------------------------------------------

--

--  GPRS RECORDS

--

------------------------------------------------------------------------------

GPRSRecord	::= CHOICE

--

-- Record values 20, 22..27 are specific

-- Record values 76, 77, 86 are MBMS specific

-- Record values 78..79 are EPC specific

--

{

sgsnPDPRecord			[20] SGSNPDPRecord,

sgsnMMRecord			[22] SGSNMMRecord,

sgsnSMORecord			[23] SGSNSMORecord,

sgsnSMTRecord			[24] SGSNSMTRecord,

sgsnLCTRecord			[25] SGSNLCTRecord,

sgsnLCORecord			[26] SGSNLCORecord,

sgsnLCNRecord			[27] SGSNLCNRecord,

--

sgsnMBMSRecord			[76] SGSNMBMSRecord,

ggsnMBMSRecord			[77] GGSNMBMSRecord,

gwMBMSRecord			[86] GWMBMSRecord,

sGWRecord				[78] SGWRecord,

pGWRecord				[79] PGWRecord

}

SGWRecord 	::= SET

{

recordType					[0] RecordType,

servedIMSI					[3] IMSI,

s-GWAddress					[4] GSNAddress,

chargingID					[5] ChargingID,

servingNodeAddress			[6] SEQUENCE OF GSNAddress,

accessPointNameNI			[7] AccessPointNameNI OPTIONAL,

pdpPDNType					[8] PDPType OPTIONAL,

servedPDPPDNAddress			[9] PDPAddress OPTIONAL,

dynamicAddressFlag			[11] DynamicAddressFlag OPTIONAL,

listOfTrafficVolumes		[12] SEQUENCE OF ChangeOfCharCondition OPTIONAL,

recordOpeningTime			[13] TimeStamp,

duration					[14] CallDuration,

causeForRecClosing			[15] CauseForRecClosing,

diagnostics					[16] Diagnostics OPTIONAL,

recordSequenceNumber		[17] INTEGER OPTIONAL,

nodeID						[18] NodeID OPTIONAL,

recordExtensions			[19] ManagementExtensions OPTIONAL,

localSequenceNumber			[20] LocalSequenceNumber OPTIONAL,

apnSelectionMode			[21] APNSelectionMode OPTIONAL,

servedMSISDN				[22] MSISDN OPTIONAL,

chargingCharacteristics		[23] ChargingCharacteristics,

chChSelectionMode			[24] ChChSelectionMode OPTIONAL,

iMSsignalingContext			[25] NULL OPTIONAL,

servingNodePLMNIdentifier	[27] PLMN-Id OPTIONAL,

servedIMEISV				[29] IMEI OPTIONAL,

rATType						[30] RATType OPTIONAL,

mSTimeZone 					[31] MSTimeZone OPTIONAL,

userLocationInformation		[32] OCTET STRING OPTIONAL,

sGWChange					[34] SGWChange OPTIONAL,

servingNodeType				[35] SEQUENCE OF ServingNodeType,

p-GWAddressUsed				[36] GSNAddress OPTIONAL,

p-GWPLMNIdentifier			[37] PLMN-Id OPTIONAL,

startTime					[38] TimeStamp OPTIONAL,

stopTime					[39] TimeStamp OPTIONAL,

pDNConnectionID				[40] ChargingID OPTIONAL,

iMSIunauthenticatedFlag 	[41] NULL OPTIONAL,

userCSGInformation			[42] UserCSGInformation OPTIONAL,

servedPDPPDNAddressExt 		[43] PDPAddress OPTIONAL,

lowPriorityIndicator		[44] NULL OPTIONAL

}

PGWRecord 	::= SET

{

recordType						[0] RecordType,

servedIMSI						[3] IMSI,

p-GWAddress						[4] GSNAddress,

chargingID						[5] ChargingID,

servingNodeAddress				[6] SEQUENCE OF GSNAddress,

accessPointNameNI				[7] AccessPointNameNI OPTIONAL,

pdpPDNType						[8] PDPType OPTIONAL,

servedPDPPDNAddress				[9] PDPAddress OPTIONAL,

dynamicAddressFlag				[11] DynamicAddressFlag OPTIONAL,

recordOpeningTime				[13] TimeStamp,

duration						[14] CallDuration,

causeForRecClosing				[15] CauseForRecClosing,

diagnostics						[16] Diagnostics OPTIONAL,

recordSequenceNumber			[17] INTEGER OPTIONAL,

nodeID							[18] NodeID OPTIONAL,

recordExtensions				[19] ManagementExtensions OPTIONAL,

localSequenceNumber				[20] LocalSequenceNumber OPTIONAL,

apnSelectionMode				[21] APNSelectionMode OPTIONAL,

servedMSISDN					[22] MSISDN OPTIONAL,

chargingCharacteristics			[23] ChargingCharacteristics,

chChSelectionMode				[24] ChChSelectionMode OPTIONAL,

iMSsignalingContext				[25] NULL OPTIONAL,

externalChargingID				[26] OCTET STRING OPTIONAL,

servinggNodePLMNIdentifier		[27] PLMN-Id OPTIONAL,

pSFurnishChargingInformation	[28] PSFurnishChargingInformation OPTIONAL,

servedIMEISV					[29] IMEI OPTIONAL,

rATType							[30] RATType OPTIONAL,

mSTimeZone 						[31] MSTimeZone OPTIONAL,

userLocationInformation			[32] OCTET STRING OPTIONAL,

cAMELChargingInformation		[33] OCTET STRING OPTIONAL,

listOfServiceData				[34] SEQUENCE OF ChangeOfServiceCondition OPTIONAL,

servingNodeType					[35] SEQUENCE OF ServingNodeType,

servedMNNAI						[36] SubscriptionID OPTIONAL,

p-GWPLMNIdentifier				[37] PLMN-Id OPTIONAL,

startTime						[38] TimeStamp OPTIONAL,

stopTime						[39] TimeStamp OPTIONAL,

served3gpp2MEID					[40] OCTET STRING OPTIONAL,

pDNConnectionID					[41] ChargingID OPTIONAL,

iMSIunauthenticatedFlag 		[42] NULL OPTIONAL,

userCSGInformation				[43] UserCSGInformation OPTIONAL,

3GPP2UserLocationInformation	[44] OCTET STRING OPTIONAL,

servedPDPPDNAddressExt 			[45] PDPAddress OPTIONAL,

lowPriorityIndicator			[46] NULL OPTIONAL

}

SGSNMMRecord	::= SET

{

recordType					[0] RecordType,

servedIMSI					[1] IMSI,

servedIMEI					[2] IMEI OPTIONAL,

sgsnAddress					[3] GSNAddress OPTIONAL,

msNetworkCapability			[4] MSNetworkCapability OPTIONAL,

routingArea					[5] RoutingAreaCode OPTIONAL,

locationAreaCode			[6] LocationAreaCode OPTIONAL,

cellIdentifier				[7] CellId OPTIONAL,

changeLocation				[8] SEQUENCE OF ChangeLocation OPTIONAL,

recordOpeningTime			[9] TimeStamp,

duration					[10] CallDuration OPTIONAL,

sgsnChange					[11] SGSNChange OPTIONAL,

causeForRecClosing			[12] CauseForRecClosing,

diagnostics					[13] Diagnostics OPTIONAL,

recordSequenceNumber		[14] INTEGER OPTIONAL,

nodeID						[15] NodeID OPTIONAL,

recordExtensions			[16] ManagementExtensions OPTIONAL,

localSequenceNumber			[17] LocalSequenceNumber OPTIONAL,

servedMSISDN				[18] MSISDN OPTIONAL,

chargingCharacteristics		[19] ChargingCharacteristics,

cAMELInformationMM 			[20] CAMELInformationMM OPTIONAL,

rATType						[21] RATType OPTIONAL,

chChSelectionMode			[22] ChChSelectionMode OPTIONAL,

cellPLMNId					[23] PLMN-Id OPTIONAL

}

SGSNPDPRecord 	::= SET

{

recordType					[0] RecordType,

networkInitiation			[1] NetworkInitiatedPDPContext OPTIONAL,

servedIMSI					[3] IMSI,

servedIMEI					[4] IMEI OPTIONAL,

sgsnAddress					[5] GSNAddress OPTIONAL,

msNetworkCapability			[6] MSNetworkCapability OPTIONAL,

routingArea					[7] RoutingAreaCode OPTIONAL,

locationAreaCode			[8] LocationAreaCode OPTIONAL,

cellIdentifier				[9] CellId OPTIONAL,

chargingID					[10] ChargingID,

ggsnAddressUsed				[11] GSNAddress,

accessPointNameNI			[12] AccessPointNameNI OPTIONAL,

pdpType						[13] PDPType OPTIONAL,

servedPDPAddress			[14] PDPAddress OPTIONAL,

listOfTrafficVolumes		[15] SEQUENCE OF ChangeOfCharCondition OPTIONAL,

recordOpeningTime			[16] TimeStamp,

duration					[17] CallDuration,

sgsnChange					[18] SGSNChange OPTIONAL,

causeForRecClosing			[19] CauseForRecClosing,

diagnostics					[20] Diagnostics OPTIONAL,

recordSequenceNumber		[21] INTEGER OPTIONAL,

nodeID						[22] NodeID OPTIONAL,

recordExtensions			[23] ManagementExtensions OPTIONAL,

localSequenceNumber			[24] LocalSequenceNumber OPTIONAL,

apnSelectionMode			[25] APNSelectionMode OPTIONAL,

accessPointNameOI			[26] AccessPointNameOI OPTIONAL,

servedMSISDN				[27] MSISDN OPTIONAL,

chargingCharacteristics		[28] ChargingCharacteristics,

rATType						[29] RATType OPTIONAL,

cAMELInformationPDP  		[30] CAMELInformationPDP OPTIONAL,

rNCUnsentDownlinkVolume		[31] DataVolumeGPRS OPTIONAL,

chChSelectionMode			[32] ChChSelectionMode OPTIONAL,

dynamicAddressFlag			[33] DynamicAddressFlag OPTIONAL,

iMSIunauthenticatedFlag 	[34] NULL OPTIONAL,

userCSGInformation			[35] UserCSGInformation OPTIONAL,

servedPDPPDNAddressExt 		[36] PDPAddress OPTIONAL,

lowPriorityIndicator		[37] NULL OPTIONAL

}

SGSNSMORecord	::= SET

{

recordType					[0] RecordType,

servedIMSI					[1] IMSI,

servedIMEI					[2] IMEI OPTIONAL,

servedMSISDN				[3] MSISDN OPTIONAL,

msNetworkCapability			[4] MSNetworkCapability OPTIONAL,

serviceCentre				[5] AddressString OPTIONAL,

recordingEntity				[6] RecordingEntity OPTIONAL,

locationArea				[7] LocationAreaCode OPTIONAL,

routingArea					[8] RoutingAreaCode OPTIONAL,

cellIdentifier				[9] CellId OPTIONAL,

messageReference			[10] MessageReference,

eventTimeStamp				[11] TimeStamp,

smsResult					[12] SMSResult OPTIONAL,

recordExtensions			[13] ManagementExtensions OPTIONAL,

nodeID						[14] NodeID OPTIONAL,

localSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

chargingCharacteristics		[16] ChargingCharacteristics,

rATType						[17] RATType OPTIONAL,

destinationNumber			[18] SmsTpDestinationNumber OPTIONAL,

cAMELInformationSMS			[19] CAMELInformationSMS OPTIONAL,

chChSelectionMode			[20] ChChSelectionMode OPTIONAL

}

SGSNSMTRecord	::= SET

{

recordType				[0] RecordType,

servedIMSI				[1] IMSI,

servedIMEI				[2] IMEI OPTIONAL,

servedMSISDN			[3] MSISDN OPTIONAL,

msNetworkCapability		[4] MSNetworkCapability OPTIONAL,

serviceCentre			[5] AddressString OPTIONAL,

recordingEntity			[6] RecordingEntity OPTIONAL,

locationArea			[7] LocationAreaCode OPTIONAL,

routingArea				[8] RoutingAreaCode OPTIONAL,

cellIdentifier			[9] CellId OPTIONAL,

eventTimeStamp			[10] TimeStamp,

smsResult				[11] SMSResult OPTIONAL,

recordExtensions		[12] ManagementExtensions OPTIONAL,

nodeID					[13] NodeID OPTIONAL,

localSequenceNumber		[14] LocalSequenceNumber OPTIONAL,

chargingCharacteristics	[15] ChargingCharacteristics,

rATType					[16] RATType OPTIONAL,

chChSelectionMode		[17] ChChSelectionMode OPTIONAL,

cAMELInformationSMS		[18] CAMELInformationSMS OPTIONAL,

originatingAddress      [19] AddressString OPTIONAL

}

SGSNMTLCSRecord			::= SET

{

recordType				[0] RecordType,

recordingEntity			[1] RecordingEntity,

lcsClientType			[2] LCSClientType,

lcsClientIdentity		[3] LCSClientIdentity,

servedIMSI				[4] IMSI,

servedMSISDN			[5] MSISDN OPTIONAL,

sgsnAddress				[6] GSNAddress OPTIONAL,

locationType			[7] LocationType,

lcsQos					[8] LCSQoSInfo OPTIONAL,

lcsPriority				[9] LCS-Priority OPTIONAL,

mlcNumber				[10] ISDN-AddressString,

eventTimeStamp			[11] TimeStamp,

measurementDuration		[12] CallDuration OPTIONAL,

notificationToMSUser	[13] NotificationToMSUser OPTIONAL,

privacyOverride			[14] NULL OPTIONAL,

location				[15] LocationAreaAndCell OPTIONAL,

routingArea				[16] RoutingAreaCode OPTIONAL,

locationEstimate		[17] Ext-GeographicalInformation OPTIONAL,

positioningData			[18] PositioningData OPTIONAL,

lcsCause				[19] LCSCause OPTIONAL,

diagnostics				[20] Diagnostics OPTIONAL,

nodeID					[21] NodeID OPTIONAL,

localSequenceNumber		[22] LocalSequenceNumber OPTIONAL,

chargingCharacteristics	[23] ChargingCharacteristics,

chChSelectionMode		[24] ChChSelectionMode OPTIONAL,

rATType					[25] RATType OPTIONAL,

recordExtensions		[26] ManagementExtensions OPTIONAL,

causeForRecClosing		[27] CauseForRecClosing

}

SGSNMOLCSRecord			::= SET

{

recordType				[0] RecordType,

recordingEntity			[1] RecordingEntity,

lcsClientType			[2] LCSClientType OPTIONAL,

lcsClientIdentity		[3] LCSClientIdentity OPTIONAL,

servedIMSI				[4] IMSI,

servedMSISDN			[5] MSISDN OPTIONAL,

sgsnAddress				[6] GSNAddress OPTIONAL,

locationMethod			[7] LocationMethod,

lcsQos					[8] LCSQoSInfo OPTIONAL,

lcsPriority				[9] LCS-Priority OPTIONAL,

mlcNumber				[10] ISDN-AddressString OPTIONAL,

eventTimeStamp			[11] TimeStamp,

measurementDuration		[12] CallDuration OPTIONAL,

location				[13] LocationAreaAndCell OPTIONAL,

routingArea				[14] RoutingAreaCode OPTIONAL,

locationEstimate		[15] Ext-GeographicalInformation OPTIONAL,

positioningData			[16] PositioningData OPTIONAL,

lcsCause				[17] LCSCause OPTIONAL,

diagnostics				[18] Diagnostics OPTIONAL,

nodeID					[19] NodeID OPTIONAL,

localSequenceNumber	    [20] LocalSequenceNumber OPTIONAL,

chargingCharacteristics	[21] ChargingCharacteristics,

chChSelectionMode		[22] ChChSelectionMode OPTIONAL,

rATType					[23] RATType OPTIONAL,

recordExtensions		[24] ManagementExtensions OPTIONAL,

causeForRecClosing		[25] CauseForRecClosing

}

SGSNNILCSRecord			::= SET

{

recordType				[0] RecordType,

recordingEntity			[1] RecordingEntity,

lcsClientType			[2] LCSClientType OPTIONAL,

lcsClientIdentity		[3] LCSClientIdentity OPTIONAL,

servedIMSI				[4] IMSI OPTIONAL,

servedMSISDN			[5] MSISDN OPTIONAL,

sgsnAddress				[6] GSNAddress OPTIONAL,

servedIMEI				[7] IMEI OPTIONAL,

lcsQos					[8] LCSQoSInfo OPTIONAL,

lcsPriority				[9] LCS-Priority OPTIONAL,

mlcNumber				[10] ISDN-AddressString OPTIONAL,

eventTimeStamp			[11] TimeStamp,

measurementDuration		[12] CallDuration OPTIONAL,

location				[13] LocationAreaAndCell OPTIONAL,

routingArea			    [14] RoutingAreaCode OPTIONAL,

locationEstimate		[15] Ext-GeographicalInformation OPTIONAL,

positioningData			[16] PositioningData OPTIONAL,

lcsCause				[17] LCSCause OPTIONAL,

diagnostics				[18] Diagnostics OPTIONAL,

nodeID					[19] NodeID OPTIONAL,

localSequenceNumber	    [20] LocalSequenceNumber OPTIONAL,

chargingCharacteristics	[21] ChargingCharacteristics,

chChSelectionMode		[22] ChChSelectionMode OPTIONAL,

rATType					[23] RATType OPTIONAL,

recordExtensions		[24] ManagementExtensions OPTIONAL,

causeForRecClosing		[25] CauseForRecClosing

}

SGSNMBMSRecord 	::= SET

{

recordType					[0] RecordType,

ggsnAddress					[1] GSNAddress,

chargingID					[2] ChargingID,

listofRAs					[3] SEQUENCE OF RAIdentity OPTIONAL,

accessPointNameNI			[4] AccessPointNameNI OPTIONAL,

servedPDPAddress			[5] PDPAddress OPTIONAL,

listOfTrafficVolumes		[6] SEQUENCE OF ChangeOfMBMSCondition OPTIONAL,

recordOpeningTime			[7] TimeStamp,

duration					[8] CallDuration,

causeForRecClosing			[9] CauseForRecClosing,

diagnostics					[10] Diagnostics OPTIONAL,

recordSequenceNumber		[11] INTEGER OPTIONAL,

nodeID						[12] NodeID OPTIONAL,

recordExtensions			[13] ManagementExtensions OPTIONAL,

localSequenceNumber			[14] LocalSequenceNumber OPTIONAL,

sgsnPLMNIdentifier			[15] PLMN-Id OPTIONAL,

numberofReceivingUE			[16] Integer OPTIONAL,

mbmsInformation				[17] MBMSInformation OPTIONAL

}

GGSNMBMSRecord 	::= SET

{

recordType					[0] RecordType,

ggsnAddress					[1] GSNAddress,

chargingID					[2] ChargingID,

listofDownstreamNodes		[3] SEQUENCE OF GSNAddress,

accessPointNameNI			[4] AccessPointNameNI OPTIONAL,

servedPDPAddress			[5] PDPAddress OPTIONAL,

listOfTrafficVolumes		[6] SEQUENCE OF ChangeOfMBMSCondition OPTIONAL,

recordOpeningTime			[7] TimeStamp,

duration					[8] CallDuration,

causeForRecClosing			[9] CauseForRecClosing,

diagnostics					[10] Diagnostics OPTIONAL,

recordSequenceNumber		[11] INTEGER OPTIONAL,

nodeID						[12] NodeID OPTIONAL,

recordExtensions			[13] ManagementExtensions OPTIONAL,

localSequenceNumber			[14] LocalSequenceNumber OPTIONAL,

mbmsInformation				[15] MBMSInformation OPTIONAL

}

GWMBMSRecord 	::= SET

{

recordType					[0] RecordType,

mbmsGWAddress				[1] GSNAddress,

chargingID					[2] ChargingID,

listofDownstreamNodes		[3] SEQUENCE OF GSNAddress,

accessPointNameNI			[4] AccessPointNameNI OPTIONAL,

pdpPDNType					[5] PDPType OPTIONAL,

servedPDPPDNAddress			[6] PDPAddress OPTIONAL,

listOfTrafficVolumes		[7] SEQUENCE OF ChangeOfMBMSCondition OPTIONAL,

recordOpeningTime			[8] TimeStamp,

duration					[9] CallDuration,

causeForRecClosing			[10] CauseForRecClosing,

diagnostics					[11] Diagnostics OPTIONAL,

recordSequenceNumber		[12] INTEGER OPTIONAL,

nodeID						[13] NodeID OPTIONAL,

recordExtensions			[14] ManagementExtensions OPTIONAL,

localSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

mbmsInformation				[16] MBMSInformation OPTIONAL,

commonTeid   				[17] CTEID OPTIONAL,

iPMulticastSourceAddress	[18] PDPAddress OPTIONAL

}

------------------------------------------------------------------------------

--

--  PS DATA TYPES

--

------------------------------------------------------------------------------

AccessPointNameNI	::= IA5String (SIZE(1..63))

--

-- Network Identifier part of APN in  dot representation.

-- For example, if the complete APN is 'apn1a.apn1b.apn1c.mnc022.mcc111.gprs'

-- NI is 'apn1a.apn1b.apn1c' and is presented in this form in the CDR..

--

AccessPointNameOI	::= IA5String (SIZE(1..37))

--

-- Operator Identifier part of APN in dot representation.

-- In the 'apn1a.apn1b.apn1c.mnc022.mcc111.gprs' example, the OI portion is 'mnc022.mcc111.gprs'

-- and is presented in this form in the CDR.

--

AFChargingIdentifier ::= OCTECT STRING

--

-- see AF-Charging-Identifier AVP as defined in TS 29.214[221]

--

AFRecordInformation	::= SEQUENCE

{

aFChargingIdentifier		[1] AFChargingIdentifier,

flows						[2] Flows OPTIONAL

}

APNSelectionMode::= ENUMERATED

--

-- See Information Elements TS 29.060 [215], TS 29.274 [223] or TS 29.275 [224]

--

{

mSorNetworkProvidedSubscriptionVerified				(0),

mSProvidedSubscriptionNotVerified					(1),

networkProvidedSubscriptionNotVerified				(2)

}

CAMELAccessPointNameNI	::= AccessPointNameNI

CAMELAccessPointNameOI	::= AccessPointNameOI

CAMELInformationMM		::= SET

{

sCFAddress						[1] SCFAddress OPTIONAL,

serviceKey						[2]	ServiceKey OPTIONAL,

defaultTransactionHandling		[3]	DefaultGPRS-Handling OPTIONAL,

numberOfDPEncountered  			[4] NumberOfDPEncountered OPTIONAL,

levelOfCAMELService				[5] LevelOfCAMELService OPTIONAL,

freeFormatData					[6] FreeFormatData OPTIONAL,

fFDAppendIndicator				[7]	FFDAppendIndicator OPTIONAL

}

CAMELInformationPDP	::= SET

{

sCFAddress						[1] SCFAddress OPTIONAL,

serviceKey						[2]	ServiceKey OPTIONAL,

defaultTransactionHandling		[3]	DefaultGPRS-Handling OPTIONAL,

cAMELAccessPointNameNI			[4] CAMELAccessPointNameNI OPTIONAL,

cAMELAccessPointNameOI			[5] CAMELAccessPointNameOI OPTIONAL,

numberOfDPEncountered			[6] NumberOfDPEncountered OPTIONAL,

levelOfCAMELService				[7] LevelOfCAMELService OPTIONAL,

freeFormatData					[8]	FreeFormatData OPTIONAL,

fFDAppendIndicator				[9]	FFDAppendIndicator OPTIONAL

}

CAMELInformationSMS		::= SET

{

sCFAddress								[1] SCFAddress OPTIONAL,

serviceKey								[2]	ServiceKey OPTIONAL,

defaultSMSHandling						[3]	DefaultSMS-Handling OPTIONAL,

cAMELCallingPartyNumber					[4] CallingNumber OPTIONAL,

cAMELDestinationSubscriberNumber		[5] SmsTpDestinationNumber OPTIONAL,

cAMELSMSCAddress						[6] AddressString OPTIONAL,

freeFormatData							[7]	FreeFormatData OPTIONAL,

smsReferenceNumber						[8] CallReferenceNumber	OPTIONAL

}

CauseForRecClosing	::= INTEGER

--

-- In PGW-CDR and SGW-CDR the value servingNodeChange is used for partial record

-- generation due to Serving Node Address list Overflow

-- In SGSN servingNodeChange indicates the SGSN change

--

-- LCS related causes belong to the MAP error causes acc. TS 29.002 [214]

--

-- cause codes 0 to 15 are defined 'CauseForTerm' (cause for termination)

--

{

normalRelease					(0),

abnormalRelease					(4),

cAMELInitCallRelease			(5),

volumeLimit						(16),

timeLimit						(17),

servingNodeChange				(18),

maxChangeCond					(19),

managementIntervention			(20),

intraSGSNIntersystemChange		(21),

rATChange						(22),

mSTimeZoneChange				(23),

sGSNPLMNIDChange 				(24),

unauthorizedRequestingNetwork	(52),

unauthorizedLCSClient			(53),

positionMethodFailure			(54),

unknownOrUnreachableLCSClient	(58),

listofDownstreamNodeChange		(59)

}

ChangeCondition	::= ENUMERATED

{

qoSChange								(0),

tariffTime								(1),

recordClosure							(2),

cGI-SAICHange							(6), 	-- bearer modification. “CGI-SAI Change”

rAIChange								(7), 	-- bearer modification. “RAI Change”

dT-Establishment						(8),

dT-Removal								(9),

eCGIChange								(10), 	-- bearer modification. “ECGI Change”

tAIChange								(11), 	-- bearer modification. “TAI Change”

userLocationChange						(12),	-- bearer modification. “User Location Change”

}

ChangeOfCharCondition	::= SEQUENCE

--

-- qosRequested and qosNegotiated are used in S-CDR only

-- ePCQoSInformation used in SGW-CDR only

--

{

qosRequested				[1] QoSInformation OPTIONAL,

qosNegotiated				[2] QoSInformation OPTIONAL,

dataVolumeGPRSUplink		[3] DataVolumeGPRS OPTIONAL,

dataVolumeGPRSDownlink		[4] DataVolumeGPRS OPTIONAL,

changeCondition				[5] ChangeCondition,

changeTime					[6] TimeStamp,

userLocationInformation		[8] OCTET STRING OPTIONAL,

ePCQoSInformation			[9] EPCQoSInformation OPTIONAL

}

ChangeOfMBMSCondition	::= SEQUENCE

--

-- Used in MBMS record

--

{

qosRequested				[1] QoSInformation OPTIONAL,

qosNegotiated				[2] QoSInformation OPTIONAL,

dataVolumeMBMSUplink		[3] DataVolumeMBMS OPTIONAL,

dataVolumeMBMSDownlink		[4] DataVolumeMBMS,

changeCondition				[5] ChangeCondition,

changeTime					[6] TimeStamp,

failureHandlingContinue		[7] FailureHandlingContinue OPTIONAL

}

ChangeOfServiceCondition	::= SEQUENCE

--

-- Used for Flow based Charging service data container

--

{

ratingGroup 					[1] RatingGroupId,

chargingRuleBaseName			[2] ChargingRuleBaseName OPTIONAL,

resultCode						[3] ResultCode OPTIONAL,

localSequenceNumber				[4] LocalSequenceNumber OPTIONAL,

timeOfFirstUsage				[5] TimeStamp OPTIONAL,

timeOfLastUsage					[6] TimeStamp OPTIONAL,

timeUsage 						[7] CallDuration OPTIONAL,

serviceConditionChange			[8] ServiceConditionChange,

qoSInformationNeg				[9] EPCQoSInformation OPTIONAL,

servingNodeAddress 				[10] GSNAddress OPTIONAL,

datavolumeFBCUplink				[12] DataVolumeGPRS OPTIONAL,

datavolumeFBCDownlink			[13] DataVolumeGPRS OPTIONAL,

timeOfReport					[14] TimeStamp,

failureHandlingContinue			[16] FailureHandlingContinue OPTIONAL,

serviceIdentifier				[17] ServiceIdentifier OPTIONAL,

pSFurnishChargingInformation	[18] PSFurnishChargingInformation OPTIONAL,

aFRecordInformation				[19] SEQUENCE OF AFRecordInformation OPTIONAL,

userLocationInformation			[20] OCTER STRING OPTIONAL,

eventBasedChargingInformation	[21] EventBasedChargingInformation OPTIONAL,

timeQuotaMechanism				[22] TimeQuotaMechanism OPTIONAL,

serviceSpecificInfo				[23] SEQUENCE OF ServiceSpecificInfo OPTIONAL,

3GPP2UserLocationInformation	[24] OCTET STRING OPTIONAL

}

ChangeLocation	::= SEQUENCE

--

-- used in SGSNMMRecord only

--

{

locationAreaCode		[0] LocationAreaCode,

routingAreaCode			[1] RoutingAreaCode,

cellId					[2] CellId OPTIONAL,

changeTime				[3] TimeStamp,

mCC-MNC					[4] PLMN-Id OPTIONAL

}

ChargingCharacteristics	::= OCTET STRING (SIZE(2))

ChargingID	::= INTEGER (0..4294967295)

--

-- Generated in P-GW, part of IP CAN bearer

-- 0..4294967295 is equivalent to 0..2**32-1

--

ChargingRuleBaseName ::= IA5String (SIZE(1..16))

--

-- identifier for the group of charging rules

-- see Charging-Rule-Base-Name AVP as desined in TS 29.212 [220]

--

ChChSelectionMode		::= ENUMERATED

{

servingNodeSupplied			(0),	-- For S-GW/P-GW

subscriptionSpecific		(1),	-- For SGSN only

aPNSpecific					(2),	-- For SGSN only

homeDefault					(3),	-- For SGSN, S-GW and P-GW

roamingDefault				(4),	-- For SGSN, S-GW and P-GW

visitingDefault				(5)		-- For SGSN, S-GW and P-GW

}

CSGAccessMode ::= ENUMERATED

{

ClosedMode  (0),

HybridMode  (1)

}

CSGId	::= OCTET STRING (SIZE(4))

--

-- Defined in 23.003[200]. Coded according to TS 29.060[215] for GTP, and in TS 29.274 [223] for 	-- eGTP.

-- 24.008 [208]

--

CTEID	::=	OCTET STRING (SIZE(4))

--

-- Defined in 32.251[11] for MBMS-GW-CDR. Common Tunnel Endpoint Identifier of MBMS GW for user --

-- plane, defined in TS23.246 [207].

--

DataVolumeGPRS	::= INTEGER

--

-- The volume of data transferred in octets.

--

DynamicAddressFlag	::= BOOLEAN

EPCQoSInformation	::= SEQUENCE

--

-- See TS 29.212 [220] for more information

--

{

qCI						[1] INTEGER,

maxRequestedBandwithUL	[2] INTEGER OPTIONAL,

maxRequestedBandwithDL	[3] INTEGER OPTIONAL,

guaranteedBitrateUL		[4] INTEGER OPTIONAL,

guaranteedBitrateDL		[5] INTEGER OPTIONAL,

aRP						[6] INTEGER OPTIONAL

}

ETSIAddress	::= AddressString

--

-- First octet for nature of address, and numbering plan indicator (3 for X.121)

-- Other octets TBCD, See TS 29.002 [214]

--

EventBasedChargingInformation  ::= SEQUENCE

{

numberOfEvents		[1] INTEGER,

eventTimeStamps  	[2] SEQUENCE OF TimeStamp OPTIONAL

}

FailureHandlingContinue  ::= BOOLEAN

--

-- This parameter is included when the failure handling procedure has been executed and new

-- containers are opened. This parameter shall be included in the first and subsequent

-- containers opened after the failure handling execution.

--

FFDAppendIndicator	::= BOOLEAN

Flows  ::= 	SEQUENCE

--

-- See Flows AVP as defined in TS 29.214 [221]

--

{

mediaComponentNumber	[1] INTEGER,

flowNumber  			[2] SEQUENCE OF INTEGER OPTIONAL

}

FreeFormatData	::=	OCTET STRING (SIZE(1..160))

--

-- Free formatted data as sent in the FurnishChargingInformationGPRS

-- see TS 29.078 [217]

--

GSNAddress	::= IPAddress

MSNetworkCapability	::= OCTET STRING (SIZE(1..8))

--

-- see TS 24.008 [208]

--

NetworkInitiatedPDPContext	::= BOOLEAN

--

-- Set to true if PDP context was initiated from network side

--

NodeID	::= IA5String (SIZE(1..20))

NumberOfDPEncountered  ::= INTEGER

PDPAddress	::= CHOICE

{

iPAddress				[0] IPAddress,

eTSIAddress			   [1] ETSIAddress

}

PDPType		::= OCTET STRING (SIZE(2))

--

-- OCTET 1: PDP Type Organization

-- OCTET 2: PDP/PDN Type Number

-- See TS 29.060 [215] for GTP, TS 29.274 [223] clause 8.14 for eGTP and TS 29.275 [224] for PMIP

--

PLMN-Id		::= OCTET STRING (SIZE (3))

--

--	This is a 1:1 copy from the Routing Area Identity (RAI) IE specified in TS 29.060 [215]

--  as follows:

--	OCTET 1 of PLMN-Id = OCTET 2 of RAI

--	OCTET 2 of PLMN-Id = OCTET 3 of RAI

--	OCTET 3 of PLMN-Id = OCTET 4 of RAI

--

PSFurnishChargingInformation  ::= SEQUENCE

{

pSFreeFormatData		[1] FreeFormatData,

pSFFDAppendIndicator  	[2] FFDAppendIndicator OPTIONAL

}

QoSInformation	::= OCTET STRING (SIZE (4..255))

--

-- This  octet string

-- is a 1:1 copy of the contents (i.e. starting with octet 5) of the "Bearer Quality of

-- Service" information element specified in TS 29.274 [223].

--

RatingGroupId 	::= INTEGER

--

-- IP service flow identity (DCCA), range of 4 byte (0...4294967259)

-- see Rating-Group AVP as used in TS 32.299 [50]

--

RATType		::= INTEGER (0..255)

--

-- Ihis integer is 1:1 copy of the RAT type value as defined in TS 29.060 [215] for GTP,

-- TS 29.274 [223] for eGTP and TS 29.275 [224] for PMIP.

--

ResultCode 	::= INTEGER

--

-- charging protocol return value, range of 4 byte (0...4294967259)

-- see Result-Code AVP as used in 32.299 [40]

--

RoutingAreaCode	::= OCTET STRING (SIZE(1))

--

-- See TS 24.008 [208]

--

ServiceConditionChange	::= BIT STRING

{

qoSChange 								 (0),	-- bearer modification

sGSNChange 								 (1), 	-- bearer modification

sGSNPLMNIDChange 						 (2), 	-- bearer modification

tariffTimeSwitch 						 (3), 	-- tariff time change

pDPContextRelease 						 (4), 	-- bearer release

rATChange 								 (5),	-- bearer modification

serviceIdledOut 						 (6),	-- IP flow idle out, DCCA QHT expiry

reserved 								 (7),	-- old: QCTexpiry is no report event

configurationChange 					 (8),	-- configuration change

serviceStop 							 (9), 	-- IP flow termination.From ”Service Stop” in

-- Change-Condition” AVP

dCCATimeThresholdReached 				(10),	-- DCCA quota reauthorization

dCCAVolumeThresholdReached 				(11), 	-- DCCA quota reauthorization

dCCAServiceSpecificUnitThresholdReached	(12), 	-- DCCA quota reauthorization

dCCATimeExhausted 						(13), 	-- DCCA quota reauthorization

dCCAVolumeExhausted 					(14), 	-- DCCA quota reauthorization

dCCAValidityTimeout 					(15), 	-- DCCA quota validity time (QVT expiry)

reserved1								(16),	-- reserved due to no use case,

-- old: return Requested is covered by (17),(18)

dCCAReauthorisationRequest 				(17), 	-- DCCA quota reauthorization request by OCS

dCCAContinueOngoingSession 				(18), 	-- DCCA failure handling (CCFH),

-- continue IP flow

dCCARetryAndTerminateOngoingSession		(19), 	-- DCCA failure handling (CCFH),

-- terminate IP flow after DCCA retry

dCCATerminateOngoingSession 			(20), 	-- DCCA failure handling,

-- terminate IP flow

cGI-SAIChange							(21), 	-- bearer modification. “CGI-SAI Change”

rAIChange								(22), 	-- bearer modification. “RAI Change”

dCCAServiceSpecificUnitExhausted		(23), 	-- DCCA quota reauthorization

recordClosure							(24),	-- PGW-CDR closure

timeLimit								(25),	-- intermediate recording. From “Service Data 													--	Time Limit” Change-Condition AVP value

volumeLimit								(26),	-- intermediate recording.From “Service Data 														--	Volume Limit” Change-Condition AVP value

serviceSpecificUnitLimit				(27),	-- intermediate recording

envelopeClosure 						(28),

eCGIChange								(29), 	-- bearer modification. “ECGI Change”

tAIChange								(30), 	-- bearer modification. “TAI Change”

userLocationChange						(31)	-- bearer modification. “User Location Change”

}

--

-- Trigger and cause values for IP flow level recording are defined for support of independent

-- online and offline charging and also for tight interworking between online and offline charging.

-- Unused bits will always be zero.

-- Some of the values are non-exclusive (e.g. bearer modification reasons).

--

SCFAddress	::= AddressString

--

-- See TS 29.002 [214]

--

ServiceIdentifier	::= INTEGER (0..4294967295)

--

-- The service identifier is used to identify the service or the service component

-- the service data flow relates to. See Service-Identifier AVP as defined in TS 29.212 [220]

--

ServingNodeType	::= ENUMERATED

{

sGSN		(0),

pMIPSGW		(1),

gTPSGW		(2),

ePDG		(3),

hSGW		(4),

mME			(5)

}

SGSNChange	::= BOOLEAN

--

-- present if first record after inter SGSN routing area update in new SGSN

--

SGWChange	::= BOOLEAN

--

-- present if first record after inter S-GW change

--

TimeQuotaMechanism	::= SEQUENCE

{

timeQuotaType					[1] TimeQuotaType,

baseTimeInterval				[2] Integer

}

TimeQuotaType	::= ENUMERATED

{

dISCRETETIMEPERIOD			(0),

cONTINUOUSTIMEPERIOD		(1)

}

UserCSGInformation	::= SEQUENCE

{

cSGId					[0] CSGId,

cSGAccessMode			[1] CSGAccessMode,

cSGMembershipIndication [2] NULL OPTIONAL

}

END

End of changes