3GPP TSG-SA5 (Telecom Management)	S5-110439

SA5#75, 24 - 28 Jan 2011; Sorrento, ITALY	revision of S5-110202

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.551 | CR | 0001 |  rev | 1 |  Current version: | 10.0.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Modify the energy saving compensation related use cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | OAM-ES |  |  |  |  |  |  | Date:  |  | 28/01/2011 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change:  |  | In the capacity limited scenario, in off-peak traffic situation, some NEs/Cells can be transferred to energySaving state and other related NEs/Cells can be transferred to compensatingforEnergySaving state; when it turns into peak traffic situation, the NEs/Cells in energySaving state shall be transferred to notEnergySaving state, and the related NEs/Cells in compensatingforEnergySaving state shall also be transferred to notEnergySaving state. But in the current use cases of energy saving deactivation, the state of the NEs/Cells in compensatingforEnergySaving state are not transferred. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | The use cases about evergySaving deactivaton are updated. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | The use cases do not reflect the real situation. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 6.4.2, 6.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

First change

### 6.4.2	Deactivatation of energy saving on selected network elements (Centralized ES on NM layer)

| Use Case Stage | Evolution / Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal (*) | Deactivating energy saving for a selected part of the network |  |
| Actors and Roles (*) | IRPManager as user |  |
| Telecom resources | Network elements (NEs) and their OSS. |  |
| Assumptions | Centralized ES on NM layer is performed. The network operator has decided to activate energy saving on selected network elements in a part of the network (network elements, e.g., base stations). This requires to activate energy saving on some network elements and to activate energy saving compensation on others. The IRPManager is continuously monitoring load on NEs. An interference control function is available to support the coverage adjustment process among NEs in compensatingForEnergySaving and EnergySaving state. |  |
| Pre conditions | The affected network elements are in ESaving state or in ES-Compensate state. Network elements (base stations) are not in a faulty state. |  |
| Begins when | The IRPManager decides to deactivate energySaving for selected network elements in a part of the network based on network load. |  |
| Step 1 (*) (M) | The IPRManager makes a decision on which NEs should remain in energySaving state or compensatingforEnergySaving state, or enter notEnergySaving state based on network load, geographic positions and maximum coverage of base stations. |  |
| Step 2 (*) (M) | Based on the output of step1, the IRPManager initiates energy saving deactivation and energy saving compensation deactivation on the NEs selected for the respective state transition. |  |
| Step 3 (*) (M) | After the completion of the energy saving deactivation process, the IRPAgent informs the IRPManager on the result of the process. |  |
| Ends when (*) | The selected network elements are in No-ES state. |  |
| Exceptions | FFS. |  |
| Post Conditions | Energy saving deactivation has been performed on the selected NEs. The selected NEs are in notEnergySaving state. The network coverage is maintained. The network capacity accommodates the increased load. |  |
| Traceability (*) | REQ-NCES-FUN-03, REQ-NCES-FUN-04 |  |

Next change

### 6.4.4	Energy saving deactivation on selected network elements (Distributed ES)

| Use Case Stage | Evolution / Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal (*) | Deactivating energy saving for a selected part of the network |  |
| Actors and Roles (*) | IRPManager as user |  |
| Telecom resources | Network elements (NEs) and their OSS. |  |
| Assumptions | The network operator has enabled the energy saving distributed function on selected network elements in a part of the network, so that the NEs can deactivate energy saving state depending on load conditions. An interference control function is available to support the coverage adjustment process among NEs in compensatingForEnergySaving and energySaving state. |  |
| Pre conditions | The affected network elements are in energySaving state or in compensatingForEnergySaving state. Network elements (e.g. base stations) are not in a faulty state. |  |
| Begins when | The distributed algorithm decides to deactivate energy saving for a selected part of the network based on network load. |  |
| Step 1 (*) (M) | The NE executes the energy saving algorithm that decides which NEs remain in energySaving state or compensatingforEnergySaving state or enter notEnergySaving state. |  |
| Step 2 (*) (M) | Based on the output of step 1, those NEs that in energySaving state or compensatingforEnergySaving state which have been selected to be transferred to notEnergySaving state, initiate energy saving deactivation or energy saving compensation deactivation and inform the IRPManager. |  |
| Ends when (*) | The selected network elements are in No-ES state. |  |
| Exceptions | FFS. |  |
| Post Conditions | Energy saving deactivation has been performed on the selected NEs. The selected NEs are in notEnergySaving state. The network coverage is maintained. The network capacity accommodates the increased load. |  |
| Traceability (*) | FFS |  |

End of changes