3GPP TSG-SA5 (Telecom Management)	S5-110440

SA5#75, 24 - 28 Jan 2011; Sorrento, ITALY	revision of S5-110205

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.551 | CR | 0002 |  rev | 1 |  Current version: | 10.0.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Add energy saving compensation deactivation requirement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | OAM-ES |  |  |  |  |  |  | Date:  |  | 28/01/2011 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change:  |  | In the capacity limited scenario, in off-peak traffic situation, some NEs/Cells can be transferred to energySaving state and other related NEs/Cells can be transferred to compensatingforEnergySaving state; when it turns into peak traffic situation, the NEs/Cells in energySaving state shall be transferred to notEnergySaving state, and the related NEs/Cells in compensatingforEnergySaving state shall also be transferred to notEnergySaving state. So, a requirement about energySaving compensation deactivation is needed. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | - Add a new requirement about energySaving compensation deactivation. - The requirement REQ-NCES-FUN-01 is reworded to align with the other requirements. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | The IRPManager cannot initiate the energySaving compensation deactivation. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 6.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

First change

## 5.1	Requirements

REQ-32.551-CON-01

The acceptable impact on services shall be determined based on operator’s policy.

Remark: What exactly is meant with “operator’s policy”, what impacted services could be and what the consequences of not meeting the policy may be needs further discussion.

REQ-32.551-CON-02

The IRPManager shall be able to monitor how the network and the user service quality are influenced by energy saving function.

REQ-32.551-CON-03

IRPManager shall be able to monitor the performance of the energy saving function.

REQ-32.551-CON-04

The IRPAgent shall support a capability allowing the IRPManager to retrieve energy consumption information for each of its managed NEs.

REQ-32.551-CON-05

The IRPAgent should support a capability allowing the IRPManager to configure for each of its managed NEs the period of time for which energy consumption information will be provided.

REQ-32.551-CON-06

The IRPManager shall be able to initiate energy saving compensation activation and/or deactivation on one or multiple cells or network elements.

This requirement applies for the use case capacity limited network, for other use cases it is FFS.

REQ-32.551-CON-07

IRPManager shall be able to enable and disable energy saving for a selected part of the network.

REQ-32.551-CON-08

The IRPAgent shall support a capability allowing the IRPManager to initiate energy saving activation/deactivation on one or multiple cells or network elements in the network.

REQ-32.551-CON-09

When a NE is in energySaving state the IRPAgents shall not consider the NE as a fault, and no alarms shall be raised to the IRPManager for any condition that is a consequence of an energySaving NE.

REQ-32.551-CON-10

The IRP Agent shall be able to allow the IRPManager to define a list of cells to prevent them from going into energySaving state.

REQ-32.551-CON-11

The  IRPAgent shall allow the IRPManager to query for all switched off cells in the network under its domain.

Remark: The reason for the switching off can be ES or something else.

REQ-32.551-CON-12

The IRPAgent shall support a capability to notify the IRPManager when a cell goes into or out of energySaving state.

REQ-32.551-CON-13

The IRPAgent shall notify the IRPManager when a cell fails to re-start as a result of going out of energySaving state.

REQ-32.551-CON-14

The IRPAgent should support a capability allowing the IRPManager to configure a cell traffic load threshold to be used for the decision if a network element goes into energySaving state.

REQ-32.551-CON-15

The IRPAgent should support a capability allowing the IRPManager to configure a cell traffic load threshold to be used for the decision if a network element goes out of energySaving state.

REQ-32.551-CON-16

The system shall bring cells within one eNB into energySaving state in the most energy efficient sequence.

REQ-32.551-CON-17

A cell must not go into energySaving state until emergency calls or Wireless Priority Service calls in the cell are completed. Forcing handovers of such calls because of ES shall be avoided.

REQ-32.551-CON-18

The IRPManager shall be able to access location and coverage information for each of its managed NEs.

REQ-32.551-CON-19

The IRPAgent shall provide the capability to allow the IRPManager to configure one or more related cells as the candidate cells to take over the coverage when the original cell is going into energySaving state.

This requirement applies to the eNodeB overlaid use case.

REQ-32.551-CON-20

The IRPAgent shall provide a capability to allow the IRPManager to indicate cell outage of an overlay cell that is a candidate cell that can take over the coverage for another cell.

This requirement applies to the eNodeB overlaid use case.

REQ-32.551-CON-21

A cell or a network element in energySaving state is a planned condition and should not be considered as an outage.

Next change

### 6.1.2	Requirements for NM Centralized ES

REQ-NCES-FUN-01

The IRPAgent shall support a capability allowing the IRPManager to initiate energy saving compensation activation to one or multiple cells or network elements.

This requirement applies for the use case capacity limited network.

REQ-NCES-FUN-05

The IRPAgent shall support a capability allowing the IRPManager to initiate energy saving compensation deactivation to one or multiple cells or network elements.

This requirement applies for the use case capacity limited network.

Editor’s note: this requirement is here presented not in sequential order for readability purpose.

REQ-NCES-FUN-02

The IRPAgent shall support a capability allowing the IRPManager to initiate energy saving activation to one or multiple cells or network elements.

REQ-NCES-FUN-03

The IRPAgent shall support a capability allowing the IRPManager to initiate energy saving deactivation to one or multiple cells or network elements.

REQ-NCES-FUN-04

The IRPAgent shall provide a capability allowing the IRPManager to monitor the network load.

End of changes