3GPP TSG-SA5 (Telecom Management)	S5-111424

SA5#76, 28 Feb- 4 Mar 2011; San Diego, USA	revision of S5-111121

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.551 | CR | 0005 |  rev | 1 |  Current version: | 10.0.1 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Clarify the description of energy saving compensation activation and deactivation over Itf-N |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | OAM-ES |  |  |  |  |  |  | Date:  |  | 04/03/2011 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change:  |  | In the SA5 #75 meeting, the group agrees to go way forward as proposed in S5-110443 to implement stage 2 of 32.551 NM centralized ES compensation requirements. As an action item to follow the conclusion, the existing EUTRAN NRM spec (32.762) and Generic RAN NRM spec (32.792) are checked for attributes which can be used in context of ES compensation. However, coverage related parameters have already been included by use case Capacity and Coverage Optimization (CCO) and they are defined by the existing specs. For example, Maximum Transmission Power (downlink transmit power) is defined in IOC EUtranGenericCell of 32.762, Antenna tilt and Antenna azimuth are defined in IOC AntennaFunction of 32.792. Therefore, reusing the coverage related parameters in use case CCO is feasible for fulfilling ES compensation. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Since there’s no specific TS for ESM stage 2, clarification of description in context of ES compensation is added in use case energy saving activation/deactivation on selected network elements (Centralized ES on NM layer). |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | No guidance can be refered to fulfill NM centralized ES compensation requirements in 32.551. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 6.4.1, 6.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

### 6.4.1	Energy saving activation on selected network elements 
(Centralized ES on NM layer)

| Use Case Stage | Evolution / Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal (*) | Activating energy saving for a selected network elements in a part of the network |  |
| Actors and Roles (*) | IRPManager as user |  |
| Telecom resources | Network elements (NEs) and their OSS. |  |
| Assumptions | Centralized ES on NM layer is performed. The network operator has decided to activate energy saving on selected network elements in a part of the network (network elements, e.g. base stations). This requires to activate energy saving on some network elements and to activate compensatingForEnergySaving on others. IRPManager is continuously monitoring load on NEs. An interference control function is available to support the coverage adjustment process among NEs in ES-Compensate and ESaving state. |  |
| Pre conditions | The network topology should allow transferring some network elements into ESaving state while maintaining coverage by transferring some other into ES-Compensate state. Network elements (e.g. base stations) are not in a faulty state. |  |
| Begins when | The IRPManager decides to activate energy saving on selected network elements in a part of the network based on monitored decreased network load. |  |
| Step 1 (*) (M) | The IPRManager makes a decision on which NEs should enter EnergySaving state, compensatingForEnergySaving state, or notEnergySaving state based on network load, geographic positions and maximum coverage of base stations. |  |
| Step 2 (*) (M) | Based on the output of step1, the IRPManager initiates energy saving activation and energy saving compensation activation on the NEs selected for the respective state transition. The energy saving compensation activation is achieved by reconfiguration of coverage related parameters over Itf-N. Such coverage related parameters are parameters to be optimized in use case Capacity and Coverage Optimization (See 32.522). |  |
| Step 3 (*) (M) | After the completion of the energy saving activation process, the IRPAgent informs the IRPManager on the result of the process. |  |
| Ends when (*) | The selected network elements are in energySaving state, and other selected network elements are in compensatingForEnergySaving state. |  |
| Exceptions | FFS |  |
| Post Conditions | Energy saving activation has been performed on some selected NEs. Other selected NEs are in compensatingForEnergySaving state. The network coverage is maintained. The network capacity is adapted to the reduced load. |  |
| Traceability (*) | REQ-NCES-FUN-01, REQ-NCES-FUN-02, REQ-NCES-FUN-04 |  |

2nd Modified Section

### 6.4.2	Deactivatation of energy saving on selected network elements (Centralized ES on NM layer)

| Use Case Stage | Evolution / Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal (*) | Deactivating energy saving for a selected part of the network |  |
| Actors and Roles (*) | IRPManager as user |  |
| Telecom resources | Network elements (NEs) and their OSS. |  |
| Assumptions | Centralized ES on NM layer is performed. The network operator has decided to activate energy saving on selected network elements in a part of the network (network elements, e.g. base stations). This requires to activate energy saving on some network elements and to activate energy saving compensation on others. The IRPManager is continuously monitoring load on NEs. An interference control function is available to support the coverage adjustment process among NEs in compensatingForEnergySaving and EnergySaving state. |  |
| Pre conditions | The affected network elements are in ESaving state or in ES-Compensate state. Network elements (base stations) are not in a faulty state. |  |
| Begins when | The IRPManager decides to deactivate energySaving for selected network elements in a part of the network based on network load. |  |
| Step 1 (*) (M) | The IPRManager makes a decision on which NEs should remain in energySaving state or enter compensatingforEnergySaving state, or notEnergySaving state based on network load, geographic positions and maximum coverage of base stations. |  |
| Step 2 (*) (M) | Based on the output of step1, the IRPManager initiates energy saving deactivation and energy saving compensation deactivation on the NEs selected for the respective state transition. The energy saving compensation deactivation is achieved by reconfiguration of coverage related parameters over Itf-N. Such coverage related parameters are parameters to be optimized in use case Capacity and Coverage Optimization (See 32.522). |  |
| Step 3 (*) (M) | After the completion of the energy saving deactivation process, the IRPAgent informs the IRPManager on the result of the process. |  |
| Ends when (*) | The selected network elements are in No-ES state or ES-Compensate state. |  |
| Exceptions | FFS. |  |
| Post Conditions | Energy saving deactivation has been performed on some selected NEs. Some selected NEs are in notEnergySaving state; other selected NEs are in compensatingForEnergySaving state. The network coverage is maintained. The network capacity accommodates the increased load. |  |
| Traceability (*) | REQ-NCES-FUN-01, REQ-NCES-FUN-03, REQ-NCES-FUN-04 |  |

End of modifications