3GPP TSG-SA5 (Telecom Management)	S5-110485

SA5#75, 24 - 28 Jan 2011; Sorrento, ITALY	revision of S5-110352

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.280 | CR | 0020 |  rev | 2 |  Current version: | 10.2.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Clarify how 'AoC information' is presented to the UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Alcatel-Lucent |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | eAOC |  |  |  |  |  |  | Date:  |  | 28/01/2011 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change:  |  | “AoC information” has been defined for an internal modelling of the overall AoC function behaviour, and only a sub-set of it is used for proceeding on different interfaces. The flows uses this “AoC information” as output for presentation to UE. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Replace “AoC information” by “AoC Information XML body” as output for presentation to UE”. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | How AoC is presented to UE is unclear |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 5.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

First change

### 5.3.2	Message Flows and Types for Offline Charging

The message flows in this chapter are based on the signalling flows specified in TS 24.647 [208].

The basic IMS session establishment for a user registered to AoC service(s) is depicted in the annex B. This basic call-flow will help describing in the future the message flows for AoC-S, AoC-D, AoC-E and also including cases where information are received from RTTI messages.

NOTE: The detailed AoC call-flows are FFS.

#### 5.3.2.1 	Successful Session Establishment: AoC-S with AoC information in reliable 1xx response (originating side)

Figure 5.3.2.1.1 shows the transactions for the successful delivery of the AoC information in 1xx response to the originating subscriber during session establishment originated by a UE.

- Figure 5.3.2.1.1: Message Sequence Chart for Session Establishment (1xx Response) with AoC-S

- An initial SIP Invite Request is received in the S-CSCF. This request is forwarded to the AoC Function.

- The AoC Function received the AoC Type = [AoC-S] and queries the OCS for Tariff Information.

- The AoC-S information is included in SIP 183 response.

- The UE acknowledges the SIP 183 with PRACK.

- AoC Function responses with SIP 200OK.

- The SIP Invite Request is received in the S-CSCF and forwards this request.

- The S-CSCF receives the SIP 200 OK response and forwards this response.

- The AoC Function queries the OCS and maps the Tariff Information into the AoC Information for further proceeding.

- The ACF inserts the  AoC-S and resulting “AoC information XML body” as defined in TS 24.647[208], in the SIP 200 OK response, and the S-CSCF forwards it towards UE

- The ACF sends a Charging Data Request with AoC service type and AoC Information indicating EVENT_RECORD to the CDF.

- The CDF generates the ACF-CDR to record the AoC service type and AoC Information.

- The CDF acknowledges the reception of the Charging Data Response.

#### 5.3.2.2 	Mid-session procedure: AoC-S with AoC information in an INFO request

Figure 5.3.2.2.1 shows the transactions for the successful delivery of the AoC information to the originating subscriber when a tariff change is detected by AoC Function.

NOTE:	This case is relevant when AoC-S is activated.

- Figure 5.3.2.2.1: Message Sequence Chart for mid-session procedure with AoC-S

- The AoC Function detects that tariff is changed and queries the OCS for Tariff Information.

- SIP INFO request is send with AoC-S and resulting “AoC information XML body” as defined in TS 24.647[208].

- The ACF sends a Charging Data Request with AoC service type and AoC Information indicating EVENT_RECORD to the CDF.

- SIP 200OK is received.

- The CDF acknowledges the reception of the Charging Data Response and generates the ACF-CDR.

#### 5.3.2.3	Session Release: AoC-E – Originating Party Clears

Figure 5.3.2.3.1 shows the transactions for the successful delivery of the AoC information to the originating subscriber when session is released by originating party.

NOTE:	This case is relevant also when AoC-D is activated.

Figure 5.3.2.3.1: Message Sequence Chart for Session Release Originating Party Clears

- A SIP session is released by sending a SIP BYE message. The S-CSCF forwards this message to the ACF and forwards this request.

- The AoC Function received the AoC Type = [AoC-E] and queries the OCS for Tariff Information.

- The S-CSCF receives the 200 OK response and forwards this response.

- The AoC Function maps the Tariff Information into the AoC Information for further proceeding.

- The ACF inserts the  AoC-S and resulting “AoC information XML body” as defined in TS 24.647 [208], in the SIP 200 OK response, and the S-CSCF forwards it towards UE

- The ACF sends a Charging Data Request with AoC service type and AoC Information indicating EVENT_RECORD to the CDF.

- The CDF generates the ACF-CDR to record the AoC service type and AoC Information.

- The CDF acknowledges the reception of the Charging Data Response.

#### 5.3.2.4	Session Release: AoC-E – Terminating party clears

Figure 5.3.2.4.1 shows the transactions for the successful delivery of the AoC information to the originating subscriber when session is released by terminating party.

NOTE:	This case is relevant also when AoC-D is activated.

- Figure 5.3.2.4.1: Message Sequence Chart for Session Release Terminating Party Clears

1)	A SIP session is released by sending a SIP BYE message. The S-CSCF forwards this message to the AoC Function and forwards this request.

2)	The AoC Function queries the OCS and converts the Tariff Information to AoC Information for AoC-E.

3)	The S-CSCF receives the 200 OK response and forwards this response.

4)	The AoC Function maps the Tariff Information into the AoC Information for further proceeding

5)	The ACF inserts the  AoC-E and resulting “AoC information XML body” as defined in TS 24.647[208], in the SIP 200 OK response, and the S-CSCF forwards it towards UE

6)   The ACF sends a Charging Data Request with AoC service type and AoC Information indicating EVENT_RECORD to the CDF.

7)	The CDF generates the ACF-CDR to record the AoC service type and AoC Information.

8)	The CDF acknowledges the reception of the Charging Data Response.

End of changes