3GPP TSG-SA5 (Telecom Management)	S5-110486

SA5#75, 24 - 28 Jan 2011; Sorrento, ITALY	revision of S5-10353

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.280 | CR | 0018 |  rev | 2 |  Current version: | 10.2.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Introduce AoC and offline charging Rf description to allow AoC offline charging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Alcatel-Lucent |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | eAOC |  |  |  |  |  |  | Date:  |  | 28/01/2011 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change:  |  | AoC and offline charging description relates to AoC control behaviour, whereas the AoC Invocation recorded for offline charging is not described. “AoC information” has been defined for an internal modelling of the overall AoC function behaviour, and only a sub-set of it is used for proceeding on different interfaces, in particular Rf interface for offline charging. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Introduce AoC and offline Rf messages charging description |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | AoC cannot be offline charged |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 4.3.3.1, 6.3.1.3 (new), 6.3.3.1, 6.3.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

First change

#### 4.3.3.1	AoC and offline charging

##### 4.3.3.1.1	Interworking for AoC Service execution

For scenarios where the ACF is interworking with the offline charging feature and the service obligatory type is AoCI estimating cost and/or tariff information may be performed using any of the following methods:

- Local determination using offline synchronization of tariff information - The AoC function may synchronize out of band the tariff information from the charging domain. In this case, the AoC function will need to have an independent rating function.

- Interactively via the CDF through Ro - AoC function may obtain the tariff information interactively from the CDF.

- Interactively via the OCS through Ro - Offline subscribers can be perceived as online subscribers with unlimited balance (or very high balance that practically implies that). This approach enables the ACF to have unified flow of messages for offline and online subscribers in providing AoC information.

For scenarios where the ACF is interworking with the offline charging feature and the service obligatory type is AoCC determing cost and/or tariff information must be via the OCS through Ro.

4.3.3.1.2	AoC invocation recording

When ACF is triggered on behalf of a service and results in the AoC Information delivered to the UE, this ACF may also generate AoC related Rf messages as described in chapter 6.3.1.3.

Next change

#### 6.3.1.3	Rf message content related to AoC service

The ACF generates charging data transferred to the CDF using the Diameter accounting application, as described in the TS 32.299 [50].

The Charging Data Request and Charging Data Response are used for this ACF Charging data transfer.

##### 6.3.1.3.1	Charging Data Request Message

The following table illustrates the basic structure of a Diameter Charging Data-Request message as used for ACF Charging data transfer.

- Table 6.3.1.3.1-1: Charging Data Request Message Contents

| Field | Category | Description |
| --- | --- | --- |
| Session Identifier | M | Described in 32.299 [50] |
| Originator Host | M | Described in 32.299 [50] |
| Originator Domain | M | Described in 32.299 [50] |
| Destination Domain | M | Described in 32.299 [50] |
| Operation Type | M | Described in 32.299 [50] |
| Operation Number | M | Described in 32.299 [50] |
| Operation Identifier | OM | Described in 32.299 [50] |
| User Name | OC | Described in 32.299 [50] |
| Operation Interval | OC | Described in 32.299 [50] |
| Origination State | OC | Described in 32.299 [50] |
| Origination Timestamp | OC | This field contains the time when the operation is requested. |
| Proxy Information | OC | Described in 32.299 [50] |
| Route Information | OC | Described in 32.299 [50] |
| Operation Token | OM | Described in 32.299 [50] |
| Service Information | OM | This field holds the 3GPP specific AoC information and is described in 6.3.2 |

##### 6.3.1.3.2	Charging Data Response Message

The following table illustrates the basic structure of a Diameter Charging Data Response message as used for ACF Charging data transfer.

- Table 6.3.1.3.2-1: Charging Data Response Message Contents

| Field | Category | Description |
| --- | --- | --- |
| Session Identifier | M | Described in 32.299 [50] |
| Operation Result | M | Described in 32.299 [50] |
| Originator Host | M | Described in 32.299 [50] |
| Originator Domain | M | Described in 32.299 [50] |
| Operation Type | M | Described in 32.299 [50] |
| Operation Number | M | Described in 32.299 [50] |
| Operation Identifier | OM | Described in 32.299 [50] |
| User Name | OC | Described in 32.299 [50] |
| Operation Interval | OC | Described in 32.299 [50] |
| Origination State | OC | Described in 32.299 [50] |
| Origination Timestamp | OC | Described in 32.299 [50] |
| Proxy Information | OC | Described in 32.299 [50] |

Next change

#### 6.3.3.1	AoC information assignment for Service Information

The components in the Service Information that are use for AoC can be found in Table 6.3.2.1-1.

Table 6.3.3.1-1: AoC Information structure

| Field | Category | Description |
| --- | --- | --- |
| AoC Information | Oc | This is a structured field and holds the 3GPP specific parameter for AoC service. |
| Tariff-Information | Oc | This is a structured field and holds the Tariff specific parameters. The details are defined in subclause 6.3.2.2. It can chain inter operator tariff. |
| AoC-Cost-Information | OC | This is a structured field and holds the AoC cost specific parameters. The details are defined in subclause 6.3.2.3. |
| AoC-Subscription-Information | OC | Used by the AoC functions to inform the OCS about the AoC Subscription and Formatting parameters received from the HSS. When used by ACF for Charging data transfer as decribed in chapter 6.3.1.3, only “AoC service” is present. |

Next change

#### 6.3.3.3	Definition of AoC-Cost-Information

Advice of charge Cost information is provided within the AoC Cost Information. The AoC Cost is only used in CCA and Charging Data Request.

The detailed structure of the AoC Cost Information can be found in the table 6.3.3.3-1.

Table 6.3.3.3-1: Structure of AoC Cost Information

| Field | Category | Description |
| --- | --- | --- |
| Accumulated_Cost | Oc | The ammount charged since the beginning of the session |
| Value_Digits | OM |  |
| Exponent | Oc |  |
| Incremental_Cost | Oc | The ammount charged since the last report. |
| Value_Digits | OM |  |
| Exponent | Oc |  |
| Currency_Code | Oc | Ommited if the ammount is in non-monetary units units |

End of changes