3GPP TSG-SA5 (Telecom Management)	S5-110503

SA5#75, 24 - 28 Jan 2011; Sorrento, Italy	revision of S5-110488

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.280 | CR | 0019 |  rev | 1 |  Current version: | 10.2.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Provide offline AoC charging when the AoC Function is provided by MMTel AS - Align with 22.173 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Alcatel-Lucent |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | eAOC |  |  |  |  |  |  | Date:  |  | 28/01/2011 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change:  |  | According to TS 22.173, AoC is one of the MMTel Supplementary services, and AoC Function description is also expected to be described in this context: as for other supplementary services, AoC is therefore possibly provided by a dedicated AS or by the MMTel AS. AoC Offline charging description is missing for this case. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Clarify that AoC in IMS architecture can be provided in the same or different AS than the service Provide flows showing AoC Offline Charging when AoC is provided by MMTel AS |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | AoC Offline charging cannot fully be provided when AoC as a supplementary service for MMTel |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 2, 4.3, 5.3.2.5 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  | x |  | O&M Specifications |  |  | 32.275 |  |  |  |
| Other comments:  |  | Linked to 32.275 CR#0040 |  |  |  |  |  |  |  |  |

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging Architecture and Principles".

[2] – [19]	Void

[20]	3GPP TS 32.260: "Telecommunication management; Charging management; IP Multimedia Subsystem (IMS) charging".

[21]	3GPP TS 32.275: "Telecommunication management; Charging management; MultiMedia Telephony (MMTel) charging.

[22] – [49] 	Void

[50]	3GPP TS 32.299: "Telecommunication management; Charging management; Diameter charging application".

[51]	3GPP TS 32.298: "Telecommunication management; Charging management; Charging Data Record (CDR) parameter description".

[52]	Void

[53]	3GPP TS 32.296: "Telecommunication management; Charging management; Online Charging System (OCS) applications and interfaces".

[54] - [99]	Void

[100]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[101]	3GPP TS 22.115 "Service aspects; Charging and billing".

[102]- [199]	Void.

[200]- [201]	Void.

[202]	3GPP TS 22.173: " Multimedia Telephony Service and supplementary services;stage 1".

[203]	3GPP TS 22.024: "Description of Charge Advice Information (CAI)".

[204]	3GPP TS 22.086: "Advice of Charge (AoC) supplementary services - Stage 1".

[205]	3GPP TS 23.086: "AoC Supplementary Service, Stage 2".

[206]	3GPP TS 24.086: "AoC Supplementary Service, Stage 3".

[207]	3GPP TS 23.078: "Customized Applications for Mobile network Enhanced Logic (CAMEL); Stage 2".

[208]	3GPP TS 24.647: "Advice Of Charge (AOC) using IP Multimedia (IM) Core; Network (CN) subsystem".

[209]	3GPP TS 29.658: "SIP Transfer of IP Multimedia Service Tariff Information; Protocol specification".

[210]	3GPP TS 24.229: " IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[211]	3GPP TS 29.364: "IP Multimedia Subsystem (IMS) Application Server (AS) service data descriptions for AS interoperability".

[212]	ETSI TS 182 012 V2.1.4: "Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); IMS based PSTN/ISDN Emulation Sub system (PES); Functional architecture".

[213]	ETSI TS 183 043 V2.3.1: "Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); IMS-based PSTN/ISDN Emulation; Stage 3 specification".

Next change

## 4.3.	AoC in IP Multimedia Subsystem (IMS) architecture

The IMS Charging Architecture is described in TS 32.260 [20]. Figure 4.3.1 shows the specific part of the IMS charging architecture that handles AoC.

Figure 4.3.1: IMS AoC architecture

Figure 4.3.1 shows functional entities that are not directly involved in AoC, but completes the picture with affected interfaces. TS 24.647 [208] specifies the AoC information transferred to the UE via involved IMS functional entities. TS 29.658 [209] specifies the procedures for the realtime transfer of charging information in interconnection scenarios.

AoC Function may reside in the same AS hosting the service for which the AoC Service is to be provided, or in a separate AS (not shown in Figure 4.3.1).

The AoC Function (ACF) requests the AoC-related subscription and formatting parameters from the HSS via Sh. Additionally, filter criteria for ACF triggering may also be retrieved from the HSS by a CSCF via Cx.

The AoC Function obtains tariff information from the charging domain via Ro or the AoC function may have local Tariff information available (see section 4.3.1.1). See the AoC interfaces for details.

Note: The AoC function may be unified with the IMS-GW function in online charging.

Editor’s note: The relationship between IMS-GWF and AoC Function in IMS offline charging is FFS.

Next change

#### 5.3.2.5 	AoC Function co-located with same AS providing the service

The following flows show the transactions between an MMTel AS embedding the AoC Function for providing AoC as a MMTel supplementary service as defined in TS 22.173 [201].

##### 5.3.2.5.1	Successful Session Establishment: AoC-S (originating side)

The following figure 5.3.2.5.1.1 shows the transactions for the successful delivery of the AoC information to the originating subscriber during session establishment for MMTel service originated by a UE.

- Figure 5.3.2.5.1.1: Message Sequence Chart for Session Establishment with AoC-S for MMTel service.

- An initial SIP Invite Request is received in the S-CSCF. This request is forwarded to the MMTel AS embedding also AoC Function.

- The AoC Function received the AoC Type = [AoC-S] and queries the OCS for Tariff Information for MMTel service.

- The AoC-S “AoC information XML body” as defined in TS 24.647[208], is included in SIP 183 response.

- The UE acknowledges the SIP 183 with PRACK.

- AoC Function responses with SIP 200OK.

- The SIP Invite Request is received in the S-CSCF and forwards this request.

- The S-CSCF receives the SIP 200 OK response and forwards this response.

- The AoC Function queries the OCS for the Tariff Information for MMTel service

- The ACF inserts the  AoC-S and resulting “AoC information XML body” as defined in TS 24.647[208], in the SIP 200 OK response, and the S-CSCF forwards it towards UE

- The MMTel AS sends a Charging Data Request indicating START to the CDF, for the start of MMTel service, and includes AoC-S and AoC Information supplementary service.

- The CDF generates the MMTel-CDR recording also the AoC-S and AoC Information.

- The CDF acknowledges the reception of the Charging Data Response.

##### 5.3.2.5.2	AOC-D for serving party in INFO request

The following figure 5.3.2.5.2.1 shows the transactions for AoC-D delivery to the serving subscriber, when a tariff change is detected by AoC Function during MMTel service.

- Figure 5.3.2.5.2.1: Message Sequence Chart for AoC-D to serving party

- The AoC Function detects that tariff is changed for MMTel service and queries the OCS for Tariff Information.

- SIP INFO request is send with AoC-S and resulting “AoC information XML body” as defined in TS 24.647[208].

- The MMTel AS sends a Charging Data Request indicating INTERIM RECORD to the CDF and includes AoC-D and AoC Information as supplementary service.

- SIP 200OK is received.

- The CDF acknowledges the reception of the Charging Data Response and updates the MMTel-CDR with AoC-D and AoC Information.

##### 5.3.2.5.3	Session Release: AoC-E – Serving Party Clears

The following figure 5.3.2.5.3.1 shows the transactions for delivery of the AoC-E to the serving subscriber when it releases MMTel service.

Figure 5.3.2.5.3.1: Message Sequence Chart for AoC-E when serving Party Clears

- A SIP session is released by sending a SIP BYE message. The S-CSCF forwards this message to the MMTel AS and forwards this request.

- The AoC Function queries the OCS for Tariff Information in order to perform AoC-E for MMTel service.

- The S-CSCF receives the 200 OK response and forwards this response.

- The AoC Function maps the Tariff Information into the AoC Information for further proceeding.

- The ACF inserts the  AoC-S and resulting “AoC information XML body” as defined in TS 24.647[208], in the SIP 200 OK response, and the S-CSCF forwards it towards UE

- The MMTel AS sends a Charging Data Request indicating STOP_RECORD to the CDF and includes AoC-E and AoC Information as supplementary service.

- The CDF closes the MMTel-CDR with AoC-E and AoC Information included.

- The CDF acknowledges the reception of the Charging Data Response.

End of changes