3GPP TSG-SA5 (Telecom Management)	S5-111213

SA5#76, 28 Feb - 4 Mar 2011; San Diego, USA	revision of S5-11abcd

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.299 | CR | 0349 |  rev | - |  Current version: | 10.0.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Add 'Advice Of Charge (AoC)' MMTel supplementary service Charging description - Align with 32.275 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Alcatel-Lucent |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | eAOC |  |  |  |  |  |  | Date:  |  | 04/03/2011 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change:  |  | According to 22.173, AoC is one of the MMTel Supplementary services, and as such need to be considered in MMTel Charging as described in 32.275. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Introduce Aoc MMTel Supplementary service for MMTel Charging: Aoc in Service-Type and Service-Mode AVPs, and AoC-information is Supplementary-Service AVP |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Not possible to charge for AoC MMTel Supplementary service |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 7.2.193, 7.2.197, 7.2.219 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | x | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
|  |  | X |  | O&M Specifications |  |  | 32.298 |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

First change

### 7.2.193	Service-Mode AVP

The Service-Mode  AVP (AVP Code 2032) is of type Unsigned32 and provides the mode for CDIV, CB and ECT MMTel supplementary services.

The following values are defined:

“Communication Forwarding Unconditional (CFU)”							0

“Communication Forwarding Busy (CFB)”									1

“Communication Forwarding No Reply (CRNR)”							2

“Communication Forwarding on Not Logged-In  (CFNL)”					3

“Communication Deflection (CD)”												4

“Communication Forwarding on Subscriber Not Reachable (CFNRc)”		5

“Incoming Call Barring (ICB)”													6

“Outgoing Call Barring (OCB)”												7

“Anonymous Communication Rejection (ACR)”								8

“Blind Transfer”																	9

“Consultative Transfer”															10

“Three-Party (3PTY)”															11

“Advice of Charge -S (AoC-S)”												12

“Advice of Charge -D (AoC-D)”												13

“Advice of Charge -E (AoC-E)”												14

Values   1024 are reserved for specific Network/Manufacturer variants

Next change

### 7.2.197	Service-Type AVP

The Service-Type AVP (AVP Code 2031) is of type Unsigned32 and identifies the type of MMTel supplementary service.

The following values are defined:

“Originating Identification Presentation (OIP)”					0

“Originating Identification Restriction (OIR)”						1

“Terminating Identification Presentation (TIP)”					2

“Terminating Identification Restriction (TIR)”					3

“Communication HOLD (HOLD)”									4

“Communications Barring (CB )”									5

“Communication Diversion (CDIV)”								6

“Communication Diversion Notification (CDIVN)”				7

“Communication Waiting (CW)”									8

“Message Waiting Indication (MWI)”								9

“Conference (CONF)”												10

“Flexible Alerting (FA)”											11

“Completion of Communication to Busy Subscriber (CCBS)”	12

“Completion of Communications on No Reply (CCNR)”		13

“Malicious Communication Identification (MCID)				14

“Customized Alerting Tone” (CAT)								15

“Closed User Group” (CUG)										16

“Personal Network management” (PNM)							17

“Customized Ringing Signal” (CRS)								18

“Advice of Charge (AoC)”											19

Values   1024  are reserved for specific Network/Manufacturer supplementary services variants

Next change

### 7.2.219	Supplementary-Service AVP

The Supplementary-Service AVP (AVP code 2048) is of type Grouped and holds the specific supplementary service details for one MMTel supplementary service.

It has the following ABNF grammar:

Supplementary-Service :: =   < AVP Header: 2048>

[ Service-Type ]

[ Service-Mode ]
[ Number-Of-Diversions ]

[ Associated-Party-Address ]

[ Service-ID ]

[ Change-Time ]

[ Number-Of-Participants ]

[ Participant-Action-Type ]

[ CUG-Information ]

[ AoC-Information ]

End of changes