3GPP TSG-SA5 (Telecom Management)	S5-111388

SA5#76, 28 Feb - 4 Mar 2011; San Diego, USA	revision of S5-111140

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.280 | CR | 0021 |  rev | 1 |  Current version: | 10.2.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Clarification on AoC in interconnect |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | eAoC |  |  |  |  |  |  | Date:  |  | 04/03/2011 |
| Category:  | D |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change:  |  | Clarification on AoC in interconnect is needed. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Correction of clause 5.4. and removal of editor’s notes. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | AoC in interconnection is not clearly described. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 3.1, 4, 4.3, 4.3.2, 4.3.3.3, 5.1, 5.4, 6.3.1, 6.3.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | x | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
|  |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

First change

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [100] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [100].

- Advice of Charge (AoC): The Advice of Charge (AoC) supplementary service provides AoC Information to the served user for information (AoCI) or for charging (AoCC) related to a corresponding event, session or usage of a service. The AoC service may be delivered prior to, during or after the service delivery.

- AoC for Information (AoCI): An AoC supplementary service where the provided information is non-binding. I.e. the provided information is an estimation of the service cost and/or tariff. The provided information and the actual charges may differ.

- AoC for Charging (AoCC): An AoC supplementary service where the provided information is binding. I.e. the provided information must correspond to the actual charges.

- AoC at communication set-up time (AOC-S): An AoC supplementary service provided at communication establishment and/or at tariff switch time. The provided information includes Tariff Information for the requested service.

- AoC during the communication (AOC-D): An AoC supplementary service provided during the communication at predefined triggering conditions. The provided information includes accumulated Cost Information for the ongoing usage.

- AoC at the end of communication (AOC-E): An AoC supplementary service provided when the communication is released. The provided information includes the total accumulated cost.

- Charge Advice information (CAI): CAI elements as described in TS 22.024 [203].

- Tariff: set of parameters defining the applied charges for the use of a particular bearer / session / service.

- Cost: monetary amount that a user has to pay for the use of a particular bearer / session / service

Add-on charge: additional charge on top of the current tariff. An add-on charge can either be metered in non-monetary units (e.g. meter pulse) or in monetary-units (e.g. currency).

- Auxiliary Advice of Charge Function (AACF): An AACF provides Tariff and/or Cost Information for the requested service. The AACF resides outside of the local AoC Function and the Charging Domain.

Note: In this release, the AACF is considered as CDP for AoCI purpose. CDP is defined in TS29.658 [209]. The terms AACF and CDP may change in the future as a result of possible addition of charging capabilities.

- Charge Determination Point (CDP): Defined in ETSI ES 201.296.

- IMS-based PSTN/ISDN Emulation Subsystem (PES): Defined in ETSI TS 182 012 [212].

Next change

# 4	Architecture Considerations

Next change

## 4.3.	AoC in IP Multimedia Subsystem (IMS) architecture

The IMS Charging Architecture is described in TS 32.260 [20]. Figure 4.3.1 shows the specific part of the IMS charging architecture that handles AoC.

Figure 4.3.1: IMS AoC architecture

Figure 4.3.1 shows functional entities that are not directly involved in AoC, but completes the picture with affected interfaces. TS 24.647 [208] specifies the AoC information transferred to the UE via involved IMS functional entities. TS 29.658 [209] specifies the procedures for the realtime transfer of charging information in interconnection scenarios.

The AoC Function (ACF) requests the AoC-related subscription and formatting parameters from the HSS via Sh. Additionally, filter criteria for ACF triggering may also be retrieved from the HSS by a CSCF via Cx.

The AoC Function obtains tariff information from the charging domain via Ro or the AoC function may have local Tariff information available (see section 4.3.1.1). See the AoC interfaces for details.

Note: The AoC function may be unified with the IMS-GW function in online charging.

Next change

### 4.3.2	AoC interfaces

AoC has the following interfaces:

Sh – for obtaining AoC-related subscription and formatting parameters from the HSS.

ISC – for receiving RTTI from Auxiliary AoC Function and for providing the AoC information to the UE. Ro / Re – for obtaining tariff and cost information; Ro MUST be used for providing AoCC service and may be used for AoCI services.

Auxiliary AoC functionality (AACF) can be embodied in external nodes such as:

- Application Server

- Charging Determination Point (CDP) in a PSTN network

- SIP node in another IMS domain

Figure 4.3.2.1 shows possible locations of Auxiliary AoC Functional nodes interacting with IMS AoC Function.

SIP-AS

IMS-GWF

ACF

OCS

MGCF

PSTN Switch

IBCF

SIP Node

CDP

S-CSCF

3rd party AS

Auxiliary Advice of Charge

Internal Node

SIP

Diameter

ISUP

Figure 4.3.2.1: Logical AoC architecture with Auxiliary AoC Function

Next change

#### 4.3.3.3	AoC and Realtime Transfer of Tariff Information

- The AoC service shall receive the tariff or cost provided in real time by the external network or service provider (e.g. interconnection scenarios or 3rd party services), according to TS 29.658 [209]. The AoC information provided to the UE may take the provided information into consideration.

Note: This feature is valid only for AoCI service in this release.

Next change

## 5.1	Common Charge Advice Principles

Advice of Charge (AoC) is a supplementary service which provides AoC Information to the UE in real-time. It may contain Cost and/or Tariff Information for the requested service and may be provided at the beginning, during or at the end of service execution.

Next change

## 5.4	AoC in Inter-connected

### 5.4.1	Principles

### In case of interconnection scenarios or 3rd Party Services, the AoC service may be based on Real Time Tariff Information (RTTI) provided by the external network or service provider, as described in clause 4.3.3.3.

### 5.4.2	Scenarios

AoC Use cases based on Real Time Tariff Information are described in Annex A.1.2 and A.1.3.

### 5.4.3	Message flows

RTTI related signalling flows are described in TS 29.658 [209].

Next change

### 6.3.1	Diameter message contents

#### 6.3.1.1	Summary of AoC Message Formats

The AoC Service uses the Credit-Control-Request (CCR) and Credit-Control-Answer (CCA) messages defined in TS 32.299 [50]. AoC service can be used in a request type price enquiry or complementary to regular CCR as described in clause 5.3.1.

The following table describes the use of these messages for AoC.

Table 6.3.1.1-1: AoC Messages Reference Table

| Command-Name | Source | Destination | Abbreviation |
| --- | --- | --- | --- |
| Credit-Control-Request | ACF | OCS | CCR |
| Credit-Control-Answer | OCS | ACF | CCA |

Next change

#### 6.3.3.2	Definition of the Tariff-Information

Tariff information is provided within the AoC Information.

The detailed structure of the Tariff Information can be found in the table 6.3.2.2-1.

Table 6.3.3.2-1: Tariff Information

| Field | Category | Description |
| --- | --- | --- |
| Tariff-Information | Oc | This is a grouped field with one of many tariffs |
| Current-Tariff | M | Tariff as defined in table 6.3.2.2-2 for the current time period. |
| Tariff-Time-Change | Oc | The tariffs switch time. |
| Next-Tariff | Oc | Tariff as defined in table 6.3.2.2-2 for the next time period. |

The detailed structure of a Tariff can be found in the table 6.3.2.2-2.

Table 6.3.3.2-2: Tariff

| Field | Category | Description |
| --- | --- | --- |
| Tariff | Oc | This is a grouped field with one of many tariffs |
| Currency_Code | Oc | Omited if non-monetary units is used |
| Scale_Factor | Oc | A scaling factor on the whole calculation. Could be used for example between HPLMN and VPLMN. |
| Value_Digits | OM |  |
| Exponent | Oc |  |
| Rate_Element | Oc | Group of cost per unit values of unit type. |
| Reason-Code | Oc | Indicates a specific charge type e.g. Usage, AddOn Charge, Set-Up-Charge or Communication-Attempt-Charge |
| Unit_Type | OM | The measuring unit; e.g. time, uplink volume, special service units |
| Unit_Value | OM | The number of consumed units that incur the charge. |
| Value_Digits | OM |  |
| Exponent | Oc |  |
| Unit_Cost | OM | The associated cost (in currency code) to be charged per Unit_value |
| Value Digits | OM |  |
| Exponent | Oc |  |
| Unit_Quota_Threshold | Oc | An upper limit for consumed units where the rate is still valid |

For example:

1. A rate of 20c for each Megabyte (total volume) up to 10 Megabyte will be depicted as Unit type - TOTAL-OCTETS, Unit Value - 1,048,576, Cost – 20 and Unit threshold - 10,485,760.

2. A rate of  30c per 60s : Cost_Value = 30, Unit_Value =60 assuming appropriate settings for currency and unit_type.

End of changes