3GPP TSG-SA5 (Telecom Management)	S5-111441

SA5#76, 28 Feb - 4 Mar 2011; San Diego, USA	revision of S5-111138

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.280 | CR | 0021 |  rev | 1 |  Current version: | 10.2.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Correction on Incremental Cost information over UNI in AoC-D |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | eAoC |  |  |  |  |  |  | Date:  |  | 04/03/2011 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change:  |  | The AoC information element on UNI interface doesn’t have the possibility to transfer incremental cost information. Instead, the UE can calculate the incremental cost be deducting the previous accumulated cost information from the current accumulated cost information. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Clarification added on how incremental cost information can be handled if the interface (UNI) doesn’t support the transfer of incremental cost information. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | It is not specified how the UE should display the incremental cost information. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | Annex C.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

First change

## C.2	AoC information mapping to Charging Information Elements

Herby is a conceptual mapping advising how to populate the Charging Information Element provided to the UE (as described in TS 24.647 [208] and ETSI TS 183 043 [213] as explicitly mentioned in the table below) out of the AoC Information.

| Charging Information Element | Mapping guidance |
| --- | --- |
| Expressing Charging Rates |  |
| - Price per time unit | When only one Rate Element with Unit Type = TIME exists |
| - Free of charge | When only one Rate Element with Unit Type = MONEY and Unit Value = 0 exists |
| - Flat rate | When only one Rate Element with Unit Type = MONEY and Unit Value > 0 exists |
| - Not available | No Rate Elements provided |
| Charged Items |  |
| - Basic communication | When the Rate Element contains Reason Code = Usage or no Reason Code is available. NOTE: If no other Charged Items are applicable, Basic Communication shall be used as default value |
| - Communication attempt | This parameter may be populated when the Rate Element contains Reason Code = Communication Attempt Charge NOTE: Alternatively, this information may be mapped into “Operation of service” (see below). |
| - Communication setup | This parameter may be populated when the Rate Element contains Reason Code = Set Up Charge NOTE: Alternatively, this information may be mapped into “Operation of service” (see below). |
| - Operation of service | This parameter may be populated depending on Service specific Information or when the Rate Element contains Reason Code <> Usage. NOTE: If this parameter is populated depending on the Reason Code, Communication Attempt and Communication setup shall not be used. |
| - pes_aoc-s_phase_duration | The value of Unit_Quota_Threshold is mapped into pes_aoc-s_phase_duration when the AoC Extended XML schema is needed (see ETSI TS 183 043 [213]) |
| Recorded Charges | Accumulated cost in AoC Cost Information |

NOTE 1:	Other legacy Charging Information Elements (e.g. Special charging code, Special charging arrangement and Billing Identification) are not supported in IMS.

NOTE 2: In the case when the UNI interface can’t transfer the incremental cost, then the user part can calculate it by deducting the previous accumulated cost information from the current accumulated cost information.

End of changes