3GPP TS 32.772 V10.0.0 (2011-03)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Home Node B Subsystem (HNS)

Network Resource Model (NRM)

Integration Reference Point (IRP):

Information Service (IS)

(Release 10)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

OAM, Home Node B Subsystem, NRM

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2011, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI currently being registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	5

1	Scope	6

2	References	6

3	Definitions and abbreviations	7

3.1	Definitions	7

3.2	Abbreviations	8

4	System Overview	9

4.1	Compliance rules	9

5	Modelling approach	9

6	Information Object Classes	9

6.1	Information entities imported and local labels	9

6.2	Class diagram	10

6.2.1	Attributes and relationships	10

6.2.2	Inheritance	12

6.3	Information object class definitions	12

6.3.1	HNBGWFunction	12

6.3.1.1	Definition	12

6.3.1.2	Attributes	12

6.3.1.3	Notifications	12

6.3.2	IuhSignLinkTp	13

6.3.2.1	Definition	13

6.3.2.2	Attributes	13

6.3.2.3	Notifications	13

6.3.3	EP_Iuh	13

6.3.3.1	Definition	13

6.3.3.2	Attributes	13

6.3.3.3	Attribute constraints	13

6.3.3.4	Notifications	13

6.3.4	HMSFunction	13

6.3.4.1	Definition	13

6.3.4.2	Attributes	13

6.3.4.3	Notifications	13

6.3.5	HNB	14

6.3.5.1	Definition	14

6.3.5.2	Attributes	14

6.3.5.3	Notifications	14

6.3.6	HNBProfile	14

6.3.6.1	Definition	14

6.3.6.2	Attributes	14

6.3.7	LocalGWFunction	14

6.3.7.1	Definition	14

6.3.7.2	Attributes	15

6.3.7.3	Notifications	15

6.4	Information relationship definitions	15

6.4.1	ConnectedTo2 (M)	15

6.4.1.1	Definition	15

6.4.1.2	Roles	15

6.4.1.3	Constraints	15

6.4.2	ConnectedTo3 (M)	15

6.4.2.1	Definition	15

6.4.2.2	Roles	16

6.4.2.3	Constraints	16

6.4.3	ConnectedTo4 (M)	16

6.4.3.1	Definition	16

6.4.3.2	Roles	16

6.4.3.3	Constraints	16

6.4.4	ServesNode (O)	16

6.4.4.1	Definition	16

6.4.4.2	Roles	16

6.4.4.3	Constraints	16

6.5	Information attribute definitions	17

6.5.1	Definition and legal values	17

6.5.2	Constraints	18

6.6	Common Notifications	18

Annex A (informative):	 Change history	19

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project Technical Specification Group Services and System Aspects, Telecommunication Management; as identified below:

32.771:	Telecommunication management; Home Node B Subsystem (HNS) Network Resource Model (NRM) Integration Reference Point (IRP): Requirements

32.772:	Telecommunication management; Home Node B Subsystem (HNS) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)

32.773:	Telecommunication management; Home Node B Subsystem (HNS) Network Resource Model (NRM) Integration Reference Point (IRP): Common Object Request Broker Architecture (CORBA) Solution Set (SS)

32.775:	Telecommunication management; Home Node B Subsystem (HNS) Network Resource Model (NRM) Integration Reference Point (IRP): Bulk CM eXtensible Markup Language (XML) file format definition

The present document is part of a set of specifications, which describe the requirements and information model necessary for the standardised Operation, Administration and Maintenance (OA&M) of a multi-vendor 3G-system.

Configuration Management (CM), in general, provides the operator with the ability to assure correct and effective operation of the 3G network as it evolves. CM actions have the objective to control and monitor the actual configuration on the Network Elements (NEs) and Network Resources (NRs), and they may be initiated by the operator or by functions in the Operations Systems (OSs) or NEs.

CM actions may be requested as part of an implementation programme (e.g. additions and deletions), as part of an optimization programme (e.g. modifications), and to maintain the overall Quality of Service (QoS). The CM actions are initiated either as single actions on single NEs of the 3G network, or as part of a complex procedure involving actions on many resources/objects in one or several NEs.

During the lifetime of a 3G network, its logical and physical configuration will undergo changes of varying degrees and frequencies in order to optimise the utilisation of the network resources. These changes will be executed through network configuration management activities and/or network engineering, see 3GPP TS 32.600 [5].

# 1	Scope

The present document is an Integration Reference Point (IRP) named "Home Node B Subsystem (HNS) network resources IRP ", through which an 'IRPAgent' (typically an Element Manager or Network Element) can communicate Configuration Management information to one or several 'IRPManagers' (typically Network Managers) concerning HNS resources.

The present document specifies the protocol neutral HNS Network Resources IRP: Network Resource Model. It reuses relevant parts of the generic NRM in 3GPP TS 32.622 [6], either by direct reuse or sub-classing, and in addition to that defines HNS specific Information Object Classes.

In order to access the information defined by this NRM, an IRP IS is needed, such as the Basic CM IRP IS (3GPP TS 32.602 [7]) or the Bulk CM IRP IS (3GPP TS 32.612 [8]). However, which IS that is applicable is outside the scope of the present document.

The present document (NRM specification) is related to the IS in 3GPP TS 32.672 [9]

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[3]	3GPP TS 32.102: "Telecommunication management; Architecture".

[4]	3GPP TS 25.467: " Technical Specification Group Radio Access Network (UTRAN); UTRAN Architecture for 3G HNB".

[5]	3GPP TS 32.600: "Telecommunication management; Configuration Management (CM); Concept and high-level requirements".

[6]	3GPP TS 32.622: "Telecommunication management; Configuration Management (CM); Generic network resources Integration Reference Point (IRP): Network Resource Model (NRM)".

[7]	3GPP TS 32.602: "Telecommunication management; Configuration Management (CM); Basic Configuration Management Integration Reference Point (IRP): Information Service (IS)".

[8]	3GPP TS 32.612: "Telecommunication management; Configuration Management (CM); Bulk CM Integration Reference Point (IRP): Information Service (IS)".

[9]	Void.

[10]	3GPP TS 32.632:  "Telecommunication management; Configuration Management (CM); Core network resources Integration Reference Point (IRP): Network Resource Model (NRM)”

[11]	3GPP TS 32.300:  "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects”

[12]		3GPP TS 23.002: "Network Architecture".

[13]		3GPP TS 25.413: "UTRAN Iu interface RANAP signalling".

[14]	IETF RFC 4293: "SNMPv2 Management Information Base for the Internet Protocol using SMIv2".

[15]	IETF  RFC3873: "Stream Control Transmission Protocol (SCTP) Management Information Base (MIB)".

[16]	3GPP TS 32.583: “Home Node B (HNB) Operations, Administration, Maintenance and Provisioning (OAM&P); Procedure flows for Type 1 Interface HNB to HNB Management System (HMS)”

[17]	3GPP TS 23.002: “Technical Specification Group Services and Systems Aspects; Network architecture“

[18]	3GPP TS 22.220: “Service requirements for Home Node B (HNB) and Home eNode B (HeNB)”

[19]	3GPP TS 32.111-2: "Telecommunication management; Fault Management; Part 2: Alarm Integration Reference Point (IRP): Information Service (IS)".

[20]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description;Stage 2".

[21]	3GPP TS 23.401 "General Packet Radio Service (GPRS) enhancements for  Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the following terms and definitions apply. For terms and definitions not found here, please refer to 3GPP TS 32.101 [2], 3GPP TS 32.102 [3] and 3GPP TS 32.600 [5].

Association: In general it is used to model relationships between Managed Objects. Associations can be implemented in several ways, such as:

(1)	name bindings;

(2)	reference attributes; and

(3)	association objects.

This IRP stipulates that containment associations shall be expressed through name bindings, but it does not stipulate the implementation for other types of associations as a general rule. These are specified as separate entities in the object models (UML diagrams).

Managed Element (ME): an instance of the Information Object Class ManagedElement defined in 3GPP TS 32.622 [6].

Managed Object (MO): in the context of the present document, a Managed Object (MO) is a software object that encapsulates the manageable characteristics and behaviour of a particular Network Resource. The MO is instance of a MO class defined in a MIM/NRM. This class, called Information Object Class (IOC) has attributes that provide information used to characterize the objects that belong to the class (the term "attribute" is taken from TMN and corresponds to a "property" according to CIM). Furthermore, the IOC can have operations that represent the behaviour relevant for that class (the term "operation" is taken from TMN and corresponds to a "method" according to CIM). The IOC may support the emission of notifications that provide information about an event occurrence within a network resource.

Management Information Model (MIM): also referred to as NRM - see the definition below.

Network Resource Model (NRM): a model representing the actual managed telecommunications network resources that a System is providing through the subject IRP
An NRM identifies and describes IOCs, their associations, attributes and operations. The NRM is also referred to as "MIM" (see above), which originates from the ITU-T TMN.

Home Node B Management System (HMS): See TS 32.583 [16].

HNB GW: See TS 25.467 [4].

Home Node B Subsystem (HNS): See TS 23.002 [17].

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

DN	Distinguished Name

GW	Gateway

HNB	Home Node B

HNS	Home Node B Subsystem

IOCs	Information Object Classes

IRP	Integration Reference Point

ME	Managed Element

NRM	Network Resource Model

UTRAN	Universal Terrestrial Radio Access Network

# System Overview

## 4.1	Compliance rules

The following defines the meaning of Mandatory and Optional IOC attributes and associations between IOCs, in Solution Sets to the IRP defined by the present document:

- The IRPManager shall support all mandatory attributes/associations. The IRPManager shall be prepared to receive information related to mandatory as well as optional attributes/associations without failure; however the IRPManager does not have to support handling of the optional attributes/associations.

- The IRPAgent shall support all mandatory attributes/associations. It may support optional attributes/associations.

An IRPAgent that incorporates vendor-specific extensions shall support normal communication with a 3GPP SA5-compliant IRPManager with respect to all Mandatory and Optional information object classes, attributes and associations without requiring the IRPManager to have any knowledge of the extensions.

Given that

- rules for vendor-specific extensions remain to be fully specified, and

- many scenarios under which IRPManager and IRPAgent interwork may exist,

It is recognised that the IRPManager, even though it is not required to have knowledge of vendor-specific extensions, may be required to be implemented with an awareness that extensions can exist and behave accordingly.

# 5	Modelling approach

The modelling approach adopted and used in this IRP is described in TS 32.622 [6].

# 6	Information Object Classes

## 6.1	Information entities imported and local labels

| Label reference | Local label |
| --- | --- |
| 3GPP TS 32.622 [6], IOC, ManagedElement | ManagedElement |
| 3GPP TS 32.622 [6], IOC, ManagedFunction | ManagedFunction |
| 3GPP TS 32.622 [6], IOC, ManagementNode | ManagementNode |
| 3GPP TS 32.622 [6], IOC, Subnetwork | Subnetwork |
| 3GPP TS 32.622 [6], IOC, Top | Top |
| 3GPP TS 32.622 [6], IOC, VsDataContainer | VsDataContainer |
| 3GPP TS 32.632 [10], IOC, IupsLink | IupsLink |
| 3GPP TS 32.632 [10], IOC, IucsLink | IucsLink |
| 3GPP TS 32.632 [10], IOC, IubcLink | IubcLink |

## 6.2	Class diagram

### 6.2.1	Attributes and relationships

NOTE 1:	The listed cardinality numbers, in particular the use of cardinality number zero, do not represent transient states.  The transient state is considered an inherent property of all IOC instances and therefore there is no need to represent them by individual IOC cardinality numbers.

Figure1: Containment/Naming and Association diagram

NOTE 1:	The listed cardinality numbers, in particular the use of cardinality number zero, do not represent transient states.  The transient state is considered an inherent property of all IOC instances and therefore there is no need to represent them by individual IOC cardinality numbers.

NOTE 2:	Each instance of the VsDataContainer shall only be contained under one IOC. The VsDataContainer can be contained under IOCs defined in other NRMs.

Figure 2: VsDataContainer Containment/Naming and Association

The VsDataContainer is only used for the Bulk CM IRP.

Each IOC is identified with a Distinguished Name (DN) according to 3GPP TS 32.300 [11] that expresses its containment

### 6.2.2	Inheritance

This clause depicts the inheritance relationships that exist between IOCs.

Figure 3: Inheritance Hierarchy

NOTE: IuhSignLinkTp is a special definition for the signalling of the EP-Iuh, and these two IOC inherit from EP-RP.

## 6.3	Information object class definitions

### 6.3.1	HNBGWFunction

#### 6.3.1.1	Definition

This IOC represents HNB GW functionality. For more information about the HNB GW, see 3GPP TS 25.467[4].

#### 6.3.1.2	Attributes

Attributes of HNBGWFunction

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| hnbGwId | M | M | - |
| iPConfigInfo | M | M | - |
| maxNbrHNBRegistered | M | M | - |
| maxPacketCapability | M | M | - |

#### 6.3.1.3	Notifications

See clause 6.6 Alarm and configuration notifications.

### 6.3.2	IuhSignLinkTp

#### 6.3.2.1	Definition

This IOC represents a signaling link on the Iuh interface and inherits from EP-RP.

#### 6.3.2.2	Attributes

Attributes of IuhSignLinkTp

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| sctpAssocLocalAddr | M | M | - |
| sctpAssocRemoteAddr | M | M | - |

#### 6.3.2.3	Notifications

See clause 6.6 Alarm and configuration notifications.

### 6.3.3	EP_Iuh

#### 6.3.3.1	Definition

This IOC represents an end point of the Iuh interface. It inherits from EP-RP. For more information Iu-h interface, see 3GPP TS 25.467[4].

#### 6.3.3.2	Attributes

Attributes of EP_Iuh

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| farEndNEIPAddr | O | M | CM |

#### 6.3.3.3	Attribute constraints

| Name | Definition |
| --- | --- |
| Condition for farEndNEIPAddr’s write qualifier | The EP_Iuh object belongs to the different Domain Manager as the NE pointed by the farEndNeIpAddr attribute. |

#### 6.3.3.4	Notifications

See clause 6.6 Alarm and configuration notifications.

### 6.3.4	HMSFunction

#### 6.3.4.1	Definition

This IOC represents HMS functionality. For more information about HMS, see 3GPP TS 32.583 [16].

#### 6.3.4.2	Attributes

There are no attributes (other than those inherited).

#### 6.3.4.3	Notifications

There are no Notifications (other than those inherited).

### 6.3.5	HNB

#### 6.3.5.1	Definition

This SupportIOC represents HNB functionality.  For more information about the HNB, see 3GPP TS 25.467 [4].  For definition of HNB, see 3GPP TS 22.220 [18].

The Home NodeB, represented by the <<SupportIOC>> HNB, has registered itself with one node represented by HMSFunction.

#### 6.3.5.2	Attributes

Attributes of HNB

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | -- | - |

#### 6.3.5.3	Notifications

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAckStateChanged | See Alarm IRP (3GPP TS 32.111-2 [19]) |  |
| notifyChangedAlarm | See Alarm IRP (3GPP TS 32.111-2 [19]) |  |
| notifyClearedAlarm | See Alarm IRP (3GPP TS 32.111-2 [19]) |  |
| notifyNewAlarm | See Alarm IRP (3GPP TS 32.111-2 [19]) |  |

### 6.3.6	HNBProfile

#### 6.3.6.1	Definition

The HNBProfile is a representation of information that a) identifies a specific set of HNB devices and b) the related configuration parameters (and their values) that are required to be configured in those identified HNB devices  during HNB registration procedure.

It contains userLabel, an attribute inherited from ManagedFunction.  This is a user friendly label assigned by operator. Examples can be “VIP configuration”, “Gold Tier configuration”, “device vendor XYZ software version 3.4”, “camel”, etc

Editor Note: The userLabel is called configurationKind in previous documents such as in S5-093276 E pCR H(e)NB Profile handling.)

#### Attributes

Attributes of HNBProfile

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | -- |
| configuration | M | M | -- |
| criterion | O | M | -- |

### 6.3.7	LocalGWFunction

#### 6.3.7.1	Definition

This SupportIOC represents local Gateway functionality.  For more information about the local getway, see 3GPP TS 23.060 [20] and 3GPP TS 23.401[21].

The Local Gateway, represented by the <<SupportIOC>> LocalGWFunction, has registered itself with one node represented by HMSFunction.

#### 6.3.7.2	Attributes

Attributes of LocalGWFunction

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | -- | - |
| iPAddr | M | -- | - |
| collocationFlag | M | -- | - |
| servedNode | M | -- | - |

#### 6.3.7.3	Notifications

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAckStateChanged | See Alarm IRP (3GPP TS 32.111-2 [19]) |  |
| notifyChangedAlarm | See Alarm IRP (3GPP TS 32.111-2 [19]) |  |
| notifyClearedAlarm | See Alarm IRP (3GPP TS 32.111-2 [19]) |  |
| notifyNewAlarm | See Alarm IRP (3GPP TS 32.111-2 [19]) |  |

## 6.4	Information relationship definitions

### 6.4.1	ConnectedTo2 (M)

#### 6.4.1.1	Definition

This represents a uni-directional relation between the Iucslink and HNBGWFunction. The role of the relation shall be mapped to a reference attribute of the IOC.

#### 6.4.1.2	Roles

| Name | Definition |
| --- | --- |
| IucsLink-HNBGWFunction | This role (when present) represents IOC Iucslink capability to identify one connected HNBGWFunction. When present, it shall contain one HNBGWFunction DN. |

#### 6.4.1.3	Constraints

| Name | Definition |
| --- | --- |
| - | - |

### 6.4.2	ConnectedTo3 (M)

#### 6.4.2.1	Definition

This represents a uni-directional relation between the Iupslink and HNBGWFunction. The role of the relation shall be mapped to a reference attribute of the IOC.

#### 6.4.2.2	Roles

| Name | Definition |
| --- | --- |
| IupsLink-HNBGWFunction | This role (when present) represents IOC Iupslink capability to identify one connected HNBGWFunction. When present, it shall contain one HNBGWFunction DN. |

#### 6.4.2.3	Constraints

| Name | Definition |
| --- | --- |
| - | - |

### 6.4.3	ConnectedTo4 (M)

#### 6.4.3.1	Definition

This represents a uni-directional relation between the Iubclink and HNBGWFunction. The role of the relation shall be mapped to a reference attribute of the IOC.

#### 6.4.3.2	Roles

| Name | Definition |
| --- | --- |
| IubcLink-HNBGWFunction | This role (when present) represents IOC Iubclink capability to identify one connected HNBGWFunction. When present, it shall contain one HNBGWFunction DN. |

#### 6.4.3.3	Constraints

| Name | Definition |
| --- | --- |
| - | - |

### 6.4.4	ServesNode (O)

#### 6.4.4.1	Definition

This unidirectional association represents the relation between a Local GW and served HNB instances.

#### 6.4.4.2	Roles

| Name | Definition |
| --- | --- |
| ServedNode | This role represents the HNB instance served by a Local GW instance. |

#### 6.4.4.3	Constraints

| Name | Definition |
| --- | --- |
| - | - |

## 6.5	Information attribute definitions

### 6.5.1	Definition and legal values

Table 6.5.1.1 defines the attributes that are present in several Information Object Classes (IOCs) of the present document.

Table 6.5.1.1: Attributes definitions and legal values

| Attribute Name | Definition | Legal Values |
| --- | --- | --- |
| id | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| hnbGwId | Unique HNB GW ID. Ref. 3GPP TS 25.467 [4] specifies that HNB GW acts as a RNC to the existing core network using existing Iu interface. | See "RNC-ID" in Ref. 3GPP TS 23.002 [12] and 3GPP TS 25.413 [13] |
| iPConfigInfo | The IP address, subnetwork mask, default gateway for HNB GW (Ref. IETF RFC 4293 [14]). |  |
| maxNbrHNBRegistered | Maximum number of registered HNB means maximum number of HNB allowed to be registered. |  |
| maxPacketCapability | The HNB GW’s ability of forwarding packets, such as maximum number of forwarded packets per second. |  |
| sCTPAssocLocalAddr | The local port and IP address of SCTP association (See RFC3873 [15]). |  |
| sCTPAssocRemoteAddr | The remote port and IP address of SCTP association (See RFC3873 [15]). |  |
| farEndEntity | The value of this attribute shall be the Distinguished Name of the far end network entity to which the reference point is related. As an example, with EP_Iucs, if the instance of EP_Iucs is contained by one RncFunction instance, the farEndEntity is the Distinguished Name of the MscServerFunction instance to which this Iucs reference point is related. |  |
| farEndNeIpAddr | The IP address(s) of the far end network entity to which the reference point is related.This is an IPv4 or an IPv6 address. |  |
| configuration | It is a location of a data set. The data set is a set of HNB attributes (with values) needed to be loaded into the HNB. The data set does not contain all configuration data needed for a device to operate. Some configuration parameters are autonomously and dynamically calculated by the serving HMS. |  |
| criterion | It is a criterion that determines if a HNB should or should not be loaded with a particular configuration. The syntax and semantics of criterion is vendor-specific. Example 1: The syntax and semantics can be “If the HNB ID range is between ABC and DEF then APPLY the related configuration ”. Example 2: The syntax is a list of strings where each string is an “attribute == value” pair. An attribute represents a TR-196 parameter. Its value is the corresponding attribute value. The semantics is “if all pairs found in criterion are also found in the home devices, then the determination is positive in that the home device should be loaded with information of the data set identified by configuration; else not”. |  |
| iPAddr | The IP address(s) assigned for the Local Gateway. |  |
| collocationFlag | This attribute indicates whether the local gateway is collocated with the HNB or HeNB that it serves (see ServedNode relation in 6.2.1 UML class diagram) or not. | Legal values are: collocated, not-collocated. |
| servedNode | This attribute contains the DN of a HNB or HeNB that is being served (see ServedNode relation in 6.2.1 UML class diagram). |  |

### 6.5.2	Constraints

None.

## 6.6	Common Notifications

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAckStateChanged | See Alarm IRP (3GPP TS 32.111-2 [19]) |  |
| notifyAttributeValueChange | O |  |
| notifyChangedAlarm | See Alarm IRP (3GPP TS 32.111-2 [19]) |  |
| notifyClearedAlarm | See Alarm IRP (3GPP TS 32.111-2 [19]) |  |
| notifyNewAlarm | See Alarm IRP (3GPP TS 32.111-2 [19]) |  |
| notifyObjectCreation | O |  |
| notifyObjectDeletion | O |  |
| notifyComments | See Alarm IRP (3GPP TS 32.111-2 [19]) |  |

Annex A (informative): 
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| Sep 2009 | SA-45 | SP-090549 | -- | -- | Presentation to SA for information | --- | 1.0.0 |
| Mar 2010 | SA-47 | SP-100060 | -- | -- | Presentation to SA for approval | 1.0.0 | 2.0.0 |
| Mar 2010 | -- | -- | -- | -- | Publication of SA approved version | 2.0.0 | 9.0.0 |
| Mar 2011 | SA-51 | SP-110099 | 001 | 2 | Add localGWFunction IOC | 9.0.0 | 10.0.0 |