3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240041

10th – 18th July 2024	(revision of S6a240xxx)

Source:	ZTE Corporation

Title:	New Solution for KI#1: Resource Owner Consent Retrieval

Spec:	3GPP TR 23.700-22 v0.3.0

Agenda item:	8.7

Document for:	Approval

Contact:	Yang Li (li.yang1226@zte.com.cn)

1. Introduction

The Key issue 1 includes the following open issues:

2. How to enable retrieval of the resource owner consent parameters by an API exposure function from the authorization function.

This paper provides a new solution related to resource owner consent retrieval by an API exposure function.

2. Reason for Change

In current specification, the consent of resource assess will only be requested by API invoker before service API invocation procedure. However, retrieval procedure of the resource owner consent by an API exposure function from the authorization function has not been considered. To support resource owner consent retrieval by an API exposure function, a retrieval procedure of the resource owner consent is proposed in this solution.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-22.

* * * First Change * * * *

6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | KI #7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  | X |  |  |  |  |
| Sol #2 | X |  |  |  |  |  |  |
| Sol #3 | X |  |  |  |  |  |  |
| Sol #4 |  |  |  | X |  |  |  |
| Sol #5 |  |  |  |  | X |  |  |
| Sol #6 |  |  |  |  |  |  | X |
| Sol #7 |  |  |  |  |  |  | X |
| Sol #8 |  |  |  |  |  |  | X |
| Sol #9 | X |  |  |  |  |  |  |
| Sol #10 | X |  |  |  |  |  |  |
| Sol #11 | X |  |  |  |  |  |  |
| Sol #12 | X |  |  |  |  |  |  |

* * * 2nd Change (all texts are new) * * * *

6.x	Solution #x: Resource Owner Consent Retrieval

6.x.1	Solution description

6.x.1.1	General

This solution is for Key issue #1 to retrieval the resource owner consent by an API exposure function.

In current specification, the consent of resource assess will only be requested by API invoker before service API invocation procedure. However, retrieval procedure of the resource owner consent by an API exposure function from the authorization function has not been considered. To support resource owner consent retrieval by an API exposure function, a retrieval procedure of the resource owner consent is needed.

The resource owner consent can be requested by AEF actively. And for the updated resource owner consent, AEF can subscribe it from CCF.

6.x.1.2		Procedure for resource owner consent retrieval

6.x.1.2.1	AEF requesting the resource owner consent from CCF

Pre-condition:

- The service API access requires obtaining consent from resource owner.

- The resource owner function has provided the user consent to CAPIF core function/Authorization function.

Figure 6.x.1.2-1:	Procedure for resource owner consent retrieval

- The API exposing function sends a resource owner consent retrieval request message to CAPIF core function/Authorization function.

- The CAPIF core function/Authorization function processes the request and sends a resource owner consent retrieval response message to the resource owner function. The response message shall include the resource owner consent parameters.

6.x.1.2.2	AEF subscribing the updated resource owner consent from CCF

Pre-condition:

- The service API access requires obtaining consent from resource owner.

- The resource owner function has provided the user consent to CAPIF core function/Authorization function.

Figure 6.x.1.2-2:	Procedure for resource owner consent retrieval

- The API exposing function sends a updated resource owner consent subscription request message to CAPIF core function/Authorization function.

- The CAPIF core function/Authorization function processes the request and sends a updated resource owner consent subscription response message to the resource owner function.

- If any update in resource owner consent, the CAPIF core function/Authorization function sends a updated resource owner consent notification message to the API exposing function. The notification message shall include the resource owner consent parameters.

- The API exposing function sends a notification acknowledgement message to the CAPIF core function/Authorization function for the notification received.

6.x.2	Architecture Impacts

Editor's note:	This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

6.x.3	Corresponding APIs

Editor's note:	This clause provides the corresponding APIs for supporting the solution.

6.x.4	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * End of Changes * * * *