3GPP TSG-SA6 Meeting #62-Ad Hoc-e	S6a240068

# Online, , 10th Jul 2024 - 18th Jul 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.700-82 | CR | 0001 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EN resolutions and missing eval and conclusion |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AIMLAPP |  |  |  |  |  | Date: |  |  | 2024-07-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are 4 Editors’ Notes in the TR, which are not controversial, but need to be resolved. Also, Solution #31 is not added in the conclusions and overall evaluation of KI#1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To solve the ENs, the following changes are proposed: - In 8.12.2, the EN is converted to NOTE since this will be discussed in normative phase. - In 8.22.3, the EN is converted to NOTE since the IE details are part of the normative work. - In 8.31, the sub-clauses 8.31.1 and 8.31.3 are updated (missing from earlier version). - In 11.2 and 12.1 the overall evaluation and conclusions for KI#1 are updated to cover the Solution #31. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ENs will remain, and Solution #31 is not concluded. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clauses: 8.12.2, 8.22.3, 8.31, 11.2, 12.1. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * * * * FIRST CHANGE * * * * * * *

### 8.12.2	Consumer-based ML Model Performance Degradation Detection

Figure 8.12.2-1: Consumer-based ML Model Performance Degradation Detection

This procedure corresponds to the step 0b in clause 8.12.1.

0.	An ADAE Server, as an AI/ML Enablement Consumer, receives trained ML model (or ML model information) from the AI/ML Enablement Server.

1.	The ADAE Server receives request from consumer for analytics, generates analytics by using the ML model provided by AI/ML Enablement Server, and responses/notifies to the consumer with the required analytics.

2.	The ADAE Server requests the consumer to feedback usage result of the analytics.

3.	The consumer uses the analytics for its operations and collects operation results. Performance degradation may be found from the operation results. The performance degradation may cause by e.g., insufficient analytics accuracy, or the current analytics cannot fulfill the changed conditions at the consumer.

4.	The consumer feedbacks the usage result of the analytics to the ADAE Server.

5.	The ADAE Server updates the analytics accuracy based on feedback information from the consumer, and checks the performance of the ML model, which is used to generate the analytics, e.g., performance degradation of the ML model.

NOTE: The capability of ADAES to detect model performance degradation will be revisited during normative phase.

If the AI/ML Enablement Consumer is e.g. VAL Server, similar procedure as shown in Figure 8.12.2-1 be performed for detecting of ML Model performance, by replacing "analytics" with e.g., "operation instructions".

* * * * * * * SECOND CHANGE * * * * * * *

### 8.22.3	Corresponding APIs

Table 8.22.3-1 describes the API for Horizontal FL training subscription request.

Table 8.22.3-1: Horizontal FL training subscription request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | The VAL server identifier. |
| AIML model and model parameters | M | Information about the AIML model and model parameters to use for HFL training/inferencing. |
| AIML enablement client list | M | A list of AIML enablement clients required for HFL training/inferencing. |
| Dataset requirements | O | Requirements for dataset required for HFL training/inferencing. |
| Number training rounds | O | Number of training rounds for HFL training |
| Number of AIML clients per round | O | Minimum number of AIML clients required for each training round |
| Notification settings | O | Settings for how often to send notifications providing status of HFL training. For example, periodic, event-based, upon completion. |

Table 8.22.3-2 describes the API for Horizontal FL training subscription response.

Table 8.22.3-2: Horizontal FL training subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the subscription request |
| Subscription ID | M | An identifier for the subscription |

Table 8.22.3-3 describes the API for Horizontal FL training subscription response.

Table 8.22.3-3: Horizontal FL training notify

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | Status for the request |
| ML model parameters | O (NOTE) | Updated model parameters from training |
| ML model outputs | O (NOTE) | ML model outputs from inferencing |
| Training elapse time | O | Total elapse time of the training |
| Error list | O | A list of errors encountered during training |
| Timestamp | O | A timestamp for the notification |
| NOTE: At least one of the information elements shall be provided. |  |  |

NOTE: Whether to include Training elapse time and Error list will be discussed during normative phase

* * * * * * * THIRD CHANGE * * * * * * *

## 8.31	Solution #31: Supporting AIML inference service in edge

### 8.31.1	Solution description

The solution addresses Key Issue #1 to enhance the architecture and related functions to support application layer AI/ML services in edge computing scenarios corresponding to hierarchical AIMLE deployment scenario as in clause 9.4.

At present, the vast majority of model inference services are concentrated in online interactive services with high real-time requirements (e.g., delay requirements of 50-150ms), such as video monitoring, industrial quality inspection. Restricted by the resources and computing capability of edge, pushing the trained model to the edge AIMLE server to perform inference tasks will solve the real-time requirements of VAL app or EAS.

Moreover, the trained model in edge AILME server is deployed on demand. When the model inference service requested by EAS is not available or is not the latest version, the edge AIMLE server needs to request the ML from the central AILME server/ML repository.

AIML enablement servers deployed at the edge may only perform inference services, and the model is also limited.

Pre-conditions:

-	AIML enablement servers are deployed at the edge, receiving trained models from central AIML enablement servers and are responsible for providing inference services for specific models for EAS.

-	EAS has connected to edge AIML enablement server at the EDN.

Figure 8.31.1-1: inference service request procedure in edge

- EAS initiates a request to Edge AIML enablement server to use AIML inference service (e.g., Intelligent Q&A service, network analysis service). The request includes EAS identifier, security credentials, service identifier, and may include other related information, e.g., the model vendor that offer the inference, inference delay requirements, performance/accuracy parameter, ML model information etc.

2.	Upon receiving the request, the Edge AIML enablement server validates if the requestor EAS is authorized to use AIML inference service. If the requestor is authorized, and the AIML enabled server already has a model that can meet the requirements of EAS and execute the current inference service, the AIML enablement server then proceed to step 7; otherwise, proceed to step 3.

3.	The Edge AIML enablement server sends a ML retrieval request to the Central AIML enablement server. The request includes security credentials, inference service identifier, and a list of requirements of the ML model, e.g., the ML model vendor, performance/accuracy parameter, ML model information.

4.	The Central AIML enablement server validates the request and verifies the security credentials provided in the request, Then Central AIML enablement server determine the appropriate ML model based on the requirements of the model in the ML retrieval request and then fetches the ML model from the ML repository (as defined in clause 7.2.4.4).

5.	The Central AIML enablement server pushes trained models that can perform the specific inference services to the edge.

6.	Upon obtaining the model, the Edge AIML enablement server executes the inference service.

7.	The Edge AIML enablement server provides the inference service result to EAS.

NOTE: The detailed information elements for the above procedure will be specified in normative phase.

### 8.31.2	Architecture Impacts

The application enabler layer architecture impacts are the following:

-	Edge AIML Enablement server is introduced to provide support the inference for the edge scenario.

-	Central AIML Enablement server has the capability of interacting with Edge AIML enablement server for providing the trained model for the edge scenario.

-	An ML repository needs to be defined for serving as ML model repository for the ML model used for edge scenario.

### 8.31.3	Corresponding APIs

This subclause provides a summary on the corresponding API for solution #31.

-	AIML Inference Service API (request-response; API provider: edge AIMLE Server; known consumers: EAS).

-	ML model retrieval API (request-response; API provider: central AIMLE Server; known consumers: edge AIMLE Server).

### 8.31.4	Solution evaluation

This solution addresses Key Issue #1 and in particular the architecture enhancements for edge scenarios (in hierarchical deployments). This solution is feasible and provides value by enabling an edge deployed AIMLE server to fetch a trained model from the central AIMLE / ML repository and support ML model inference locally at the edge platform. This solution doesn’t have any dependencies with other 3GPP WGs.

* * * * * * * FOURTH CHANGE * * * * * * *

## 11.2	Key issue #1: Support of Architecture Enhancement and Functions for Application Layer AI/ML Services

The open issues studied in the Key Issue #1 are as follows:

- Whether and how to enhance the architecture and related functions to support application layer AI/ML services.

- Whether and how the above architecture enhancement and related functions supporting the management/execution of AI/ML lifecycle operations.

- Whether and how the architecture enhancement and related functions leveraging the existing 5GC capabilities and assistance for the application layer AI/ML services.

- Whether and how to enhance the architecture and the above related functions to support application layer AI/ML services in edge computing scenarios.

- Whether and how to provide the exposure and related functionality to assist the AIML VAL applications for its AIML service operations like service lifecycle management (e.g., assist service creation, service status, service operation).

This clause provides an overall evaluation of the key issue #1. The solutions #1, #11, #12, #13, #17, #23, and #29 cover different aspects for the open issues in the KI #1.

Solution #1 addresses the open issue 1 by introducing the functional model of AIML enablement service for supporting application layer AI/ML services, and enhance ADAE to consume AIML enablement services for ML-based analytics. The main content about On-Network AIML enablement layer functional architecture has been added to clause 7.2.1.

Solution #17 addresses the open issues 1 and 5 by introducing a high-level procedure to enable VAL layer to offload processing and/or monitoring of AIML operations to the AIML Enablement layer. Solution #17 can be considered as candidate for normative work with necessary update for detail AIML task.

Solution #11 and #12 address the open issues 2 and 5 by introducing AIML service lifecycle management and AI/ML model lifecycle management, respectively. Solution #11 provides a generic procedure to enable VAL Server request to start, pause, continue, and finish an AIML service operation via the AIMLE Server. Solution #12 provides a procedure to support AI/ML model lifecycle management for ML model re-training and update if requested by the AIMLE consumer when model performance degradation is observed. Solution #11 can be considered as candidate for normative work for AIML service lifecycle management, and Solution #12 can be considered as candidate for normative work for AI/ML model lifecycle management.

Solution #13 addresses the open issue 3 by introducing the interaction between the AIMLE Server with ADAES and NEF for analytics and assistance information to support FL member (re)selection. This solution provides architecture enhancement for leveraging the existing ADAE services and 5GC capabilities for the application layer AI/ML services, for example the procedure for analytics and assistance information collection can be used to support ML model training and inference (e.g. FL member (re)selection). Solution#13 can be considered as candidate for normative work.

Solutions #23,#29 and #31 address the open issue 4 by introducing enhancement of the architecture and related functions to support application layer AI/ML services in edge computing scenarios. Solution #29 provides a generic procedure to enable consumer (e.g., CAS, EAS) request assistance from AIMLE Server for edge computing process. Solution#23 introduces to deploy AIMLE Server to EDN and distributes AIML service to the Edge AIMLE Servers. The procedure provided in Solution #29 can be used by Solution #23 for collecting assistance information from AIMLE Server on the AIML service distribution process. Solution #31 provides support for ML model inference requested by an EAS at an edge deployed AIMLE server, and describes the interaction between central and edge AIMLE servers to fetch the corresponding models. Solutions #23,#29 and #31 can be considered as candidate for normative work for support AIML services in edge computing scenarios.

Solution #11 can be used by Solution #17 to start, pause, continue, and finish an AIML service operation. Solution #29 can be used by the consumer in Solution #11 for assistance information for decision making on AI/ML operations.

* * * * * * * FIFTH CHANGE * * * * * * *

## 12.1	Conclusions of key issue #1

Following solution considerations will be considered for the normative work for KI #1:

- Solution #11 can be considered for normative work for AIML service lifecycle management, and #12 can be considered as candidate for normative work for AI/ML model lifecycle management.

- Solution #13 can be considered for normative work as a generic architecture for leveraging the existing ADAE services and 5GC capabilities for the application layer AI/ML services.

- Solution #17 can be considered for normative work with necessary update for detail AIML task.

- Solutions #23,#29 and #31 can be considered for normative work for support AIML services in edge computing scenarios.

NOTE:	The possible impact of solutions related to edge on the AIMLAPP architecture will be further discussed.