3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240075

10th – 18th July 2024	(revision of S6a240xxx)

Source:	China Mobile

Title:	New solution for KI#6: XR EAS selection based on N6 delay measurement

Spec:	3GPP TR 23.700-23

Agenda item:	8.5

Document for:	Approval

Contact:	zhengshaowen@chinamobile.com

1. Introduction

This paper provides a new solution for KI #6: XR EAS selection based on N6 delay measurement.

2. Reason for Change

The target of KI #6 is to reduce service latency for XR applications. N6 delay measurement has already been studied and concluded in KI#2, TR23.700-49. This paper proposes a new solution for KI #6 to take advantage of the N6 delay measurement mechanism. In this solution, the EES selects the EAS based on the N6 delay of candidate EAS(s) and UPF(s), which can be obtained from 5GC.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-23.

* * * First Change (all new text) * * * *

7.X	Solution #X: XR EAS selection based on N6 delay measurement

7.X.1	Architecture Impacts

This solution is based on architecture of EDGEAPP as described in 3GPP TS 23.558 [11].

7.X.2	Solution description

7.X.2.1	General

SA2 supports EAS and UPF selection enhancement based on N6 delay measurement. This solution utilizes such mechanism. In EAS discovery procedure, the EES sends N6 delay measurement request to 5GC and obtains the N6 delay of each pair of discovered XR EAS and UPF for the DNAI of the XR EAS. The EES selects the XR EAS and determines the target N6 delay. Then the EES sends the XR EAS IP and the N6 delay to 5GC for UPF selection, which guarantees the selection of the optimal XR EAS and UP path.

7.X.2.2	Procedure

EAS

UE

ECS

EES

5GC

2. EAS discovery request

6. N6 delay measurement response

1. Service Provisioning Procedure

3. EES discovers the EAS(s)

EAS

4. N6 delay measurement request

5. N6 delay measurement

7. EES determines the EAS and the target N6 delay

8. AF traffic influence

9. EAS discovery response

Pre-condition:

1.	The EEC obtains the ECS address information.

Procedure:

1.	Service provisioning procedure is performed as described in TS 23.558 clause 8.3.3.2.2.

2.	The EEC send the EAS discovery request message to the EES as described in TS 23.558 clause 8.5.2.2

3.	Upon receiving the request message from EEC, the EES discovers the candidate XR EAS(s) as described in TS 23.558 clause 8.5.2.2.

4.	The EES sends N6 delay measurement request to the 5GC, including: an indication of considering N6 delay, IP address(es) of candidate EAS(s), DNAI(s) of the candidate EAS(s), the measurement protocol(s) and optionally the protocol-specific configuration parameters.

5.	The 5GC performs N6 delay measurement for each pair of candidate XR EAS and UPF associated with requested DNAI(s). Detailed measurement procedure is defined in SA2.

6.	The 5GC sends N6 delay measurement response to the EES, including the N6 delay(s) related to  each candidate XR EAS.

7.	The EES determines the XR EAS and corresponding N6 delay based on the response received from 5GC in step 6.

8.	The EES triggers AF traffic influence procedure and sends the IP address of the XR EAS and the N6 delay determined in step 7.

9.	The EES sends the EAS discovery response message to the EEC, including the XR EAS information.

7.X.3	Solution evaluation

This solution addresses KI#6 to support XR EAS selection based on the measurement of N6 delay of XR EAS(s) and UPF(s) performed by 5GC. This solution utilizes N6 delay measurement mechanism that has already been supported by SA2.

* * * End of Change(s) * * * *