3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240076

10th – 18th July 2024	(revision of S6a240xxx)

Source:	China Mobile

Title:	Sol for KI#4 Application enablement layer capabilities usage to supporting the differentiated XR services

Spec:	3GPP TR 23.700-01

Agenda item:	8.6

Document for:	Approval

Contact:	Shaowen Zheng, zhengshaowen@chinamobile.com

1. Introduction

This contribution propose to address the following open issues in KI#4,

-	Whether and how the identified application enablement capabilities could be utilized to jointly support the XR service.

-	Whether and how the application enablement architecture could support the XR service taking advantage of the identified application enablement capabilities.

2. Reason for Change

It is proposed to provide differentiated XR services with different QoS requirements to different XR service users, based on the interaction with NSCE server and SEALDD server.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-01.

* * * First Change * * * *

7.x	Solution #x: Application enablement layer capabilities usage to supporting the differentiated XR services

7.x.1	Architecture Impacts

Editor's note:	Whether there will be architecture impact(e.g., introduced by XRApp server/client) depends on the architecture discussion.

7.x.2	Solution description

Figure 7.x.2-1 illustrates the procedure to support the differentiated XR services.

Pre-conditions:

1.	The XRAPP/SEALDD server acts as NSaaS provider, and is authorized to get network slice related capabilities from NSCE server.

Figure 7.x.2-1: procedure of supporting the differentiated XR services

- The VAL server sends a differentiated XR service request to XRApp/SEALDD server, with target identifier along with the service requirements, Service Description.

2a.	The XRApp/SEALDD server determines whether and which slice is needed based on the service requirements and may initiate the slice related communication service lifecycle management as defined in TS 23.435, clause 9.12.2. For example, a HDLLC slice would be request for key consumer, and eMBB slice for others.

Before triggering the network slice management service, to determine the target UE of network slice(s), the XRApp/SEALDD server acting as AF, may receive a UE location report or a monitoring event report from 5GC (assuming that XRApp/SEALDD server has subscribed to consume 5GC services like LCS or NEF monitoring events related to UE actual location, or UE mobility analytics from NWDAF) or Location service from SEAL.

2b.	The XRApp/SEALDD server determines whether the SEALDD enabled bandwidth control is needed, and may initiate the SEALDD enabled bandwidth control for different VAL users as defined in TS23.433, clause 9.8.2.

2c.	The XRApp server/SEALDD determines whether it is needed to request a specific session. If needed, the XRApp/SEALDD server initiates AF session with required QoS update as defined in TS 23.502, clause 4.15.6.6a, or Multi-member AF session with required QoS as defined in TS 23.502, clause 4.16.5.3.

3.	The XRApp/SEALDD server sends the differentiated XR service response to the VAL server indicating the result of the request.

Editor's note:	Whether the executing entity is XRApp server or SEALDD server depends on the architecture discussion.

7.x.3	Solution evaluation

This solution addresses KI#4, the differentiated XR services with different QoS requirements is provided to different users, based on the interaction with NSCE server and SEALDD server.

* * * Next Change * * * *

* * * Next Change * * * *

<Proposed change in revision marks>