3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240077

10th – 18th July 2024	(revision of S6a240xxx)

Source:	China Mobile

Title:	Sol for KI#7 EAS discovery with computing requirements to optimize XR service requirements

Spec:	3GPP TR 23.700-23

Agenda item:	8.5

Document for:	Approval

Contact:	Shaowen Zheng, zhengshaowen@chinamobile.com

1. Introduction

This contribution propose to address the following open issues in KI#7,

- How to support the E2E KPI optimization to meet real-time XR service requirements, taking the processing delay of XR application server into account, e.g. considering the potential enhancements to EDGEAPP in XR application server discovery or instantiation.

- What available information from 5G network can be used by enable layer to optimize the real-time XR service requirements.

2. Reason for Change

XR service requires for computing resource, and the required computing resource may depends on the specific task. It is optimized that selection of a suitable EAS with appropriate computing resource for XR service. The solution proposed to discovery EAS with computing resource requested by UE to optimize E2E KPI of XR service.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-23.

* * * First Change * * * *

7.x	Solution #x: Sol for KI#7 EAS discovery with computing requirements to optimize XR service requirements

7.x.1	Architecture Impacts

This solution is presented based on the existing EDGEAPP architecture in TS 23.558 [11].

7.x.2	Solution description

Figure 7.x.2-1 illustrates the procedure to support the EAS discovery with computing requirements for XR services.

Pre-conditions:

- EAS performs EAS registration procedure, as described in clause 8.4.3.2.2 of TS 23.558. EAS sends an EAS registration request to the EES. The request shall include the EAS profile and may include proposed expiration time for the registration, computing resources for XR service.

- EES registers to ECS according to EES Registration procedure in clause 8.4.4 of TS 23.558 [11].

Figure 7.x.2-1: procedure of supporting EAS discovery with computing requirements

- AC sends application access request to EEC, including computing requirements for XR application (if needed).

- The EEC sends the EAS discovery request to the EES, including computing requirements for XR application.

- Upon receiving the request from the EEC, the EES checks if the EEC is authorized to discover the requested EAS(s). Then the EES identifies the EAS(s) based on the provided EAS discovery filters, the UE location and computing requirements for XR application.

- The EES sends the EAS discovery response to EEC with the EAS endpoint information.

7.x.3	Solution evaluation

This solution enables E2E KPI (computing requirements) optimization with EAS discovery request carrying computing requirements. This solution is updated based on EAS discovery procedure in clause 8.5.2.2 of TS 23.558 and EAS registration procedure in clause 8.4.3.3.2 of TS 23.558. The solution addresses the all open issues in key issue #7 and has minimal architectural impacts.

* * * End Change * * * *