3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240078

10th – 18th July 2024	(revision of S6a240xxx)

Source:	China Mobile

Title:	Update Solution #11

Spec:	3GPP TR 23.700-23

Agenda item:	8.5

Document for:	Approval

Contact:	Shaowen Zheng, zhengshaowen@chinamobile.com

1. Introduction

This contribution propose to address the following open issues in KI#5,

2. Reason for Change

It is proposed to addresses KI#5 by utilizing PINAPP to do the tethered device discovery, utilizing the SEALDD to do the traffic transmission and measurement (including tethered device link). To measure the Tethered XR device, the SEALDD client on the 3GPP UE interacts with XR client on the tethered UE to do the measurement. To measure the Tethered display device, the SEALDD client on the 3GPP UE interacts with SEALDD client on the tethered UE to do the measurement.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-23.

* * * First Change * * * *

7.11	Solution #11: Support the tethered UE based on PINAPP

7.11.1	Architecture Impacts

As discussed in TR 26.806 [8], there are three types of tethered UE, Tethered Standalone AR Glasses, Tethered Display AR Glasses, and Tethered AR Glasses with 5G Relay. To focus more on the interaction with applications, the architecture of the application enablement layer can be divided into the following two types based on the deployment of the XR application. One is the tethered XR device, where XR application client is deployed on the tethered XR device, i.e., Tethered Standalone AR Glasses, and Tethered AR Glasses with 5G Relay. The other is the Tethered Display XR device, who deploys the XR application on the 5G client.

Usually the tethered XR device is none-3GPP device so it should connecting to the 5G UE to access the 3GPP network. Considering the PEGC is a UE with subscription data related to PIN within the 5GS, so the 5G UE could be the  PEGC and PEMC, the tethered XR device can act as a PINE.

Then the proposed application layer architecture for the tethered XR device is shown in Figure 7.11.1-1. The XR application client runs on the tethered XR device. The tethered XR device is tethered to a 3GPP UE and potentially uses the PINAPP capabilities to support the XR application, e.g., 5G UE acting as the PEGC and PEMC could be enhanced to provide the measurement service.

Figure 7.11.1-1. Application enablement architecture based on PINAPP to support Tethered XR device

SEALDD could utilize PINAPP’s PIN management capability to do the tethered device discovery, then to do the tethering link measurement of the tethered XR device. Some enhancement is needed to support the interaction between the SEALDD client and XR client on different device over SEALDD-C. As shown in the figure, the SEALDD client on the 3GPP UE, acting as PIN client, could get tethered device information from the PEMC using the PIN-3 interface.

Figure 7.11.1-2. Application enablement architecture based on SEALDD and PINAPP to support Tethered XR device

For the Tethered Display device, the proposed application layer architecture is shown in Figure 7.11.1-2. The 3GPP UE includes the XR application and functions. The Tethered Display device is connected to the 3GPP UE for displaying. The PIN client is installed on the Tethered Display device to measure and manage the media transmission.

Figure 7.11.1-3. Application enablement architecture based on PINAPP to support Tethered display device

SEALDD could utilize PINAPP’s PIN management capability to do the tethered device discovery, then to do the tethering link measurement of the tethered display UE, with some architecture enhancement. As shown in the figure, the SEALDD client on the 3GPP UE, acting as PIN client, could get tethered device information from the PEMC using the PIN-3 interface. The SEALDD-UUc interface needs to be defined to support the interaction between the SEALDD client on the tethered UE and SEALDD client on the 3GPP UE.

Figure 7.11.1-4. Application enablement architecture based on SEALDD and PINAPP to support Tethered display device

7.11.2	Solution description

7.11.2.1	SEALDD measurement based on PIN for Tethered XR device

Figure 7.11.2.1-1 illustrates the SEALDD measurement based on PIN for Tethered XR device procedure.

Pre-conditions:

1.	The UE or PINE has been pre-configured or has discovered the address (e.g. IP address, FQDN, URI) of the PIN server;

2.	The UE or PINE has already been registered in PIN server;

Figure 7.11.2.1-1: SEALDD measurement based on PIN for Tethered XR device procedure

1.	The PIN is successfully created and in use. The 3GPP UE acting as PEGC and PEMC, the Tethered XR devices acting as a PINE. The SEALDD server can acting as PIN server. During the PIN creation, the detail information of the tethered device has been provide to PEMC on 3GPP UE from PINAPP server as defined in clause 8.5.2, TS 23.542[x].

- 2.	The regular data transmission connection is established, with the information received in step1.

- The connection between the SEALDD server and 3GPP UE is established as defined in clause 9.2.2.2, TS 23.433.

- The VAL server sends a SEALDD transmission quality measurement subscription request to the SEALDD server. Optionally, the request contains the tethered device information. If authorization is successful, the SEALDD server sends a response to the VAL server with the subscription ID, expiration time.

- The SEALDD transmission quality measurement as defined in clause 9.7.2.1, TS 23.433.

- The SEALDD client measures the QoS between the SEALDD client and tethered device, with the information of the tethered device from the PEMC.

- To measure the Tethered XR device as shown in Figure 7.11.1-2, step 5 of clause 9.7.2.1, TS 23.433 is enhanced as follows. The SEALDD client on the 3GPP UE interacts with XR client on the tethered UE to do the measurement based on ICMP ping protocol.

- The SEALDD server reports the data transmission quality measurement results (e.g. latency, jitter, bitrate, packet loss rate) to the VAL server via the notification message.

7.11.2.2	SEALDD measurement based on PIN for Tethered display device

Figure 7.11.2.2-1 illustrates the SEALDD measurement based on PIN for Tethered display device procedure.

Pre-conditions:

1.	The UE or PINE has been pre-configured or has discovered the address (e.g. IP address, FQDN, URI) of the PIN server;

2.	The UE or PINE has already been registered in PIN server;

Figure 7.11.2.2-1: SEALDD measurement based on PIN for Tethered display device procedure

1.	The PIN is successfully created and in use. The 3GPP UE acting as PEGC and PEMC, the Tethered XR devices acting as a PINE. The SEALDD server can acting as PIN server. During the PIN creation, the detail information of the tethered device has been provide to PEMC on 3GPP UE from PINAPP server as defined in clause 8.5.2, TS 23.542[x].

2.	The regular data transmission connection is established, with the information received in step1.

The connection between the SEALDD server and 3GPP UE is established as defined in clause 9.2.2.2, TS 23.433.

Optionally, the SEALDD client may send the regular data transmission connection request to the tethered UE using the information form PEMC to establish a connection between the 3GPP UE and tethered device.

- The VAL server sends a SEALDD transmission quality measurement subscription request to the SEALDD server. Optionally, the request contains the tethered device information. If authorization is successful, the SEALDD server sends a response to the VAL server with the subscription ID, expiration time.

- The SEALDD transmission quality measurement as defined in clause 9.7.2.1, TS 23.433.

- The SEALDD client measures the QoS between the SEALDD client and tethered device, with the information of the tethered device from the PEMC.

To measure the Tethered display device as shown in Figure 7.11.1-2, step 5 of clause 9.7.2.1, TS 23.433 is enhanced as follows. The SEALDD client on the 3GPP UE interacts with SEALDD client on the tethered UE to do the measurement based on ICMP ping protocol, or using monitoring packet in established tethering link SEALDD connection.

- The SEALDD server reports the data transmission quality measurement results (e.g. latency, jitter, bitrate, packet loss rate) to the VAL server via the notification message.

7.11.3	Solution evaluation

This solution addresses KI#5 by to utilizing PINAPP to do the tethered device discovery, the SEALDD to do the traffic transmission and measurement (including tethered device link). To measure the Tethered XR device, the SEALDD client on the 3GPP UE interacts with XR client on the tethered UE to do the measurement based on ICMP ping protocol. To measure the Tethered display device, the SEALDD client on the 3GPP UE interacts with SEALDD client on the tethered UE to do the measurement based on ICMP ping protocol, or using monitoring packet in established tethering link SEALDD connection.

* * * Next Change * * * *

* * * Next Change * * * *

<Proposed change in revision marks>