3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240080

10th – 18th July 2024	(revision of S6a240xxx)

Source:	China Mobile

Title:	Evaluation and conclusion on KI#5

Spec:	3GPP TR 23.700-23

Agenda item:	8.5

Document for:	Approval

Contact:	zhengshaowen@chinamobile.com

1. Introduction

This paper provides evaluation and conclusion for KI #5: Support the tethered UE.

2. Reason for Change

The evaluation and conclusion for KI #5 is needed.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-23

* * * First Change * * * *

9.3.X	Key Issue #5 solutions evaluation

Key issue #5 studies how to identify and measure the traffic flows from the tethered devices behind the UE. There are three solutions addressing the related issues: solution #4, solution #9 and solution #11.

-	Solution #4 proposes that the SEALDD client on the tethering device to measures the QoS between the SEALDD client and tethered device.

-	Solution #9 proposes that ADAEC to collect the data from the tethered VAL UE(SEALDD client or VAL client), and analyze the delay for end-to-end XR application service, and the tethered link.

-	Solution #11 proposes to utilize PINAPP to do the tethered device discovery, and utilize SEALDD to measure the delay. The detailed architecture impact is analyzed to support the tethered device.

Regarding the tethered device identification, solution #11 proposes to utilize PINAPP to do the tethered device discovery and management.

Regarding the tethered link measurement, solution #11 and solution#4 propose to enhance the SEALDD to do the measurement. Solution #11 focuses on how the PINAPP and SEALDD collaborate to do the tethered link measurement, while solution #4 elaborates the detail of measurement implemented by SEALDD. Solution #9 focuses on how the analysis implemented based on collected data.

* * * Next Change (all new text) * * * *

10.X	Conclusions of key issue #5

It is agreed to adopted the following principles in the normative work:

- The PINAPP is utilized to do the tethered device discovery and management.

- The SEALDD is enhanced to do the measurement.

- The ADAES is utilized to do the delay analysis.

* * * End of Change(s) * * * *